| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0394 |
| --- | --- |

| Title: | Crosscheck of JCTVC-J0207: Improved type coding for SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1479 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document cross-checks the improved type coding for SAO in JCTVC-J0207. The cross-check results exactly match to the results provided by the proponent.

# Introduction

JCTVC-J0207 [1] proposes to replace unary binarization with truncated unary binarization for SAO type coding. The proponent provided the source codes and test results. It has been confirmed that the source codes are modified as decried in JCTVC-J0207 and that the cross-check results exactly match to the results by the proponent.

# Experimental Results

The proposed method in JCTVC-J0207 is compared with HM-7.0 anchor under the common test condition [2]. The results are shown in Table 1. Please note that encoding and decoding time may not be accurate.

Table 1. BD-rate results of JCTVC-J0207 compared to HM-7.0 anchor.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% |
| Class B | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.3% |
| Class C | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.3% | -0.4% | 0.0% | -0.2% | -0.2% |
| Overall | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% |
|  | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | -0.2% |
| Class F | 0.0% | -0.2% | -0.2% | 0.0% | -0.3% | -0.3% |
| Enc Time[%] | 95% |  |  | 99% |  |  |
| Dec Time[%] | 95% |  |  | 98% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.6% | -0.4% | 0.1% | -0.3% | -0.3% |
| Class B | 0.0% | -0.3% | -0.3% | 0.0% | -0.3% | -0.4% |
| Class C | 0.0% | -0.2% | -0.3% | 0.0% | -0.1% | -0.2% |
| Class D | 0.0% | -0.2% | -0.3% | 0.0% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.3% |
|  | 0.0% | -0.3% | -0.3% | 0.0% | -0.2% | -0.3% |
| Class F | 0.0% | -0.2% | -0.2% | 0.0% | -0.2% | -0.4% |
| Enc Time[%] | 104% |  |  | 106% |  |  |
| Dec Time[%] | 106% |  |  | 109% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.5% | 0.0% | -0.2% | -0.1% |
| Class C | -0.1% | -0.5% | -0.4% | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.2% | 0.2% | 0.0% | -0.2% | -0.1% |
| Class E | -0.1% | -0.5% | -0.4% | 0.0% | -0.5% | -0.3% |
| Overall | -0.1% | -0.4% | -0.3% | 0.0% | -0.3% | -0.2% |
|  | -0.1% | -0.4% | -0.3% | 0.0% | -0.2% | -0.1% |
| Class F | -0.2% | -0.5% | -0.4% | 0.1% | -0.1% | 0.6% |
| Enc Time[%] | 100% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 104% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.2% | -0.7% | 0.0% | -0.2% | -0.3% |
| Class C | -0.1% | -0.6% | -0.6% | 0.0% | -0.4% | -0.2% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | -0.4% | -0.7% |
| Class E | 0.0% | -0.8% | -0.6% | -0.1% | 0.0% | -0.3% |
| Overall | 0.0% | -0.4% | -0.5% | 0.0% | -0.3% | -0.4% |
|  | 0.0% | -0.4% | -0.4% | 0.0% | -0.3% | -0.3% |
| Class F | -0.1% | -0.1% | -0.9% | -0.1% | -0.4% | -0.4% |
| Enc Time[%] | 105% |  |  | 103% |  |  |
| Dec Time[%] | 107% |  |  | 101% |  |  |

# Conclusion

JCTVC-J0207 is verified. The required changes in CD test and HM software are smaill, but provided 0.2% - 0.5% gain in chroma and 0.1% gain in luma for LB-Main.

# Reference

- S. Jeong, S. Park, S. Byun, S. Lee and B. Jeon, “Improved type coding for SAO,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J0207, May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1100, May 2012.