| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0398 |
| --- | --- |

| Title: | AHG6: Crosscheck of the reference picture comparison for boundary strength in JCTVC-J0211 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang } @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck simplification of deblocking decision tree proposed by LG in JCTVC-J0211. In this contribution, the comparison of reference picture in the deblocking boundary strength decision tree is removed for simplification. The verification task was done successfully and the results matched exactly those provided by LG. No bit rate loss is observed.

# Verification

LG provided the HM source code of their proposed codec described in JCTVC-J0211 [1]. One test with normal QP (QP = 22, 27, 32, 37) condition is evaluated. The verifications are carried out by setting the following parameters in the provided code:

#define NOREFIDX_FOR_BS_TYPE3 1

Four configurations (RA-Main, RA-HE10, LB-Main, LB-HE10) were run for the proposed simplification of boundary strength decision with the provided code. The running of the tests includes encoding, decoding, and collecting the run time of encoding and decoding processes.

# Results

The BD-rates from our experimental results match exactly those provided by LG. The encoding time and decoding time are roughly similar to the anchor. The detailed results are listed in JCTVC-J0398.xls.

# Conclusion

The results presented by LG in JCTVC-J0211 are confirmed.

# References

- J. Kim, Hendry, B. Jeon, “On the reference picture comparison for boundary strength,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0211, July, 2012.