| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0400 |
| --- | --- |

| Title: | Crosscheck of simplification of the scaling process for MV prediction in JCTVC-J0155 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Chih-Ming Wang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, cm.wang yuwen.huang}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the simplification of the MV scaling process proposed by InterDigital in JCTVC-J0155r1. In this contribution, a one-step scaling factor derivation is proposed to replace the two-step derivation in HM-7.0. The verification task was done successfully and the results matched exactly those provided by InterDigital. No bit rate loss is observed. The software and text are reviewed and are matched. The method of JCTVC-J0155r1 is also implemented into hardware. The synthesis results show that the gate count of the scaling factor derivation is increased by 55% and the gate count of the entire motion vector scaling is increased by 21%. The proposed method may reduce software instruction count slightly but will increase the hardware cost significantly.

# Verification

In HM-7.0 MV scaling, the scaled MV is derived by following steps:

- Clip TB and TD to the range of -128 to 127, inclusive.

- Calculate iX=(214+ ( Abs(TD)>>1) )/TD

- Calculate iScale = (iX×TB+32)>>6

- Clip iScale in the range of -4096 to 4095, inclusive.

- Calculate MV2

  - Horizontal component: 	MV2_x=Sign(iScale × MV1_x) × ((|iScale × MV1_x| + 127) >> 8)

  - Vertical component: 	MV2_y=Sign(iScale × MV1_y) × ((|iScale × MV1_y| + 127) >> 8)

- Clip MV2 to the range of -32768 to 32767, inclusive. Note that the unit of MV2 is quarter-pixel.

In JCTVC-J0155r1, the MV scaling process is modified as follow:

- Clip TB and TD in the range of -32 to 31, inclusive.

- Calculate iScale = ((TB<<9) + Sign2(TB)*Abs(TD))/(TD<<1).

- Clip iScale in the range of -4096 to 4095, inclusive.

- Calculate MV2

  - Horizontal component: 	MV2_x=Sign(iScale × MV1_x) × ((|iScale × MV1_x| + 127) >> 8)

  - Vertical component: 	MV2_y=Sign(iScale × MV1_y) × ((|iScale × MV1_y| + 127) >> 8)

- Clip MV2 to the range of -32768 to 32767, inclusive. Note that the unit of MV2 is quarter-pixel.

Two major modifications are:

- Reduce the range of TB and TD from [-128,127] to [-32,31]

- Replace the two-step equation (the step 2 and step 3 in HM-7.0) by one-step equation (the step 2 in JCTVC-J0155r1)

InterDigital provided the HM source code of their proposed codec described in JCTVC-J0155r1 [1]. One test with normal QP (QP = 22, 27, 32, 37) condition is evaluated. The verifications are carried out by setting the following parameters in the provided code:

#define JCTVC_J0155 1

Four configurations (RA-Main, RA-HE10, LB-Main, LB-HE10) were run for the proposed simplification of the MV scaling process with the provided code. The running of the tests includes encoding, decoding, and collecting the run time of encoding and decoding processes.

Since the proposed modifications affect the hardware implementation, the hardware of proposed MV scaling module is also implemented. The MV scaling in HM-7.0 and the proposed MV scaling in JCTVC-J0155r1 are implemented with Verilog HDL and synthesized TSMC 40 nm process library.

# Results

The BD-rates from our experimental results match exactly those provided by InterDigital. The encoding time and decoding time are roughly similar to the anchor. The detailed results are listed in JCTVC-J0400.xls. For the hardware implementation, Table 1 and Table 2 show the comparisons for the scaling factor derivation and the entire motion vector scaling, respectively.

Table 1.  Synthesis result of HM-7.0 scaling factor derivation and J0155r1 scaling factor derivation

Table 2.  Synthesis result of HM-7.0 MV scaling and J0155r1 MV scaling

The step 2 in JCTVC-J0155r1 can be implemented by a general divider or a 32x32x16-bit LUT. In Table 1 and Table 2, we use a 32x32x16-bit LUT for the implementation. The synthesis results show that the gate counts of the scaling factor derivation and the entire motion vector scaling are increased by 54% and 21%, respectively. Moreover, if both TD and TB are clipped within [-32, 31] in HM-7.0 for fair comparison, the gate counts of the scaling factor derivation and the entire motion vector scaling are increased by 168% and 43%, respectively.

# Conclusion

The BD-rate results presented by InterDigital in JCTVC-J0155r1 are confirmed. The software and text are matched. The proposed method may require fewer instructions in software implementation, but will increase the cost in hardware implementation.

# References

- J. Dong, Y. Ye, “Simplification of the scaling process for MV prediction,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0155, July, 2012.

# Appendix

In the first version of JCTVC-J0155, it proposed two options. The first option is to limit the range of TB and TD within [-127,127] and remove the “clip iScale in the range of -4096 to 4095.” It increases the bit-depth of iScale, which will increase the hardware cost of deriving scaled MV. The second option is to replace the two-step equation (the step 2 and step 3 in HM-7.0) by one-step equation, “iScale = ((TB<<9) + TD)/(TD<<1) .”

The proposed methods in the first version of JCTVC-J0155 are also implemented with Verilog HDL and synthesized TSMC 40 nm process library. Table 3 shows the synthesis results. The synthesis results show that the proposed option-1, option-2, and combined method increases 25%, 61%, and 86% gate counts, respectively, for the entire motion vector scaling.

Table 3.  Synthesis result of HM-7.0 MV scaling and the first version of J0155 MV scaling