| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0403 |
| --- | --- |

| Title: | AHG6: Cross-check of JCTVC-J0096 Suppression of blocking artifacts at 32x32 transform boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shuo Lu Ohji Nakagami Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 Shuo.Lu@jp.sony.com ohji.nakagami@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document reports the cress-check results of JCTVC-J0096 which proposes to supress blocking artifacts at 32x32 transform boundaries by new syntax elements tu32_beta_offset_div2 and tu32_tc_offset_div2 in PPS or slice header. It is confirmed that source code exactly reflects the proposal. BD-rate matches with what provided by the proposer too.

# Introduction

To reduce visible blocking artifacts in some sequences,  JCTVC-J0096 proposes to add new syntax elements tu32_beta_offset_div2 and tu32_tc_offset_div2 in PPS or slice header. The two syntax is applied only to 32x32 transform boundaries.

Table1. Signaling of tu32_beta_offset_div2 and tu32_tc_offset_div2 in PPS

| if( deblocking_filter_control_present_flag ) { |  |
| --- | --- |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| if (Log2MaxTrafoSize == 5) { |  |
| tu32_beta_offset_div2 | se(v) |
| tu32_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |

Table2. Signaling of tu32_beta_offset_div2 and tu32_tc_offset_div2 in slice header

| if( deblocking_filter_control_present_flag ) { |  |
| --- | --- |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_override_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !slice_header_disable_deblocking_filter_flag ) { |  |
| beta_offset_div2 | se(v) |
| tc_offset_div2 | se(v) |
| if (Log2MaxTrafoSize == 5) { |  |
| tu32_beta_offset_div2 | se(v) |
| tu32_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

# Objective Results

Test Configurations:

tu32_beta_offset_div2 = 13, tu32_tc_offset_div2 = 13

beta_offset_div2 = 0, tc_offset_div2 = 0

The tested results are comparing with HM7.0 anchor. The results match what is in JCTVC-J0096.

Table3. BD-rate results of proposal vs. HM7.0 anchor

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 2.8% | 1.0% | 1.3% | 3.2% | 1.7% | 1.7% |
| Class B | 1.3% | 0.4% | 0.4% | 1.5% | 0.4% | 0.4% |
| Class C | 0.5% | 0.3% | 0.3% | 0.5% | 0.3% | 0.3% |
| Class D | 0.6% | 0.2% | 0.2% | 0.6% | 0.2% | 0.2% |
| Class E | 1.4% | 0.1% | 0.0% | 1.5% | 0.2% | 0.1% |
| Overall | 1.3% | 0.4% | 0.5% | 1.5% | 0.6% | 0.5% |
|  | 1.3% | 0.4% | 0.5% | 1.5% | 0.6% | 0.6% |
| Class F | 7.7% | 1.8% | 1.2% | 2.0% | 1.8% | 1.4% |
| Enc Time[%] | 93% |  |  | 94% |  |  |
| Dec Time[%] | 94% |  |  | 95% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1.7% | 0.4% | 0.5% | 2.5% | 1.3% | 1.7% |
| Class B | 1.1% | 0.4% | 0.4% | 1.2% | 0.4% | 0.3% |
| Class C | 1.3% | 0.4% | 0.4% | 1.3% | 0.4% | 0.3% |
| Class D | 0.7% | 0.3% | 0.4% | 0.8% | 0.3% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 1.2% | 0.4% | 0.4% | 1.4% | 0.6% | 0.6% |
|  | 1.3% | 0.4% | 0.4% | 1.5% | 0.6% | 0.6% |
| Class F | 2.5% | 1.3% | 1.2% | 2.1% | 1.4% | 1.2% |
| Enc Time[%] | #NUM! |  |  | 96% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.6% | 0.5% | 0.4% | 1.6% | 0.5% | 0.4% |
| Class C | 1.8% | 0.4% | 0.4% | 2.0% | 0.3% | 0.6% |
| Class D | 0.8% | 0.3% | 0.4% | 0.8% | 0.3% | 0.5% |
| Class E | 3.5% | 1.9% | 1.8% | 4.3% | 2.0% | 1.5% |
| Overall | 1.8% | 0.7% | 0.7% | 2.0% | 0.7% | 0.7% |
|  | 1.8% | 0.7% | 0.7% | 2.0% | 0.7% | 0.7% |
| Class F | 2.4% | 0.8% | 1.0% | 2.5% | 1.7% | 1.9% |
| Enc Time[%] | 108% |  |  | 107% |  |  |
| Dec Time[%] | 107% |  |  | 107% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.8% | 0.4% | 0.0% | 1.3% | 0.4% | 0.5% |
| Class C | 1.5% | 0.3% | 0.3% | 1.6% | 0.3% | 0.5% |
| Class D | 0.7% | 0.6% | 0.4% | 0.7% | 0.5% | -0.3% |
| Class E | 2.9% | 1.4% | 1.8% | 3.7% | 2.1% | 1.9% |
| Overall | 1.3% | 0.6% | 0.5% | 1.7% | 0.7% | 0.6% |
|  | 1.3% | 0.6% | 0.4% | 1.7% | 0.6% | 0.7% |
| Class F | 2.2% | 1.1% | 1.3% | 2.1% | 1.2% | 0.5% |
| Enc Time[%] | 109% |  |  | 108% |  |  |
| Dec Time[%] | 108% |  |  | 107% |  |  |

Table4. Objective results of proposed sequences

| Sequences | Method | QP | Bitrate (Kbps) | Y-PSNR (dB) | Cb-PSNR (dB) | Cr-PSNR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Riverbed | HM-7.0 + max offsets | 32 | 5852.75 | 34.62 | 39.03 | 41.43 |
|  |  | 37 | 2851.39 | 31.71 | 38.13 | 40.90 |
|  | Proposed (max TU32 offsets) | 32 | 5854.84 | 34.63 | 39.03 | 41.44 |
|  |  | 37 | 2853.23 | 31.71 | 38.13 | 40.91 |
| DucksTakeOff | HM-7.0 + max offsets | 32 | 18569.44 | 30.29 | 32.63 | 36.60 |
|  |  | 37 | 7590.24 | 27.72 | 31.56 | 35.97 |
|  | Proposed (max TU32 offsets) | 32 | 18568.74 | 30.33 | 32.63 | 36.59 |
|  |  | 37 | 7591.43 | 27.73 | 31.56 | 35.97 |

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1003, May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1100, May 2012.

[3] Do-Kyoung Kwon, Madhukar Budagavi, “Suppression of blocking artifacts at 32x32 transform boundaries”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCTVC-J0096

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).