| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0409 |
| --- | --- |

| Title: | Crosscheck of JCTVC-J0181: AHG6: On deblocking filter parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1479 d-kwon@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document cross-checks the proposal on deblocking filter parameters in JCTVC-J0181. Source codes and encoder configurations for test were provided by the proponents. It was confirmed that the source codes are written as described in JCTVC-0181 and that the cross-check results exactly match to ones provided by the proponent.

# Introduction

JCTVC-J0181 [1] proposes to increase the adaptive capability of deblocking filter by expanding effective value range of variable β for high QPs. Two independent methods are proposed: In the first proposal, a modified β is proposed and, in the second proposal, beta_step_log2 and tc_step_log2 which are signaled in a bitstream are proposed in order to achieve fine-grain adjustment for low QP and coarse-gain adjustment for high QP. These two methods also combined together.

# Experimental Results

Both methods proposed in JCTVC-0181 has on influence on common test conditions [2]. So different conditions are used to evaluated them.

## Proposal-1: Modification on β table

The proposed β table is evaluated at QP = 37, 42, 47 and 51, but still with the common test conditions. As shown in Table 1, it is confirmed that the cross-check results exactly matched to ones provided by the proponents. Test results also can be found in the attached spreadsheet (HM-7.0_vs_J0181_P1_Test1.xls).

Table 1. BD-rate results of JCTVC-J0181 Proposal 1, compared to HM-7.0 anchor at QP = 32, 37, 42 and 51.

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.4% | 0.1% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 113% |  |  | 111% |  |  |
| Dec Time[%] | 112% |  |  | 108% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.7% | -1.1% | -0.1% | 0.1% | 0.8% |
| Class B | -0.1% | -0.1% | 0.4% | 0.0% | 0.1% | 0.0% |
| Class C | -0.1% | 0.0% | 0.2% | -0.1% | 0.5% | 0.4% |
| Class D | -0.1% | 0.1% | -0.1% | 0.0% | 0.1% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Overall | -0.1% | -0.1% | -0.1% | 0.0% | 0.1% | 0.2% |
|  | -0.1% | -0.1% | -0.2% | 0.0% | 0.2% | 0.3% |
| Enc Time[%] | 93% |  |  | 101% |  |  |
| Dec Time[%] | 91% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.7% | 0.3% | -0.1% | -1.1% | -1.0% |
| Class C | -0.1% | 0.5% | 0.0% | -0.2% | 0.2% | 0.1% |
| Class D | 0.0% | 0.1% | 0.1% | 0.0% | 0.2% | -3.1% |
| Class E | -0.2% | -0.1% | 0.2% | 0.0% | -1.2% | -0.9% |
| Class F | -0.1% | -1.6% | -0.6% | -0.1% | -0.9% | -0.1% |
| Overall | 0.0% | 0.0% | 0.0% | -0.1% | -0.6% | -1.0% |
|  | 0.0% | -0.2% | -0.5% | -0.1% | -0.7% | -1.0% |
| Enc Time[%] | 104% |  |  | 100% |  |  |
| Dec Time[%] | 107% |  |  | 101% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.9% | 0.0% | 0.7% | -0.1% | 0.3% | 1.9% |
| Class C | -0.7% | 0.2% | 0.0% | -0.3% | -1.4% | 0.4% |
| Class D | -0.8% | -0.1% | -1.9% | -0.7% | -0.7% | 1.2% |
| Class E | -0.9% | 0.3% | -0.8% | -0.4% | -0.9% | -0.1% |
| Class F | -0.3% | 0.0% | -1.5% | -0.3% | -0.8% | 0.1% |
| Overall | -0.7% | 0.1% | -0.6% | -0.3% | -0.6% | 0.8% |
|  | -0.7% | 0.2% | -0.5% | -0.3% | -0.4% | 0.6% |
| Enc Time[%] | 100% |  |  | 95% |  |  |
| Dec Time[%] | 101% |  |  | 91% |  |  |

The proposed β table is also evaluated using three sequences (Riverbed, DucksTakeOff and ChinaSpeed) that are agreed among AHG6 participants at LB, QP = 32, 37 and 41. And beta_offset_div2 and tc_offset_div2 are set to 13. It was confirmed that the cross-check results for three sequences also matched to ones by the proponents. The results can be found in the attached spreadsheet, J0181_P1_Test2.xls.

## Proposal-2: Adaptive control on deblocking filter parameters

The second proposal that introduces beta_step_log2 and tc_step_log2 are evaluated only using three sequences (Riverbed, DucksTakeOff and ChinaSpeed) that are agreed among AHG6 participants at LB, QP = 32, 37 and 41. And beta_offset_div2 and tc_offset_div2 are set to 13. It was confirmed that the cross-check results on Proposal 2 also matched to ones by the proponents. The results can be found in the attached spreadsheet, J0181_P2_Test2.xls.

# Conclusion

Two proposed methods on deblocking filter parameters in JCTVC-J0181 are verified.

# Reference

- S. Lu, O. Nakagami, M. Ikeda, T. Suzuki, “AHG6: On deblocking filter parameters,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J0181, May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1100, May 2012.