| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0411 |
| --- | --- |

| Title: | Crosscheck of Burst IPCM Coding Signalling (JCTVC-J0153) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gordon Clare (Orange Labs) Félix Henry (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

Contribution JCTVC-J0153 proposes to modify the signalling of Burst IPCM mode. Instead of counting I_PCM blocks, a flag is written after each I_PCM block. The proponents assert that this reduces delay and complexity during encoding. The source code has been studied and appears to match the description of the proponent. The source code has been tested and matches the results provided by the proponent on a specific test sequence.

# Introduction

JCTVC-J0153 proposes that the signaling of IPCM burst mode is modified. Currently, the number of subsequent IPCM blocks is coded with a syntax element. The contribution proposes that a flag is used after each IPCM block in order to reduce encoder delay.

# Compression performance verification

The source code provided by the proponents of JCTVC-J0153 has been compiled and run under the following test conditions:

- JCTVC-I1100 test conditions [1] with intra and PCM activated (anchor)

- JCTVC-I1100 test conditions with intra and proposed PCM activated (measured)

The software used for the first experiment produced the performance recorded in Table 1. The rate-distortion performance matches the one measured by the proponents. There is no difference with the anchor since I_PCM is not selected in common test conditions.

Furthermore, the proposed method has been activated and measured on a specific sequence provided by the proponent, called Sandstorm, were I-PCM blocks are actually selected. The performance measured by the proponent and the crosschecker are presented in table 2 and they match.

Table 1: Rate-distortion performance measured by the crosschecker for intra common test conditions with PCM activated.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0,0% | 0,0% | 0,0% |
| Class B | 0,0% | 0,0% | 0,0% |
| Class C | 0,0% | 0,0% | 0,0% |
| Class D | 0,0% | 0,0% | 0,0% |
| Class E | 0,0% | 0,0% | 0,0% |
| Overall | 0,0% | 0,0% | 0,0% |
|  | 0,0% | 0,0% | 0,0% |
| Class F | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2: Rate-distortion performance measured by the proponent (left) and the crosschecker (right) on a specific sequence with I_PCM selections, the MAIN profiles were cross checked:

| Sand Storm CIF 33 frames |  |  |  |  |  |  | Results from cross-checker |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Bitrate (kbps) |  |  | PSNR-Y(dB) |  |  |  |  |  |  |
| Config | QP | Anchor | J-0153 | %change | Anchor | J-0153 |  |  |  |  |  |
| AI-main | 0 | 37797,78 | 37187,07 | -1,62% | 99,99 | 99,99 | 37797,78 | 37187,07 | -1,62% | 99,99 | 99,99 |
|  | 4 | 37797,78 | 37179,93 | -1,63% | 99,99 | 99,99 | 37797,78 | 37179,93 | -1,63% | 99,99 | 99,99 |
|  | 8 | 37797,78 | 37172,52 | -1,65% | 99,99 | 99,99 | 37797,78 | 37172,52 | -1,65% | 99,99 | 99,99 |
|  | 12 | 37797,54 | 37179,69 | -1,63% | 99,99 | 99,99 | 37797,54 | 37179,69 | -1,63% | 99,99 | 99,99 |
| AI_HE10 | 0 | 37797,78 | 37187,07 | -1,62% | 99,99 | 99,99 | 37797,78 | 37187,07 | -1,62% | 99,99 | 99,99 |
|  | 4 | 37797,78 | 37179,93 | -1,63% | 99,99 | 99,99 | 37797,78 | 37179,93 | -1,63% | 99,99 | 99,99 |
|  | 8 | 37797,78 | 37172,52 | -1,65% | 99,99 | 99,99 | 37797,78 | 37172,52 | -1,65% | 99,99 | 99,99 |
|  | 12 | 37797,78 | 37179,93 | -1,63% | 99,99 | 99,99 | 37797,78 | 37179,93 | -1,63% | 99,99 | 99,99 |
| RA-Main | 0 | 37798,31 | 37245,67 | -1,46% | 99,99 | 99,99 | 37798,31 | 37245,67 | -1,46% | 99,99 | 99,99 |
|  | 4 | 37798,31 | 37241,19 | -1,47% | 99,99 | 99,99 | 37798,31 | 37241,19 | -1,47% | 99,99 | 99,99 |
|  | 8 | 37798,31 | 37238,64 | -1,48% | 99,99 | 99,99 | 37798,31 | 37238,64 | -1,48% | 99,99 | 99,99 |
|  | 12 | 37798,28 | 37231,31 | -1,50% | 99,99 | 99,99 | 37798,28 | 37231,31 | -1,50% | 99,99 | 99,99 |
| RA_HE10 | 0 | 37798,49 | 37245,85 | -1,46% | 99,99 | 99,99 | 37798,49 | 37245,85 | -1,46% | 99,99 | 99,99 |
|  | 4 | 37798,49 | 37241,37 | -1,47% | 99,99 | 99,99 | 37798,49 | 37241,37 | -1,47% | 99,99 | 99,99 |
|  | 8 | 37798,37 | 37238,70 | -1,48% | 99,99 | 99,99 | 37798,37 | 37238,7 | -1,48% | 99,99 | 99,99 |
|  | 12 | 36982,57 | 36596,41 | -1,04% | 78,02 | 78,02 | 36982,57 | 36596,41 | -1,04% | 78,0172 | 78,0172 |
| LB-Main | 0 | 37798,58 | 37263,16 | -1,42% | 99,99 | 99,99 | 37798,58 | 37263,16 | -1,42% | 99,99 | 99,99 |
|  | 4 | 37798,58 | 37261,01 | -1,42% | 99,99 | 99,99 | 37798,58 | 37261,01 | -1,42% | 99,99 | 99,99 |
|  | 8 | 37798,55 | 37257,05 | -1,43% | 99,99 | 99,99 | 37798,55 | 37257,05 | -1,43% | 99,99 | 99,99 |
|  | 12 | 37798,52 | 37248,09 | -1,46% | 99,99 | 99,99 | 37798,52 | 37248,09 | -1,46% | 99,99 | 99,99 |
| LB-HE10 | 0 | 37798,66 | 37263,24 | -1,42% | 99,99 | 99,99 | 37798,66 | 37263,24 | -1,42% | 99,99 | 99,99 |
|  | 4 | 37798,66 | 37261,09 | -1,42% | 99,99 | 99,99 | 37798,66 | 37261,09 | -1,42% | 99,99 | 99,99 |
|  | 8 | 37798,66 | 37257,16 | -1,43% | 99,99 | 99,99 | 37798,66 | 37257,16 | -1,43% | 99,99 | 99,99 |
|  | 12 | 37798,12 | 37247,75 | -1,46% | 99,19 | 99,19 | 37798,12 | 37247,75 | -1,46% | 99,1857 | 99,1857 |

# Source code examination

The source code has been studied and matches the description of the algorithm provided in JCTVC-J0153.

# Bibliography

[1] F. Bossen, “Common test conditions”, JCTVC-I1100, Geneva, May 2012.