| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July | Document: JCTVC-J0412 |
| --- | --- |

| Title: | Cross-check of “AHG4/AHG9: Syntax modifications for tile width constraint” (JCTVC-J0042) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Michael Horowitz Shilin Xu 1450-1055 W. Hastings Street Vancouver, BC V6E 2E9 Canada | Tel: Email: | +1 778.331.3432 x323 michael@ebriskvideo.com |
| Source: | eBrisk Video, Inc. |  |  |

_____________________________

# Abstract

This contribution presents results of a cross-check for "AHG4/AHG9: Syntax modifications for tile width constraint" proposed by MediaTek in contribution JCTVC-J0042. The source code was inspected at a reasonably detailed level and it appeared to be consistent with the text description in JCTVC-J0042. In addition, the source code compiled, built, and executed without problem in a Windows environment.

# Introduction

The technique analyzed in "AHG4/AHG9: Syntax modifications for tile width constraint" adds constraints of tile width in the encoder to ensure that the width of the tile which the decoder receives shall equal or greater than 384 luma samples.  In order to make the constraint of tile width normative, several syntax elements are added in the PPS. According to contribution JCTVC-J0042, the decoder can parse the tile width independently within the PPS.

The balance of this contribution is organized as follows. Section 2 contains cross-check results. Section 3 contains comments on the source code. Section 4 contains comments on the draft text. References are contained in section 5.

# Cross-check results

We found the new syntax elements added in the PPS can constrain all tile width equal or greater than 384 luma samples in all cases thus the decoder design doesn’t need to cover the case of tile width less than 384 luma samples.  There are no BD-rate and run-time reports in contribution JCTVC-J0042.

# Comments on source code

We found the code to be relatively clean and believe it performs the reported functionality. We found that the proposal required relatively minor changes to HM 7.0 (about 23 instances of code modification adding a few code lines for each instance). In addition, while we believe the code performs as reported by the proponent, we note the following:

- The source code introduced a macro named NOT_CONSTRAINED_TILE_HEIGHT, but this macro seems irrelevant with the proposal which only focused on the constraint of tile width.

# Comments on draft text

We found the draft text explicitly explained the new syntax elements proposed by this contribution and integrated them on the top of specification draft 7 in a nice manner.

# References

- B. Bross, W. –J. Han, G. J. Sullivan, J.-R. Ohm, T. Wiegand, “High Efficiency Video Coding (HEVC) text specification draft 7,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1003, May, 2012.

- Chia-Yang Tsai , Chih-Wei Hsu, Yu-Wen Huang, Yung-Chang Chang, Chih-Ming Wang, Chia-Yun Cheng, Shawmin Lei, “AHG4/AHG9: Syntax modifications for tile width constraint” Document of Joint Collaborative Team on Video Coding, JCTVC-J0042, July, 2012.