| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0414 |
| --- | --- |

| Title: | Crosscheck of CU skip and split CABAC context modification for flexible wavefront parallel parsing in JCTVC-J0279 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {cw.hsu, yuwen.huang} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck CU skip and split CABAC context modification for flexible wavefront parallel parsing in JCTVC-J0279. The verification task was done successfully, and the results match exactly those provided by Qualcomm.

# Verification

Qualcomm provided the HM source codes of their proposed codec described in JCTVC- J0279 [1]. The verification is carried out by setting the following macros in the provided code:

#define			WPP_CTX_SPLIT		1

#define			WPP_CTX_SKIP		1

Eight configurations (AI-Main, RA-Main, LB-Main, LP-Main, AI-HE10, RA-HE10, LB-HE10, LP-HE10) were run for the modifications. The running of the tests includes encoding, decoding, and collecting the run time of encoding and decoding processes.

# Results

The BD-rates from our experimental results match exactly those provided by Qualcomm. Moreover, there is no significance difference for encoding time and decoding time while large variations are observed from the excel sheet provided by Qualcomm. The detailed results are listed in JCTVC-J0414.xls.

It is claimed that the proposed modifications can enable flexible parsing for WPP by removing the line buffer of skip and split flags. We also think the proposed method can remove the last two syntax elements of CABAC coding depending on the information from the upper LCUs, which not only saves the line buffers but also better unify and simplify the CABAC design.

# Conclusion

The results presented by Qualcomm in JCTVC-J0279 are confirmed.

# References

- M. Coban, W.-J. Chien,M. Karczewicz , “CU skip and split CABAC context modification for flexible wavefront parallel parsing,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0279, July, 2012.