| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0415 |
| --- | --- |

| Title: | Cross-verification of JCTVC-J0138 NSQT simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Liwei Guo 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the BD-rate results of JCTVC-J0138 which is proposed by Huawei.

# Introduction

JCTVC- J0138 described a scheme for NSQT simplification.

# Experimental results

The codebase is HM 7.0. The coding efficiency is evaluated under common conditions as specified in [1]. The coding efficiency is measured by using 4 point BD-Rate. The anchor is HM7.0+NSQT (for both Main and HE10 settings).

Conclusions  Test results of JCTVC-J0138

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | -0.2% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.2% | 0.2% | 0.0% | 0.2% | 0.1% |
| Class D | 0.1% | 0.3% | 0.2% | 0.0% | 0.4% | 0.2% |
| Class E |  |  |  |  |  |  |
| Class F | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.3% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.2% | 0.1% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.3% | 0.0% | 0.3% | 0.2% |
| Class C | 0.0% | 0.6% | 0.4% | 0.0% | 0.4% | 0.4% |
| Class D | 0.0% | 0.6% | 0.6% | 0.1% | 0.5% | 0.3% |
| Class E | -0.2% | 0.0% | 0.2% | 0.0% | -0.8% | -0.1% |
| Class F | 0.0% | -0.3% | -0.4% | 0.1% | 0.1% | 1.7% |
| Overall | 0.0% | 0.3% | 0.2% | 0.0% | 0.2% | 0.5% |
|  | 0.0% | 0.2% | 0.3% | 0.0% | 0.2% | 0.6% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Conclusions

This contribution reports the cross-verification result of JCTVC-J0138. The BD-rate exactly matches the data reported by the proponent.

# References

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, Geneva, CH, April/May, 2012.