| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0418 |
| --- | --- |

| Title: | AHG6: Cross-check of JCTVC-J0090 Transform Dependent Deblocking Strength |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shuo Lu Ohji Nakagami Sony City Osaki. 2-10-1 Osaki Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2746 Shuo.Lu@jp.sony.com ohji.nakagami@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This document reports the cress-check results of JCTVC-J0090 which proposes to proposes to signal specific deblocking adjustment parameters to control the deblocking strength in case that at least one of two adjacent video blocks P and Q is included in a TU of maximum size by new syntax elements pps_beta_offset_max_tu and pps_tc_offset_max_tu_div2 in PPS and slice_header_beta_offset_max_tu and slice_header_tc_offset_max_tu_div2 in slice header. It is confirmed that source code exactly reflects the proposal. BD-rate matches with what provided by the proposer too.

# Introduction

JCTVC-J0096 proposes to add new syntax elements pps_beta_offset_max_tu and pps_tc_offset_max_tu_div2 in PPS, slice_header_beta_offset_max_tu and slice_header_tc_offset_max_tu_div2 in slice headerr. The additional syntax is applied to control deblocking filter strength for predefined TU size boundaries. Also, new threshold values are proposed to work with TU dependent filter.

Table1. PPS syntax

| … |  |
| --- | --- |
| deblocking_filter_control_present_flag | u(1) |
| if( deblocking_filter_control_present_flag ) { |  |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_disable_deblocking_filter_flag | u(1) |
| if( !pps_disable_deblocking_filter_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| deblocking_max_tu_offset_enabled_flag | u(1) |
| if (deblocking_max_tu_offset_enabled_flag) { |  |
| pps_beta_offset_max_tu | se(v) |
| pps_tc_offset_max_tu_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |

Table2. slice header syntax

| … |  |
| --- | --- |
| if( deblocking_filter_control_present_flag ) { |  |
| if( deblocking_filter_override_enabled_flag ) |  |
| deblocking_filter_override_flag | u(1) |
| if( deblocking_filter_overriding_flag ) { |  |
| slice_header_disable_deblocking_filter_flag | u(1) |
| if( !disable_deblocking_filter_flag ) { |  |
| slice_header_beta_offset_div2 | se(v) |
| slice_header_tc_offset_div2 | se(v) |
| if (deblocking_max_tu_offset_enabled_flag) { |  |
| slice_header_beta_offset_max_tu | se(v) |
| slice_header_tc_offset_max_tu_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |

# Objective Results

## Results based on HM7.0

Test Configurations:

The configuration parameters are set as follows:

DeblockingFilterControlPresent	       : 1

LoopFilterOffsetInAPS         	: 0

LoopFilterDisable             	: 0

LoopFilterBetaOffset_div2     	: 0

LoopFilterTcOffset_div2       	: 0

LoopFilterOffsetMaxTUEnabled      : 1

LoopFilterBetaOffsetMaxTU         : 26

LoopFilterTcOffsetMaxTUDiv2	       : 13

Table3. Objective results with QuadtreeTULog2MaxSize = 5

| Sequences | QP | Bitrate (Kbps) | Y-PSNR (dB) | Cb-PSNR (dB) | Cr-PSNR (dB) |
| --- | --- | --- | --- | --- | --- |
| Riverbed | 32 | 5853.81 | 34.63 | 39.03 | 41.44 |
|  | 37 | 2854.03 | 31.71 | 38.13 | 40.90 |
|  | 41 | 1554.13 | 29.53 | 37.78 | 40.61 |
| DucksTakeOff | 32 | 18569.44 | 30.33 | 32.63 | 36.60 |
|  | 37 | 7591.72 | 27.73 | 31.56 | 35.97 |
|  | 41 | 3714.33 | 25.98 | 30.95 | 35.67 |
| ChinaSpeed | 32 | 1465.74 | 34.96 | 40.11 | 39.70 |
|  | 37 | 676.87 | 31.54 | 38.60 | 37.66 |
|  | 41 | 346.87 | 29.05 | 37.68 | 36.30 |
| WestWindEasy | 32 | 3746.05 | 36.19 | 44.79 | 47.57 |
|  | 37 | 1669.72 | 33.00 | 43.67 | 46.65 |
|  | 41 | 852.94 | 30.77 | 43.04 | 46.06 |

Table4. Objective results with QuadtreeTULog2MaxSize = 4

| Sequences | QP | Bitrate (Kbps) | Y-PSNR (dB) | Cb-PSNR (dB) | Cr-PSNR (dB) |
| --- | --- | --- | --- | --- | --- |
| Riverbed | 32 | 5974.05 | 34.37 | 39.04 | 41.28 |
|  | 37 | 2927.87 | 31.50 | 38.20 | 40.63 |
|  | 41 | 1606.81 | 29.36 | 37.77 | 40.20 |
| DucksTakeOff | 32 | 18862.21 | 30.15 | 32.60 | 36.63 |
|  | 37 | 7808.201 | 27.59 | 31.52 | 35.98 |
|  | 41 | 3870.22 | 25.85 | 30.88 | 35.61 |
| ChinaSpeed | 32 | 1482.32 | 34.89 | 40.02 | 39.63 |
|  | 37 | 682.08 | 31.48 | 38.45 | 37.60 |
|  | 41 | 350.71 | 28.95 | 37.52 | 36.31 |
| WestWindEasy | 32 | 3762.94 | 36.01 | 44.75 | 47.16 |
|  | 37 | 1687.40 | 32.88 | 43.46 | 45.82 |
|  | 41 | 863.97 | 30.65 | 42.72 | 45.13 |

Table5. Objective results with QuadtreeTULog2MaxSize = 3

| Sequences | QP | Bitrate (Kbps) | Y-PSNR (dB) | Cb-PSNR (dB) | Cr-PSNR (dB) |
| --- | --- | --- | --- | --- | --- |
| Riverbed | 32 | 6285.04 | 33.99 | 38.73 | 40.85 |
|  | 37 | 2988.25 | 31.03 | 37.64 | 40.35 |
|  | 41 | 1596.20 | 28.81 | 37.20 | 40.04 |
| DucksTakeOff | 32 | 19752.04 | 29.86 | 32.53 | 36.55 |
|  | 37 | 8276.92 | 27.33 | 31.38 | 35.81 |
|  | 41 | 4054.41 | 25.57 | 30.67 | 35.45 |
| ChinaSpeed | 32 | 1505.82 | 34.70 | 39.86 | 39.55 |
|  | 37 | 690.91 | 31.28 | 38.23 | 37.51 |
|  | 41 | 353.07 | 28.79 | 37.30 | 36.20 |
| WestWindEasy | 32 | 3812.53 | 35.66 | 44.09 | 46.59 |
|  | 37 | 1698.58 | 32.59 | 43.09 | 45.64 |
|  | 41 | 852.57 | 30.34 | 42.55 | 44.57 |

## Results based on proposed Beta table

The proposed beta values are shown as below:

Beta={ 0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,6,7,8,9,10,11,12,13,14,15,16,17,18,20,22,24,26,28,30,32,34,36,38,40,42,44,60,76,92,108,124,140,159,172,188,204}

Results based on the new beta values are generated with the same configurations as in 2.1.

Table6. Objective results with QuadtreeTULog2MaxSize = 5

| Sequences | QP | Bitrate (Kbps) | Y-PSNR (dB) | Cb-PSNR (dB) | Cr-PSNR (dB) |
| --- | --- | --- | --- | --- | --- |
| Riverbed | 32 | 5852.45 | 34.56 | 39.03 | 41.43 |
|  | 37 | 2852.32 | 31.70 | 38.13 | 40.91 |
|  | 41 | 1552.00 | 29.54 | 37.78 | 40.64 |
| DucksTakeOff | 32 | 18689.27 | 30.17 | 32.63 | 36.59 |
|  | 37 | 7614.43 | 27.67 | 31.56 | 35.97 |
|  | 41 | 3719.16 | 25.95 | 30.94 | 35.66 |
| ChinaSpeed | 32 | 1479.82 | 34.81 | 40.14 | 39.68 |
|  | 37 | 680.54 | 31.43 | 38.61 | 37.67 |
|  | 41 | 348.07 | 28.95 | 37.70 | 36.31 |
| WestWindEasy | 32 | 3747.06 | 36.14 | 44.77 | 47.56 |
|  | 37 | 1667.93 | 33.00 | 43.67 | 46.64 |
|  | 41 | 852.79 | 30.78 | 43.05 | 45.98 |

Table7. Objective results with QuadtreeTULog2MaxSize = 4

| Sequences | QP | Bitrate (Kbps) | Y-PSNR (dB) | Cb-PSNR (dB) | Cr-PSNR (dB) |
| --- | --- | --- | --- | --- | --- |
| Riverbed | 32 | 5971.08 | 34.23 | 39.03 | 41.28 |
|  | 37 | 2926.26 | 31.47 | 38.20 | 40.62 |
|  | 41 | 1604.98 | 29.35 | 37.77 | 40.23 |
| DucksTakeOff | 32 | 19075.96 | 29.86 | 32.60 | 36.63 |
|  | 37 | 7847.11 | 27.48 | 31.52 | 35.97 |
|  | 41 | 3876.26 | 25.80 | 30.88 | 35.61 |
| ChinaSpeed | 32 | 1502.67 | 34.66 | 40.03 | 39.65 |
|  | 37 | 689.47 | 31.32 | 38.44 | 37.60 |
|  | 41 | 350.70 | 28.83 | 37.53 | 36.33 |
| WestWindEasy | 32 | 3765.66 | 35.89 | 44.76 | 47.16 |
|  | 37 | 1685.62 | 32.85 | 43.44 | 45.81 |
|  | 41 | 863.12 | 30.68 | 42.77 | 45.18 |

Table8. Objective results with QuadtreeTULog2MaxSize = 3

| Sequences | QP | Bitrate (Kbps) | Y-PSNR (dB) | Cb-PSNR (dB) | Cr-PSNR (dB) |
| --- | --- | --- | --- | --- | --- |
| Riverbed | 32 | 6288.82 | 33.74 | 38.73 | 40.85 |
|  | 37 | 2981.05 | 30.94 | 37.65 | 40.34 |
|  | 41 | 1592.64 | 28.80 | 37.20 | 40.04 |
| DucksTakeOff | 32 | 20144.60 | 29.38 | 32.53 | 36.55 |
|  | 37 | 8341.11 | 27.13 | 31.38 | 35.81 |
|  | 41 | 4065.25 | 25.48 | 30.67 | 35.45 |
| ChinaSpeed | 32 | 1548.73 | 34.26 | 39.86 | 39.54 |
|  | 37 | 701.35 | 31.00 | 38.24 | 37.50 |
|  | 41 | 354.22 | 28.60 | 37.30 | 36.18 |
| WestWindEasy | 32 | 3833.03 | 35.42 | 44.09 | 46.60 |
|  | 37 | 1697.15 | 32.50 | 43.11 | 45.68 |
|  | 41 | 849.90 | 30.31 | 42.57 | 44.56 |

Note: highlighted data are not included in JCTVC-J0090.

# Reference

- B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, and T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1003, May 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-I1100, May 2012.

[3] Geert Van der Auwera, Rajan Joshi, Marta Karczewicz, “AHG6: Transform Dependent Deblocking Strength”, ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JCTVC-J0090

# Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).