| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 |  |  | Document: JCTVC-J0419 |
| --- | --- | --- | --- |
| Title: | Crosscheck report of On Deblocking Filter and DC Component of Quantization Matrices (JCTVC-J0186) |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the crosscheck result of JCTVC-J0186, proposal of On Deblocking Filter and DC Component of Quantization Matrices by Sony.  It has been verified that the results presented in JCTVC-J0186 are accurate.

# Cross Check Procedure

The source code corresponds to JCTVC-J0186 was provided by Sony, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.

Using the provided software, all the referenced data are generated with the following macro setting:

#define DEBLOCK_SL_DC    0

while all the tested data are generated with the following macro setting:

#define DEBLOCK_SL_DC    1

According to the test description in JCTVC-I1100 (common test condition) with modifications to configuration file to use a quantization matrix file, “scaling_list_symmetry7_rd.txt” which was used in past CE4 activities, the following performance was measured:

- PSNR and bitrate results

# Results

## Simulation results

Cross check results for JCTVC-J0186 are summarized in Table 1, where accompanied “scaling_list_symmetry7_rd.txt” is used as the quantization matrices.

Table 1: BD-rate results for JCTVC-J0186 using scaling_list_symmetry7_rd.txt

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.2% | 0.0% | -0.1% | -0.9% | -1.0% |
| Class B | 0.1% | 0.2% | 0.1% | -0.1% | -0.4% | -0.3% |
| Class C | 0.1% | -0.1% | 0.0% | -0.1% | -0.7% | -0.6% |
| Class D | 0.1% | -0.1% | 0.0% | 0.0% | -0.6% | -0.6% |
| Class E | 0.5% | 0.6% | 0.8% | 0.1% | -0.2% | 0.0% |
| Overall | 0.2% | 0.2% | 0.2% | -0.1% | -0.6% | -0.5% |
|  | 0.2% | 0.1% | 0.2% | -0.1% | -0.6% | -0.5% |
| Class F | 0.1% | -0.1% | 0.0% | 0.0% | -0.6% | -0.7% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | -0.5% | -0.4% | 0.0% | -1.1% | -1.4% |
| Class B | 0.2% | -0.2% | -0.2% | 0.0% | -0.4% | -0.4% |
| Class C | 0.2% | -0.5% | -0.4% | 0.0% | -0.7% | -0.5% |
| Class D | 0.2% | -0.6% | -0.6% | 0.1% | -0.8% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | 0.2% | -0.4% | -0.4% | 0.0% | -0.7% | -0.7% |
|  | 0.2% | -0.4% | -0.4% | 0.0% | -0.7% | -0.7% |
| Class F | 0.1% | -0.5% | -0.3% | 0.0% | -0.8% | -0.8% |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.2% | 0.0% | -0.2% | 0.1% | -0.1% | -0.1% |
| Class C | 0.2% | 0.1% | 0.1% | 0.2% | -0.1% | -0.1% |
| Class D | 0.1% | -0.3% | -0.2% | 0.1% | -0.2% | 0.1% |
| Class E | 0.3% | -0.4% | 0.2% | 0.1% | 0.2% | -0.5% |
| Overall | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% | -0.1% |
|  | 0.2% | -0.1% | 0.0% | 0.1% | -0.1% | -0.1% |
| Class F | 0.1% | -0.6% | -0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

It has been verified that the results of “On Deblocking Filter and DC Component of Quantization Matrices” provided as JCTVC-J0186 are accurate.