| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 |  |  | Document: JCTVC-J0420 |
| --- | --- | --- | --- |
| Title: | Crosscheck report of QM bypass for transform skip mode (JCTVC-J0201) |  |  |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the crosscheck result of JCTVC-J0201, proposal of QM bypass for transform skip mode by ETRI.  It has been verified that the results presented in JCTVC-J0201 are accurate.

# Cross Check Procedure

The source code corresponds to JCTVC-J0201 was provided by ETRI, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.

Using the provided software, all the referenced data are generated with the following macro setting:

#define TRANSSKIP_QM    0

while all the tested data are generated with the following macro setting:

#define TRANSSKIP_QM    1

According to the test description in JCTVC-I1100 (common test condition) with modifications to configuration files to use default quantization matrices and transform skip for all configurations, the following performance was measured:

- PSNR and bitrate results

# Results

## Simulation results

Cross check results for JCTVC-J0201 are summarized in Table 1.

Table 1: BD-rate results for JCTVC-J0201

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | -0.1% | 0.6% | 0.6% | -0.1% | 0.6% | 0.6% |
| Class C | -0.5% | 3.0% | 3.2% | -0.3% | 3.2% | 3.2% |
| Class D | -0.5% | 4.0% | 3.8% | -0.3% | 3.9% | 3.8% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Overall | -0.2% | 1.5% | 1.5% | -0.1% | 1.6% | 1.6% |
|  | -0.2% | 1.5% | 1.5% | -0.1% | 1.5% | 1.5% |
| Class F | -2.8% | 1.0% | -0.7% | -2.6% | 2.2% | 0.6% |
| Enc Time[%] | 106% |  |  | 105% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | -0.4% | -0.3% | 0.0% | -0.1% | -0.4% |
| Class B | 0.2% | 0.6% | 0.4% | 0.1% | 0.4% | 0.3% |
| Class C | -0.7% | 0.5% | 0.6% | -0.7% | 0.7% | 0.9% |
| Class D | -0.4% | 1.6% | 1.1% | -0.5% | 1.5% | 1.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.6% | 0.4% | -0.2% | 0.6% | 0.5% |
|  | -0.2% | 0.5% | 0.4% | -0.2% | 0.6% | 0.4% |
| Class F | -3.6% | -2.6% | -3.9% | -3.6% | -1.3% | -2.6% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.3% | 0.0% | 0.1% | 0.0% |
| Class C | 0.0% | 0.2% | 0.2% | -0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.6% | 0.0% | 0.0% | 0.5% | 0.4% |
| Class E | 0.0% | 0.8% | 0.2% | 0.0% | -0.4% | 0.5% |
| Overall | 0.0% | 0.4% | 0.2% | 0.0% | 0.1% | 0.2% |
|  | 0.0% | 0.4% | 0.2% | 0.0% | 0.1% | 0.1% |
| Class F | -2.1% | -1.2% | -2.2% | -1.9% | 0.1% | -1.9% |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

# Conclusion

It has been verified that the results of “QM bypass for transform skip mode” provided as JCTVC-J0201 are accurate.