| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0421 |
| --- | --- |

| Title: | Cross-check of Removal of the 8x2/2x8 coefficient groups (J0256) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | C. Rosewarne 1 Thomas Holt Drive, North Ryde, NSW 2113, Australia. | Tel: Email: | +61 2 9805 2844 chris.rosewarne@cisra.canon.com.au |
|  | M. Maeda 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan. | Tel: Email: | +81-3-3758-2111 maeda.mitsuru@canon.co.jp |
| Source: | Canon Information Systems Research Australia, Canon Inc. |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of Qualcomm proposal JCTVC-J0256 for ‘Removal of the 8x2/2x8 coefficient groups’.  A second version named ‘M2’ was also provided and simulated.  Both source codes were studied and found to match the description in the contribution document.  The simulation result for all configurations were reported by Qualcomm and tested by Canon.  The cross check result confirms the simulation result reported by Qualcomm.

# Cross Check Procedure

The source code for JCTVC-J0256 [1] was provided by Qualcomm.  Canon compiled the source code using GCC 3.4.6 and the simulation was performed on a Linux cluster. Note that timing results are not comparable due to differences in the simulation environment.

# Analysis

The source code was studied and the following was found:

- The context count for the significance map was reduced from 45 to 42:

  - The last three values from the initialization table (now unused) were removed and no retraining was performed.

- 4x4 sub-blocks are now used for horizontal and vertical scans:

  - Horizontal and vertical scanning is applied within each sub-block.

  - Horizontal and vertical scanning is applied at the coefficient group (CG) level.

- Context selection is modified:

  - Position-based context derivation for 8x8 transform units (TUs) is replaced with the CG template scheme used for larger TUs.

  - Dedicated contexts are allocated for diagonal vs other scan patterns for 8x8 TUs.

  - Context selection within each 4x4 sub-block accords with the patterns presented in [1].

This description accords with [1].

# Coding Performance Measurement

Table 1 and Table 2 provies the cross-check result for JCTVC-J0256 (M1, M2 variants).

Table 1. JCTVC-J0256 simulation results in HM-7.0 (M1 variant).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% | 0.1% | -0.1% | -0.1% |
| Class C | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class E | 0.2% | 0.0% | 0.0% | 0.3% | 0.0% | 0.0% |
| Overall | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | -0.2% | -0.1% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.5% | -0.4% | 0.0% | 0.0% | 0.0% |
| Class B | -0.1% | 0.0% | -0.2% | 0.0% | 0.1% | -0.1% |
| Class C | 0.0% | 0.2% | -0.1% | 0.0% | 0.1% | 0.0% |
| Class D | 0.0% | 0.2% | 0.1% | 0.0% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |
|  | -0.1% | 0.0% | -0.2% | 0.0% | 0.0% | -0.1% |
| Class F | 0.0% | 0.1% | -0.2% | 0.0% | 0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.2% | -0.1% | 0.0% | 0.4% | 0.3% |
| Class C | 0.0% | 0.1% | 0.3% | 0.0% | 0.3% | 0.2% |
| Class D | 0.0% | 0.5% | 0.0% | 0.0% | 0.3% | -0.6% |
| Class E | -0.1% | -0.8% | 0.0% | 0.1% | 0.0% | 0.8% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.3% | 0.1% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.3% | 0.2% |
| Class F | -0.1% | 0.0% | 0.0% | 0.1% | 0.6% | 0.9% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2. JCTVC-J0256 M2 simulation results in HM-7.0 (M2 variant).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% | -0.2% |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class C | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% | 0.0% |
| Class E | 0.2% | 0.0% | 0.0% | 0.2% | 0.1% | 0.1% |
| Overall | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.1% | 0.0% | -0.2% | 0.0% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.5% | -0.2% | -0.1% | 0.4% | 0.4% |
| Class B | -0.1% | 0.1% | 0.0% | 0.0% | 0.2% | 0.0% |
| Class C | -0.1% | 0.3% | 0.2% | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.0% | 0.3% | 0.0% | #VALUE! | #VALUE! | #VALUE! |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | 0.0% | #VALUE! | #VALUE! | #VALUE! |
|  | -0.1% | 0.0% | -0.1% | #VALUE! | #VALUE! | #VALUE! |
| Class F | 0.0% | 0.2% | 0.0% | 0.0% | 0.3% | 0.0% |
| Enc Time[%] | 100% |  |  | #NUM! |  |  |
| Dec Time[%] | 100% |  |  | #NUM! |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.6% | 0.1% | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.0% | 0.3% | 0.3% | 0.1% | 0.2% | 0.3% |
| Class D | 0.0% | 0.2% | 0.1% | 0.0% | 0.6% | 0.2% |
| Class E | -0.1% | 0.3% | -0.4% | 0.3% | -0.2% | 0.6% |
| Overall | 0.0% | 0.4% | 0.0% | #VALUE! | #VALUE! | #VALUE! |
|  | 0.0% | 0.3% | 0.0% | #VALUE! | #VALUE! | #VALUE! |
| Class F | -0.1% | -0.2% | 0.4% | 0.1% | 0.7% | 1.2% |
| Enc Time[%] | 100% |  |  | #NUM! |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Conclusion

This cross-check includes results for JCTVC-J0256 M1 and M2 source codes provided by Qualcomm. The cross-check result confirms the results reported by Qualcomm.

# References

- J. Sole, R. Joshi, M. Karczewicz, “Removal of the 8x2/2x8 coefficient groups”, JCTVC-J0256, 10th JCT-VC Meeting, Stockholm, Sweden, Jul 2012.