| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0424 |
| --- | --- |

| Title: | Cross-check results for JCTVC-J0254 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yinji Piao JeongHoon Park | Email: | yj1003.piao@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of JCTVC-J0254[1] on greater1 flag context derivation. The software implementation is consistent with the algorithm description and the crosscheck results match the results from proponent.

# Code-level check

The proposal was implemented on top of HM7.0. Current context derivation of coeff_abs_level_greater1 flag was replaced by a position-based scheme to remove the dependency in consecutive coeff_abs_level_greater1 flags. 21 modifications were found in source code and a new function was additionally defined to derive the context offset with information of context set, last non-zero position and relative position of coeff_abs_level_greater1 flag in coding order. There is no change in context initialization table although the statistic property would be different.

# Test Results

The tests were carried out under the test conditions specified in the JCTVC-I1100[2]. S/w provided by proponent was compiled on 64-bit Linux and executed on Linux cluster. The summary results are presented in following tables. More details can be found in accompanying excel sheet.

Table I. Test results in HM7.0 under common test condition with RDOQ_on

|  | All Intra Main |  |  | Random Access Main |  |  | Low delay B Main |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |  |  |  |  |  |
| Class A | 0.2% | 0.2% | 0.2% | 0.0% | -0.1% | 0.3% |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.2% | 0.0% | 0.2% | 0.1% | 0.0% | 0.2% | 0.0% |  |  |  |  |  |  |  |  |
| Class C | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 0.0% |  |  |  |  |  |  |  |  |
| Class D | 0.1% | 0.2% | 0.2% | 0.1% | 0.2% | 0.0% | 0.1% | 0.8% | -0.1% |  |  |  |  |  |  |  |  |
| Class E | 0.2% | 0.2% | 0.2% |  |  |  | 0.0% | 0.1% | 0.4% |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.3% | 0.1% |  |  |  |  |  |  |  |  |
|  | 0.1% | 0.2% | 0.2% | 0.1% | 0.1% | 0.1% | 0.0% | 0.3% | 0.0% |  |  |  |  |  |  |  |  |
| Class F | 0.3% | 0.4% | 0.5% | 0.3% | 0.3% | 0.2% | 0.2% | 0.6% | 0.3% |  |  |  |  |  |  |  |  |
| Enc Time | 107% |  |  | 101% |  |  | 101% |  |  |  |  |  | 101% |  | 101% |  |  |
| Dec Time | 102% |  |  | 98% |  |  | 98% |  |  |  |  |  | 101% |  | 98% |  |  |
|  | All Intra HE10 |  |  | Random Access HE10 |  |  | Low delay B HE10 |  |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |  |  |  |  |  |  |  |  |
| Class A | 0.2% | 0.2% | 0.2% | 0.0% | 0.3% | 0.7% |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.1% | 0.2% | 0.1% | 0.1% | 0.0% | 0.1% | 0.0% | 0.2% | 0.0% |  |  |  |  |  |  |  |  |
| Class C | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% | 0.0% | 0.0% | 0.2% | 0.2% |  |  |  |  |  |  |  |  |
| Class D | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | -0.3% | 0.1% | 0.4% | -0.5% |  |  |  |  |  |  |  |  |
| Class E | 0.2% | 0.2% | 0.2% |  |  |  | 0.2% | -0.2% | -0.1% |  |  |  |  |  |  |  |  |
| Overall | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% | 0.1% | 0.1% | 0.2% | -0.1% |  |  |  |  |  |  |  |  |
|  | 0.1% | 0.2% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.2% | -0.1% |  |  |  |  |  |  |  |  |
| Class F | 0.4% | 0.0% | 0.2% | 0.2% | 0.2% | -0.2% | 0.4% | 0.8% | 0.6% |  |  |  |  |  |  |  |  |
| Enc Time | 107% |  |  | 103% |  |  | 103% |  |  | 103% |  |  |  |  |  | 103% |  |
| Dec Time | 101% |  |  | 99% |  |  | 101% |  |  | 99% |  |  |  |  |  | 101% |  |

# Conclusions

Test result of JCTVC-J0254 is verified in this report. The simulation results in common test condition matched the data provided by the proponents.

# References

- JCT-VC, “Position-based context derivation for coeff_abs_level_greater1_flag coding”, JCTVC-J0254, 10th JCT-VC Meeting, Stockholm, SE, 11-20 July, 2012

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-I1100, 9th JCT-VC Meeting, Geneva, May, 2012