| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0425 |
| --- | --- |

| Title: | Improving parallelization efficiency of WPP using Overlapped Wavefront |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | M. Alvarez-Mesa, V. George, T. Schierl. Fraunhofer HHI Einsteinufer 37 10587, Berlin, Germany C. C. Chi, B. Juurlink Technische Universität Berlin Einsteinufer 17 10587, Berlin, Germany | Tel: Email: Email: | +49 30 31002-227 mauricio.alvarez.mesa@hhi.fraunhofer.de valeri.george@hhi.fraunhofer.de thomas.schierl@hhi.frauhofer.de chi.c.chi@tu-berlin.de b.juurlink@tu-berlin.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this document we present Overlapped Wavefront (OWF) execution which improves the parallelization efficiency of WPP.  For straightforward implementation OWF would require a restriction on the downwards motion vector size. The required motion vector restriction can be applied to the specification of HEVC levels as it has been done before for H.264/AVC. We implemented the OWF approach on a multithreaded decoder and evaluated it on a parallel machine.

# Introduction

Wavefront Parallel Processing (WPP) allows high-level parallelization of HEVC decoding and encoding. WPP consists of processing rows of CTBs in parallel while preserving all dependencies [6]. HEVC draft 7 specifies that WPP should have one entry point for each CTB row, meaning that a number of processors up to the number of CTB rows can work in parallel [2]. For WPP the rate-distortion loss is small compared to a non-parallel/one slice per picture bitstream, because dependencies are not broken.  However, the variable number of parallel CTB rows that appear at the beginning and at the end of each picture results in the WPP roll-on/roll-off delay for a high number of used processors.

# Overlapped Wavefront (OWF)

The roll-on/roll-off delay of wavefront processing for high numbers of used processors can be mitigated by overlapping the execution of consecutive pictures. When a thread has finished a CTB row in the current picture and no more rows are available it can start processing the next picture instead of waiting for the current picture to finish. We call this non-normative technique Overlapped Wavefront (OWF), which may be used to enhance implementation efficiency in parallelism. In Figure 1 we illustrate this method: thread 4 (T4) can start decoding the first CTB row of the next picture when its reference area (in red) has been decoded.

Figure 1 – Overlapped Wavefront. Multiple threads are assigned to decode CTB rows in wavefront order. The first CTB row of the next picture (not shown) can be decoded if its reference area (red part) has been decoded.

OWF requires a change in the decoder implementation. On the implementation side, OWF requires that all the decoding steps leading to the final reference picture are to be performed on a CTB granularity instead of separated picture passes. This means that in addition to the entropy decode, reconstruction, and deblocking filter, the SAO filter and ALF (if it is enabled) must also be performed in the CTB decoding loop.

On the codec side, the size of the motion vectors need to be restricted to ensure that all the reference area is available. This can be allowed by limiting only the maximum downwards length of the vertical component.

Table 1 shows the level of parallelism using OWF for different values of the motion vector restriction and different resolutions. The second column shows the result for a vertical motion vector restriction of 512 luma samples. This value corresponds to the restriction applied in H.264/AVC for levels higher than 3.0.

| Resolution | Max down. MV 512 | Max down. MV 1/2 PicHeightLuma | Max down. MV 3/8 PicHeightLuma | Max down. MV 1/4 PicHeightLuma |
| --- | --- | --- | --- | --- |
| 1920x1080 | 8 | 8 | 10 | 12 |
| 2560x1600 | 16 | 12 | 15 | 18 |
| 3840x2160 | 25 | 16 | 20 | 25 |

Table 1. Parallelism for OWF with 64x64 CTB rows

On the one hand, limiting the size of the motion vectors reduces the ability of the codec to describe fast motion. On the other hand, reducing the size further would allow more threads to be used.

Assuming a certain value of the motion vector restriction OWF can be implemented by having a number of threads for meaningful parallelism. In this way, OWF is an implementation optimization of WPP for a high number of used processors requiring no normative changes to the codec. Because OWF allows having more than one picture ‘in flight’, the decoder implementation needs an extra entry in the decoded picture buffer. This will result in more buffering that the minimum specified in the Hypothetical Reference Decoder (HRD). A decoder implementation, however, can decide to use OWF for increasing the performance by adding an extra entry in the decoded picture buffer.

# Performance results

We implemented the parallel decoder that supports both WPP and OWF. We executed the parallel code on a real multicore machine and measured the execution time. Average speedups for each resolution are reported.

## Experimental setup

The parallel decoder has been implemented on top of the HM-4.1 reference decoder. The deblocking filter, however, has been ported from HM-6.0. Additional optimizations have been performed for reducing redundant memory buffers and operations. The optimized code is used as the baseline for measuring the parallelization speedup.

Multithreading has been performed using the C++ Boost libraries. For the experiments, sequences encoded with 1 slice per picture are used as the baseline against which speedup will be measured. The system employed to measure performance is a cache-coherent shared memory system with two Intel Xeon Westmere X5680 processors, each with 6 cores. More details of the hardware/software environment are listed in Table 2.

Table 2 - Experimental setup

| System |  | Software |  |
| --- | --- | --- | --- |
| Processor | Intel X5680 | Boost C++ | 1.46.1 |
| architecture | Westmere | Compiler | gcc 4.6.1 |
| Sockets | 2 | Opt. level | -O3 |
| Cores/socket | 6 | Kernel | 3.0.0-16 |
| Frequency | 3.33 GHz | OS | Kubuntu 11.10 |
| L3-cache | 12MB/socket | Measurement | perf, PAPI |
| SMT | disabled | tools | taskset, time |
| TurboBoost | disabled | HM | 4.1-dev (r1527) |

## Encoder configuration and test sequences

The experiments are performed according to [3] using the Random Access High Efficiency configuration. Because parallelization would be required mainly for high definition video we tested only Class A and Class B sequences. Additionally to the common test conditions, two 2160p sequences have been added [4].  Videos has been encoded the HM-4.1 reference encoder.

## Speedup

Figures 8, 9 and 10 show the average speedup of WPP and OWF for 1080p (class B), 1600p (class A) and 2160p (class ´S´) resolutions respectively. The maximum number of threads that can be used depends on the motion vector restriction. In the case of Class B (Figure 8) the vertical lines show the limit on the number of threads imposed by different motion vector restrictions. For class A and class ‘S’, up to 12 threads can be used safely if the motion vectors are restricted to half of the picture height.

For all the resolutions and all the thread numbers OWF has a higher speedup compared to WPP. As the number of processors increases, the roll-on/roll-off delay of WPP becomes more evident.

Figure 1 - Speedup for Class B (1080p)

Figure 2 - Speedup for Class A (1600p)

Figure 3 - Speedup for Class 'S' (2160p)

## Parallelization Efficiency

Table 3 shows the parallelization efficiency of WPP and OWF for different number of cores. 100% efficiency means that the speedup is equal to the number of cores. The baseline for the efficiency calculation is the sequential code.

| Number of threads | 1080p |  | 1600p |  | 2160p |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | WPP | OWF | WPP | OWF | WPP | OWF |
| 1 | 99.76 | 99.91 | 100.93 | 102.07 | 102.15 | 103.41 |
| 2 | 92.53 | 98.82 | 96.20 | 100.76 | 100.40 | 102.60 |
| 3 | 87.87 | 97.51 | 92.10 | 99.39 | 96.97 | 102.04 |
| 4 | 80.38 | 94.78 | 88.32 | 98.36 | 94.47 | 101.04 |
| 5 | 76.05 | 93.39 | 89.25 | 96.34 | 93.19 | 99.74 |
| 6 | 73.54 | 92.34 | 81.22 | 95.05 | 89.90 | 99.61 |
| 7 | 66.39 | 86.89 | 76.84 | 91.29 | 87.14 | 96.22 |
| 8 | 61.88 | 83.08 | 72.56 | 89.33 | 81.49 | 95.43 |
| 9 | 58.22 | 81.77 | 70.50 | 88.46 | 80.18 | 93.85 |
| 10 | 53.67 | 78.27 | 66.65 | 87.94 | 76.07 | 91.94 |
| 11 | 49.61 | 74.58 | 63.52 | 83.51 | 75.56 | 89.31 |
| 12 | 46.17 | 71.07 | 61.21 | 81.23 | 72.09 | 88.86 |

Table 3 – Parallel efficiency in %

# Support for OWF

As mentioned before, supporting OWF only needs a restriction of the size of the vertical motion vector. Table 4 shows a possible specification of such restrictions taking the H.264/AVC level definition as a base and extending it for higher resolutions beyond 1080p. The applied restriction is approximately equal to half of the picture height (in luma samples) for the maximum resolution allowed in each level. In the following, we provide the level limits table with the possible extension for vertical MV component range.

| Level | Max luma sample rate MaxLumaPR (samples/sec) | Max luma picture size MaxLumaFS (samples) | Max bit rate MaxBR (1000 bits/s) | Min Compression Ratio MinCR | MaxDpbSize (picture storage buffers) | Max CPB size (1000 bits) | Vertical MV component range MaxVmvR (luma frame samples) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | 6 | 350 | [-64,+63.75] |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | 6 | 1,000 | [-128,+127.75] |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | 6 | 5,000 | [-256, +255.75] |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 | [-512, +511.75] |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | 6 | 15,000 | [-512, +511.75] |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | 6 | 30,000 | [-512, +511.75] |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | 6 | 30,000 | [-512, +511.75] |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | 6 | 50,000 | [-512, +511.75] |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | 6 | 50,000 | [-512, +511.75] |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | 6 | 100,000 | [-512, +511.75] |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | 6 | 150,000 | [-1024,+1023.75] |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 6 | 300,000 | [-1024,+1023.75] |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 6 | 500,000 | [-2048,+2047.75] |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 6 | 800,000 | [-2048,+2047.75] |

Table 4 – Level limits.

# Conclusions

We presented a non-normative implementation technique, called OWF, which improves the parallelization efficiency of WPP for a high number of used processors. Experimental results, using a parallel decoder, show significant execution time reductions, if a high number of processors is used. For straightforward implementation OWF would require to impose a restriction of the vertical motion vector size. We suggest a restriction of vertical motion vector component ranges, similar to the restrictions applied for H.264/AVC.

# References

- F. Henry and S. Pateux, “Wavefront Parallel Processing”, JCTVC-E196, March 2011.

- Y.-K. Wang, M. Coban and F. Henry, “Wavefront parallel processing simplification”, JCTVC-I0360, April 2012.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, December. 2011.

- SVT High Definition Multiformat Test Set. Sveriges Television, 2006. ftp://vqeg.its.bldrdoc.gov/HDTV/SVT_MultiFormat/SVT_MultiFormat_v10.pdf

# Patent rights declaration(s)

Fraunhofer-Gesellschaft zur Förderung der angewandten Forschung e. V. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Technische Universität Berlin does not have any current or pending patent rights relating to the technology described in this contribution.