| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0426 |
| --- | --- |

| Title: | Crosscheck of Syntax harmonisation of the I_PCM and TransQuantBypass modes (JCTVC-J0168) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gordon Clare (Orange Labs) Félix Henry (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

Contribution JCTVC-J0168 proposes to harmonise the handling of I_PCM and TransQuantBypass in HM7. The proponents assert that only lossless coding is affected with minor losses and reduced encoder complexity.  The source code has been studied and appears to match the description of the proponent. The source code has been tested and matches the results provided by the proponent.

# Introduction

JCTVC-J0168 proposes harmonizations related to I_PCM and TransQuantBypass.  The harmonizations fall into the following broad categories:

- Control activation of I_PCM and TransQuantBypass using similar flags in both the SPS and PPS.

- Control the granularity of TransQuantBypass mode (currently any sized CU) as is currently done for the I_PCM mode.

- Move the pcm_flag signaling from the (current) PU to the CU level, to be consistent with the CU-level transquant_bypass_enable_flag.

# Compression performance verification

The source code provided by the proponents of JCTVC-J0168 has been compiled and run using two “scenarios.”

The first, testing with QP=0 on all sequences:

- JCTVC-I1100 test conditions [1] with TransQuantBypass activated using a single QP=0 (anchor)

- JCTVC-I1100 test conditions with TransQuantBypass activated using a single QP=0, and the proposed software (measured)

The second, testing with QPs 22, 27, 32, 37 on the two sequences SlideEditing and SlideShow:

- JCTVC-I1100 test conditions [1] with TransQuantBypass activated using regions as described in J0168 (anchor)

- JCTVC-I1100 test conditions with TransQuantBypass activated using using regions as described in J0168, and the proposed software (measured)

For scenario 1, the rate distortion and run-time performance match the ones measured by the proponents, as shown in table 1.

Table 1: cross check rate distortion and run-time performance of scenario 1

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A | 2,0 | 2,0 | 0,00% |
| Class B | 2,0 | 2,0 | 0,00% |
| Class C | 1,9 | 1,9 | 0,00% |
| Class D | 1,7 | 1,7 | 0,00% |
| Class E | 2,6 | 2,6 | 0,00% |
| Overall (w/o F) | 2,0 | 2,0 | 0,00% |
| Overall (w/ F) | 2,4 | 2,4 | 0,01% |
| Class F | 4,3 | 4,3 | 0,03% |
| Enc Time[%] | 87% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A | 2,3 | 2,3 | 0,00% |
| Class B | 2,3 | 2,3 | 0,00% |
| Class C | 2,4 | 2,4 | 0,00% |
| Class D | 2,6 | 2,6 | 0,00% |
| Class E |  |  |  |
| Overall (w/o F) | 2,4 | 2,4 | 0,00% |
| Overall (w/ F) | 7,1 | 7,1 | 0,00% |
| Class F | 27,1 | 27,0 | 0,10% |
| Enc Time[%] | 84% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B Main |  |  |
|  | compression ratio |  | Bit-rate saving |
|  | Reference | Tested |  |
| Class A |  |  |  |
| Class B | 2,3 | 2,3 | 0,00% |
| Class C | 2,5 | 2,5 | 0,00% |
| Class D | 2,6 | 2,6 | 0,00% |
| Class E | 3,1 | 3,1 | 0,00% |
| Overall (w/o F) | 2,6 | 2,6 | 0,00% |
| Overall (w/ F) | 11,0 | 11,0 | 0,01% |
| Class F | 45,0 | 44,9 | 0,06% |
| Enc Time[%] | 84% |  |  |
| Dec Time[%] | 100% |  |  |

For scenario 2, the rate distortion performance matches the one measured by the proponents, as shown in table 2.  There were encoder-side complexity reductions consistent with the ones measured by the proponents.

Table 2: cross check rate distortion and run-time performance of scenario 2, only slideshow and slideediting in the “class F” results

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class F | 0,0% | 0,0% | 0,0% |
| Enc Time[%] | 98% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class F | 0,1% | 0,0% | 0,0% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class F | 0,4% | 0,4% | 0,5% |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 100% |  |  |

# Source code examination

The source code has been studied and matches the description of the algorithm provided in JCTVC-J0168.

# Bibliography

[1] F. Bossen, “Common test conditions”, JCTVC-I1100, Geneva, May 2012.