| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0427 |
| --- | --- |

| Title: | Crosscheck of JCTVC-J0337: Fix for ALF Padding Process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | +1-214-4870-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Cross-check results

This document cross-checks JCTVC-J0337 which proposes a fix to the coding block boundary padding process which is leading to additional line buffers being required in ALF when loop filter is not allowed across slice boundary. Source code and proposed text changes were checked and found to match. BD-Rate results matched too. Average BD-Rate results are reported to be 0.0%. when compared to HM-7.0 will following settings:

-          Slice argument is 1500 bytes per slice (--SliceMode=2 --SliceArgument=1500)

-          Loop filter is not allowed across slice boundary (--LFCrossSliceBoundaryFlag=0)