| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0428 |
| --- | --- |

| Title: | AHG10: Mental cross-check of Selective inter-layer prediction signalling (JCTVC-J0239) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This document reports a mental cross-check of the JCTVC-J0239, “Selective inter-layer prediction signalling for HEVC scalable extension.” The contribution document was examined and understood by the cross-checker, and it was confirmed that the proposed draft text was consistent with the proposal.

# Understanding of the problem statement

For scalable extension inter-layer prediction is a useful tool to improve the coding performance. On the other hand, in some cases, inter-layer prediction may provide marginal or no contribution to the coding of a higher layer.

Thus it is assumed that an enhancement layer may not use basement layer slice for prediction. In this case, if a decoder wants to decode an enhancement layer, it is not necessary to decode or even extract bitstream of an associated slice in the base layer. If a mechanism which enables to discard unnecessary base layer stream, the bandwidth between the bitstream extractor and the decoding device can be significantly reduced.

# Understanding of the proposal

To realize the mechanism stated above, JCTVC-J0239 [1] proposes a flag, "nal_sip_flag", to be added to NAL unit header.

If nal_sip_flag indicates that a NAL unit contains a slice which is not used by the decoding of an enhancement layer, this NAL unit can be discarded before extracting the bitstream.

# Draft text check

The proposed draft text just adding one bit for the indicator, and one sentence describing the semantics of the proposed indicator. It was confirmed that the description is simple and clear, and it is consistent with the proposal.

# References

- Jizheng Xu, “AHG10: Selective inter-layer prediction signalling for HEVC scalable extension,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0239, Stockholm, SE, July 2012