| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0431 |
| --- | --- |

| Title: | Cross-check of “Sign data hiding simplification” (JCTVC-J0099) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Andrea Gabriellini Marta Mrak Matteo Naccari | Email: | andrea.gabriellini@bbc.co.uk marta.mrak@bbc.co.uk matteo.naccari@bbc.co.uk |
| Source: | BBC R&D |  |  |

_____________________________

# Abstract

This document reports the cross-check of the method 2 from proposal JCTVC-J0099 on Sign data hiding simplification. HEVC enables reduction of the number of coded signs by using sign data hiding (SDH) method. Instead of the application of the hiding criterion once every 16 coefficients, the cross-checked method applies a new criterion only once per transform unit. Additionally, it does not need to keep track of the position of the first and last non-zero coefficients in the coefficient group.

The software provided by the proponent was examined and tests were performed for common test conditions and for RDOQ "off" case. The checks match proposed method and provided results. The text changes are consistent with the described simplifications.

This document also reports the cross-check of an additional, encoder-only, technique applied to the RDOQ function. The results obtained during the course of this cross-check perfectly match those published by the proponents of JCTVC-J0099.

# Verification results

The performance of the method 2 from JCTVC-J0099 is compared to HM7.0 anchor in terms of Bjøntegaard Delta (BD) bit-rate saving. Table 1 and 2 show the results for common conditions and for RDOQ "off", respectively.

BD-rate results for both cases match the results provided by the proponent. This simplification is unlikely to change the run times of encoder and decoder. This is confirmed by our results for RDOQ "on" case. At the time of writing of this report, the run-times for RDOQ "off" case cannot be confirmed, although such run times should be close to the anchor's run times.

Table 1: Cross-check of "Once per TU sign hiding criterion" under common test conditions

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class D | -0.1% | -0.1% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class E | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | -0.2% | -0.3% | 0.2% | -0.1% | 0.2% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 105% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.1% | 0.2% | 0.0% | 0.7% | 0.6% |
| Class B | 0.0% | 0.3% | 0.0% | 0.1% | 0.3% | 0.2% |
| Class C | -0.1% | 0.1% | 0.0% | 0.0% | 0.3% | 0.2% |
| Class D | -0.1% | 0.3% | -0.2% | 0.0% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.2% | 0.0% | 0.0% | 0.3% | 0.3% |
|  | 0.0% | 0.2% | 0.0% | 0.0% | 0.3% | 0.2% |
| Class F | 0.0% | 0.0% | 0.0% | 0.3% | 0.4% | 0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | 0.8% | 0.8% | 0.1% | 1.0% | 0.2% |
| Class C | -0.1% | 0.3% | 0.3% | 0.1% | 0.2% | 0.3% |
| Class D | -0.1% | 1.0% | 0.6% | 0.1% | 0.7% | 0.2% |
| Class E | -0.1% | -0.1% | 0.3% | 0.2% | -1.0% | -1.3% |
| Overall | -0.1% | 0.5% | 0.5% | 0.1% | 0.3% | -0.1% |
|  | -0.1% | 0.4% | 0.5% | 0.1% | 0.4% | 0.1% |
| Class F | -0.2% | -0.4% | 0.1% | 0.2% | 1.1% | 0.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 2: Cross-check of "Once per TU sign hiding criterion" for RDOQ "off" case

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.07% | 0.05% | -0.05% | -0.17% | 0.24% | 0.11% |
| Class B | -0.14% | 0.21% | 0.22% | -0.18% | 0.29% | 0.26% |
| Class C | -0.13% | 0.02% | 0.05% | -0.12% | 0.06% | 0.05% |
| Class D | -0.11% | 0.11% | 0.11% | -0.11% | 0.18% | 0.09% |
| Class E | -0.09% | 0.09% | 0.03% | -0.11% | 0.12% | 0.07% |
| Overall | -0.11% | 0.11% | 0.08% | -0.14% | 0.19% | 0.13% |
|  | -0.11% | 0.10% | 0.08% | -0.14% | 0.19% | 0.13% |
| Class F | -0.08% | 0.08% | -0.02% | 0.12% | 0.07% | 0.25% |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.14% | 0.05% | 0.60% | -0.18% | 0.53% | 0.72% |
| Class B | -0.14% | 0.32% | 0.54% | -0.15% | 0.45% | 0.39% |
| Class C | -0.23% | 0.12% | 0.37% | -0.16% | 0.30% | 0.28% |
| Class D | -0.27% | 0.37% | 0.11% | -0.19% | 0.26% | 0.65% |
| Class E |  |  |  |  |  |  |
| Overall | -0.19% | 0.22% | 0.41% | -0.17% | 0.39% | 0.50% |
|  | -0.19% | 0.22% | 0.42% | -0.16% | 0.33% | 0.51% |
| Class F | -0.05% | -0.06% | 0.07% | 0.12% | 0.49% | 0.67% |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.19% | 0.44% | 0.84% | -0.19% | 0.71% | 0.63% |
| Class C | -0.34% | -0.10% | 0.00% | -0.29% | 0.23% | 0.21% |
| Class D | -0.37% | 0.12% | 0.41% | -0.21% | 0.76% | -0.15% |
| Class E | -0.16% | 0.11% | 0.26% | -0.05% | 0.63% | -0.09% |
| Overall | -0.27% | 0.16% | 0.41% | -0.19% | 0.59% | 0.19% |
|  | -0.26% | 0.16% | 0.32% | -0.19% | 0.60% | 0.27% |
| Class F | -0.37% | -0.27% | -0.46% | 0.04% | 0.04% | 0.28% |

# Conclusions

The cross-checked method, i.e. the second method from JCTVC-J0099, proposes a simplification for SDH. The simplification is achieved by checking a SDH criterion only once per TU. Additional benefit is that there is no need to track the first and the last non-zero coefficients in a CG / TU. The software and text modifications match the proposed method. The BD rate changes introduced by this technique are negligible and the computational complexity is not increased. It would therefore be beneficial to adopt this proposal.