| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0435 |
| --- | --- |

| Title: | Lossless Coding and Loop Filter Control for Transform Skip |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera, Rajan Joshi, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 | Email: | geertv@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The contribution JCTVC-J0238 proposes the use of transform skip mode in conjunction with QPY value equal to 4. The problem is that cu_qp_delta is signalled only when a TU with non-zero residual is encountered. Secondly, it is claimed that the deblocking filter is disabled on the lossless samples if QPY value is equal to 4, however, this cannot be guaranteed. This contribution proposes and evaluates two alternatives to mitigate the observed problems. The first proposal (A) is based on JCTVC-J0238 and proposes to define a “slice_transquant_bypass_flag” to control the loop filters if the transform_skip_flag is equal to 1 and QPY is equal to 4, while enforcing that cu_delta_qp shall be signalled at the beginning of the CU or minimum CU quantization group size. The second proposal (B) unifies HM7 “TransQuantBypass” lossless coding with transform skipping to solve the reported issues.

# Problem Statement

The contribution JCTVC-J0238 [4] proposes the use of transform skip mode in conjunction with QPY value equal to 4, which corresponds with quantization step size 1, to support lossless coding and replace the “TransQuantBypass” mode [2].

In the current HEVC text specification [2], the transform_skip_enabled_flag is signalled in the SPS. If it is enabled and if the transform_skip_flag in the residual coding syntax is equal to 1, then the transform is skipped for the 4x4 intra [2] or potentially inter TU [3, 4, and 7].

The “TransQuantBypass” mode, which bypasses transform, quantization, sign hiding, and loop filtering, is enabled at the PPS level through the transquant_bypass_enabled_flag and the cu_transquant_bypass_flag at the CU level. The “TransQuantBypass” mode based on signalling solves the issue that exists with signalling of cu_qp_delta values for setting the QPY value equal to 0 for enabling lossless coding. The problem is that cu_qp_delta is signalled only when a TU with non-zero residual is encountered. Thus, for the TUs that are processed before cu_qp_delta is signalled, predicted value of QP is assumed. If this is different than 0, the decoder applies loop filtering which should be disabled for lossless coding. The same problem exists in contribution [4], which proposes to use QPY value equal to 4 for this purpose, which is also achieved by signalling cu_qp_delta. Secondly, contribution [4] claims that the deblocking filter is disabled on the lossless samples if QPY value is equal to 4, however, this cannot be guaranteed if the QPY of a neighboring CU is large enough so that the average QP, which is used to set the deblocking strength, is larger than value 17.

This contribution proposes and evaluates two alternatives to mitigate the observed problems. The first proposal (A) is based on proposal [4] and the second proposal (B) unifies HM7 “TransQuantBypass” lossless coding with transform skipping.

# Proposal Descriptions

## Proposal A: Lossless Coding Via Transform Skipping & QP=4

This proposal builds upon JCTVC-J0238 [4] and addresses the problem that cu_delta_qp is signalled only when a TU with non-zero residual is encountered. It is proposed to define a “slice_transquant_bypass_flag” and signal it in the slice header.

If the slice_transquant_bypass_flag value is equal to 1, then within the slice, all loop filters (deblocking, SAO, ALF) are bypassed on samples of the 4x4 TUs with transform_skip_flag equal to 1 within the quantization group that has QPY value equal to 4. In addition, if slice_transquant_bypass_flag is equal to 1, then cu_delta_qp shall be signalled at the beginning of the CU or minimum CU quantization group size. The syntax changes are specified in Section 4.1.

Note that the transform_skip_flag is not signalled if the coded block flag (cbf) is equal to 0 for the 4x4 TU [2]. In that case, the loop filters may be applied and the 4x4 block may not be lossless. Therefore, the following alternative solutions for this problem are presented:

- Prop. A.1: If the slice_transquant_bypass_flag is equal to 1 and if QPY value is equal to 4, then the signaling of the transform_skip_flag for 4x4 TUs is enforced even if the coded block flag is equal to 0. The syntax changes are specified in Section 4.1.

- Prop. A.2: If the slice_transquant_bypass_flag is equal to 1, it is enforced that only 4x4 TUs are allowed within a CU (or minimum CU quantization group) that has QPY value equal to 4 and that the transform_skip_flag value of each 4x4 TU shall be equal to 1. If the transform_skip_flag is not present (cbf = 0), then transform_skip_flag value is inferred equal to 1. No syntax table changes are required.

Comparing proposals A.1 and A.2, proposal A.2 requires less syntax change and is therefore recommended if lossless coding through QPY value equal to 4 is considered together with enforcing the signalling of cu_delta_qp.

The following proposal B does not rely on QPY value equal to 4 for enabling lossless coding and is based on the HM7 “TransquantBypass” mode.

## Proposal B: Unification of “TransQuantBypass” Lossless Coding and Transform Skipping

To mitigate the reported problems, it is proposed to signal a flag to indicate whether the quantization, sign hiding and loop filters are bypassed in addition to skipping the transform. This “transform_skip_lossless_flag” can be signaled for a CU (or group of coding units [5] or linked with allowed IPCM block sizes [6]). As an alternative name for the “transform_skip_lossless_flag”, the cu_transquant_bypass_flag name may be reused from [2].

The signalling of the transform_skip_lossless_flag (cu_transquant_bypass_flag) is dependent on a higher-level enable flag, such as transquant_bypass_enabled_flag, which is signalled in the PPS [2].

If the transform_skip_lossless_flag (cu_transquant_bypass_flag) is equal to 1 for a coding unit, the steps of quantization, sign hiding and loop filtering (deblocking, SAO, ALF) are bypassed in addition to the transform for the 4x4 intra (or potentially inter TUs) with transform_skip_flag equal to 1. In this case, the QPY value (predicted QPy,pred + optionally cu_delta_qp) is assigned to the lossless blocks for use by the deblocking filter only for filtering the boundary of the lossless block on the lossy side of the boundary (cfr., IPCM blocks and lossless “TransQuantBypass” mode blocks [2]). In case the transform_skip_lossless_flag (cu_transquant_bypass_flag) is equal to 0, then the QPY and optional cu_delta_qp values shall be used as normal, meaning by the quantization and the deblocking filter.  illustrates this concept.

ts_lossless = 0

ts_lossless = 0

ts_lossless = 1

ts_lossless = 0

ts_lossless = 0

ts_lossless = 1

ts_lossless = 0

ts_lossless = 1

ts_lossless = 1

ts_lossless = 0

CU

Minimum CU group size for signalling ts_lossless

Lossless region consisting of 4x4 TUs with ts_flag = 1

Figure 1 Unification of “TransQuantBypass” lossless coding and transform skipping (“ts_lossless” = “transform_skip_lossless_flag”; “ts_flag” = transform_skip_flag ).

As is the case in proposal A, the transform_skip_flag is not signalled if the coded block flag (cbf) is equal to 0 for the 4x4 TU [2]. In that case, all loop filtering may be applied and the 4x4 block may not be lossless. Therefore, the following alternatives are presented:

- Prop. B.1: If the transform_skip_lossless_flag (cu_transquant_bypass_flag) is equal to 1 for a CU, then the signalling of the transform_skip_flag for 4x4 TUs is enforced even if the coded block flag is equal to 0. The syntax changes are specified in Section 4.1.

- Prop. B.2: If the transform_skip_lossless_flag (cu_transquant_bypass_flag) is equal to 1 for a CU, it is enforced that only 4x4 TUs are allowed within this CU and that the transform_skip_flag value of each 4x4 TU shall be equal to 1. If the transform_skip_flag is not present (cbf = 0), then the transform_skip_flag value is inferred equal to 1. No additional syntax table changes are required.

Proposal B.2 requires fewer syntax changes than proposal B.1 and is therefore recommended if lossless coding through unification of “TransQuantBypass” mode and transform skipping is considered.

# Conclusion

It is recommended to adopt proposal A.2 into the HEVC DIS if lossless coding through QPY value equal to 4 is considered together with enforcing the signalling of cu_delta_qp. If lossless coding through unification of “TransQuantBypass” mode and transform skipping is considered, then it is recommended to adopt proposal B.2.

# HEVC Draft Text

## Proposal A

Syntax changes are based on [2]. Yellow highlights indicate syntax changes for enforcing the signalling of cu_delta_qp, while green highlights indicate syntax changes for enforcing signalling of the transform_skip_flag (Prop. A.1). Blue highlights indicate conditions that are not required for inter transform skipping.

The following describes the semantics of slice_transquant_bypass_flag:

slice_transquant_bypass_flag value equal to 1 means that within the slice, all loop filters (deblocking, SAO, ALF) are bypassed on samples of the 4x4 TUs with transform_skip_flag equal to 1 within the quantization group that has QPY value equal to 4. In addition, if slice_transquant_bypass_flag is equal to 1, then cu_delta_qp shall be signalled at the beginning of the CU or minimum CU quantization group size.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| slice_transquant_bypass_flag | u(1) |
| byte_alignment( ) |  |
| } |  |

| coding_tree( x0, y0, log2CbSize, ctDepth ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( diff_cu_qp_delta_depth > 0 ) && log2CbSize >= Log2MinCUDQPSize ) |  |
| { |  |
| IsCuQpDeltaCoded = 0 |  |
| IsQPY4 = 0 |  |
| } |  |
| … |  |

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| CurrCbAddrTS = MinCbAddrZS[ x0 >> Log2MinCbSize ][ y0 >> Log2MinCbSize ] |  |
| if( transquant_bypass_enable_flag ) { |  |
| cu_transquant_bypass_flag | ae(v) |
| } |  |
| if(slice_transquant_bypass_flag && ( diff_cu_qp_delta_depth > 0 ) && !IsCuQpDeltaCoded) |  |
| { |  |
| cu_qp_delta | ae(v) |
| IsCuQpDeltaCoded = 1 |  |
| if (QP’Y == 4) |  |
| IsQPY4 = 1 |  |
| } |  |
| … |  |

| transform_tree( x0L, y0L, x0C, y0C, xBase, yBase, log2CbSize, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| … |  |
| transform_tree( x0L, y0L, x0C, y0C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1L, y1L, x1C, y1C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x2L, y2L, x2C, y2C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x3L, y3L, x3C, y3C, x0L, y0L, log2CbSize, log2TrafoWidth − 1, log2TrafoHeight − 1, trafoDepth + 1, 3 ) |  |
| } else { |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0L ][ y0L ][ trafoDepth ] | ae(v) |
| if( transform_skip_enabled_flag && (PredMode = = MODE_INTRA) && IsQPY4) |  |
| { |  |
| if ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) |  |
| { |  |
| transform_skip_flag[ x0L ][ y0L ][ 0 ] | ae(v) |
| if (blkIdx == 3) |  |
| { |  |
| transform_skip_flag[ x0C ][ y0C ][ 1 ] | ae(v) |
| transform_skip_flag[ x0C ][ y0C ][ 2 ] | ae(v) |
| } |  |
| } else if ( log2TrafoWidth = = 3) && (log2TrafoHeight = = 3) |  |
| { |  |
| transform_skip_flag[ x0C ][ y0C ][ 1 ] | ae(v) |
| transform_skip_flag[ x0C ][ y0C ][ 2 ] | ae(v) |
| } |  |
| } |  |
| transform_unit (x0L, y0L, x0C, y0C, log2TrafoWidth, log2TrafoHeight, trafoDepth, blkIdx) |  |
| } |  |
| } |  |

| residual_coding( x0, y0, log2TrafoWidth, log2TrafoHeight, scanIdx, cIdx ) { | Descriptor |
| --- | --- |
| if( log2TrafoWidth = = 1 ¦ ¦ log2TrafoHeight = = 1 ) { |  |
| log2TrafoWidth = 2 |  |
| log2TrafoHeight = 2 |  |
| } |  |
| If( transform_skip_enabled_flag && !cu_ transquant_bypass _flag && (PredMode = = MODE_INTRA) && !IsQPY4 && ( log2TrafoWidth = = 2) && (log2TrafoHeight = = 2) ) |  |
| transform_skip_flag[ x0 ][ y0 ][ cIdx ] | ae(v) |
| … |  |

## Proposal B:

Semantics based on [2]

Section 7.4.6

cu_transquant_bypass_flag equal to 1 specifies that the scaling and transform process as specified in subclause Error: Reference source not found and the in-loop filter process as specified in subclause Error: Reference source not found are bypassed if the transform_skip_flag is equal to 1. When cu_transquant_bypass_flag is not present, it is inferred to be equal to 0.

Recommended proposal option B.2 adds:

cu_transquant_bypass_flag equal to 1 enforces that only 4x4 TUs are allowed within the coding unit and that the transform_skip_flag value of each 4x4 TU shall be equal to 1. If the transform_skip_flag is not present (cbf = 0), then the transform_skip_flag value is inferred equal to 1.

The following syntax changes are based on [2]. Yellow highlights indicate syntax changes for the option of defining a minimum “TransQuantBypass” group.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ….. |  |
| slice_granularity | u(2) |
| diff_cu_qp_delta_depth | ue(v) |
| if ( transquant_bypass_enable_flag) |  |
| diff_cu_transquant_bypass_depth | ue(v) |
| …. |  |

| coding_tree( x0, y0, log2CbSize, ctDepth ) { |  |
| --- | --- |
| …. |  |
| if( ( diff_cu_qp_delta_depth > 0 ) && log2CbSize >= Log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| if( transquant_bypass_enable_flag && diff_cu_transquant_bypass_depth > 0 && log2CbSize >= Log2MinCUTransquantSize ) |  |
| IsCuTransquantBypassCoded = 0 |  |
| …. |  |

| coding_unit( x0, y0, log2CbSize ) { | Descriptor |
| --- | --- |
| …. |  |
| if( transquant_bypass_enable_flag && diff_cu_transquant_bypass_depth > 0 && !IsCuTransquantBypassCoded) { |  |
| cu_transquant_bypass_flag | ae(v) |
| IsCuTransquantBypassCoded = 1 |  |
| } |  |
| …. |  |

- References

[1] JCTVC-I0408 Intra transform skipping [C. Lan (Xidian Univ.), J. Xu, G. J. Sullivan, F. Wu (Microsoft)]

[2] B. Bross, W.-J. Han, J.-R. Ohm, G. J. Sullivan, T. Wiegand, “High efficiency video coding (HEVC) text specification draft 7,” 9th JCT-VC Meeting, Geneva, CA, USA, Apr.-May 2012, Doc. JCTVC-I1003_d4.

[3] A. Gabriellini, M. Mrak, D. Flynn, M. Naccari, “Transform Skipping for Inter Predicted Coding Units,” 10th JCT-VC Meeting, Stockholm, Sweden, July 2012, Doc. JCTVC-J0077

[4] C. Lan, J. Xu, D. He, X. Yu, “Lossless coding via transform skipping,” 10th JCT-VC Meeting, Stockholm, Sweden, July 2012, Doc. JCTVC-J0238

[5] W. Gao, M. Jiang, H. Yu, “AHG11: New signalling mechanism for lossless coding,” 10th JCT-VC Meeting, Stockholm, Sweden, July 2012, Doc. JCTVC-J0340

[6] E. Francois, P. Onno, G. Laroche, T. Poirier, M. Shima, “AHG11: Syntax harmonisation of the I_PCM and TransQuantBypass modes,” 10th JCT-VC Meeting, Stockholm, Sweden, July 2012, Doc. JCTVC-J0168

[7] X. Peng, C. Lan, J. Xu, G. J. Sullivan, “Inter transform skipping,” 10th JCT-VC Meeting, Stockholm, Sweden, July 2012, Doc. JCTVC-J0237

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).