| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0442 |
| --- | --- |

| Title: | Summary of discussion and input contributions on constraint on number of bits/bins per LCU |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono NEC Corporation Minhua Zhou Texas Instruments Inc., USA Shevach Riabtsev CSR Rik Allen Altera European Technology Centre, High Wycombe | Tel: Email: | +81 44 431 7588 chono@ct.jp.nec.com +1-214-480-3816 zhou@ti.com sr22@csr.com UK 01494602334 rallen@altera.com |
| Source: | NEC, Texas Instruments Inc, CSR, Altera |  |  |

_____________________________

# Abstract

This report summarizes the discussions on the reflector on constraint on number of bits/bins per LCU between the 9th JCT-VC meeting and the 10th JCT-VC meeting, and lists of the related input contributions.

# Related activities

There were 24 email exchanges on the jctvc reflector relating to constraint on number of bits/bins per LCU. There was no suggestion for considering a sub-LCU bit/bin constraint; the following LCU bit/bin constraints and alternative ones (including no constraint) were suggested:

[LCU bit constraint]

- The bit limit should be a raw size multiplied by some scaling factor (e.g., by 1.3).

- The bit limit should not exceed 20%-30% more than bits required for raw size.

The following opinion was expressed on the LCU bit constraint approach:

- In encoder architectures where CABAC is completely separated from other operations, it is difficult to meet the bit limit. Therefore bin constraint might be helpful for such encoder architectures.

[LCU bin constraint]

- The bin limit should be a raw size multiplied by some scaling factor (e.g., by 1.7).

The following opinions was expressed on LCU bit constraint:

- The LCU size in terms of bins does not cap a bit limit; some serious study might be required.

[Alternative]

- No constraint

- On top of LCU bit/bin constraint, define the maximum number of the violation occurrences per a set of successive LCUs

# Related input contributions

There are 3contributions. Each proposal is summarize below.

JCTVC-J0059 proposes to consider either of the following bit or bin constraint.

- The number of bits generated by an LCU shall not exceed (N>>3)*(N>>3)*( Num16x16LCUSample*BitDepth )*1.25, where 2N is the size of the LCU and Num16x16LCUSample is the number of 16x16 LCU luma and chroma samples.

- The number of bins generated by an LCU shall not exceed (N>>3)*(N>>3)*( Num16x16LCUSample*BitDepth )*1.25*1.2, where 2N is the size of the LCU and Num16x16LCUSample is the number of 16x16 LCU luma and chroma samples.

JCTVC-J0082 proposes to consider the following bit constraint, which is equivalent to +28% of AVC limit for LCU size of 16x16:

- A LCU size of 2Nx2N the number of bits generated by the LCU shall not be greater than (128*BitDepthY + Raw16x16CUBits)* (N>>3)*(N>>3), where Raw16x16CUBits is raw number of bits of a 16x16 CU.

JCTVC-J0151 proposes to consider no constraint.

# Recommendation

Review the related contributions and discuss the appropriate bit/bin constraint as well as its necessity.

# List of related contributions

JCTVC-J0059	AHG4: Suggested constraint on number of bits/bins per LCU

JCTVC-J0082	AHG9: On number of bits per LCU limit

JCTVC-J0151	Study on HEVC profiles and levels