| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0444 |
| --- | --- |

| Title: | Repeat picture output and HRD management |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez, A. K Katti, and H-Y Hwang 5030 Sugarloaf Parkway Lawrenceville GA 30044 USA | Tel: Email: | +1 770 236 6483 rodrigaa@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This contribution proposes signalling that conveys repeated picture output and the corresponding HRD operation when fixed_pic_rate_flag is equal to one.  The signalling is deemed simpler or necessary for proper HRD operation of a coded video sequence (CVS) corresponding to a source with a picture rate lower than the intended fixed picture output rate, such as when the source corresponds to 24 Hz film content.  The proposed technique would also solve potential issues with DPB management when a bitstream transitions from one CVS to another CVS, the no_output_of_prior_pics_flag is not equal to one, and the fixed picture output rate does not change.

Apology

The authors sincerely apologize for the tardiness of this contribution and would greatly appreciate its consideration.

# Introduction

A number of video applications require the bitstream to have a fixed picture rate and a constant clock tick, specified via num_units_in_tick and time_scale, and  defined as tc = (num_units_in_tick ÷ time_scale), to avoid changes in the physical video output signal that would impart a viewing disruption.  These applications require support for CVSes that may correspond to sources with respective picture rates lower than the fixed picture rate intended for output, such as when the source corresponds to 24 Hz film content and the fixed picture rate is 60 Hz.

These applications also require support for a bistream in which any two consecutive CVSes may correspond to two respective sources with different picture rates, such as when the CVS corresponds to a 60 Hertz source and the preceding CVS corresponds to a 24 Hertz source.  Furthermore, to minimize or eliminate the number of blank output pictures across transitions from one CVS to another CVS,  the no_output_of_prior_pics_flag is not equal to one, such as when pic_width_in_luma_samples, pic_height_in_luma_samples, and max_dec_picture_buffering equal the three respectively corresponding values as in the preceding CVS.  Thus, there is need to specify when a picture in the DPB is to be repeated for output and the associated HRD operation.

Three alternatives are presented to signal repeat_pic_output_flag when fixed_pic_rate_flag =1.  When repeat_pic_output_flag is equal to 1, pic_out_idx is present and corresponds to the entry of Table 7.x that specifies picture output repetition pattern for the picture. The corresponding HRD operation is specified.

# Proposal A

If fixed_pic_rate_flag = 1,  pic_output_flag is not present and inferred equal to 1.  Each decoded picture is output at least once, except when PicOutputFlag = 0 for a TFD picture associated with either the preceding BLA picture or the CRA picture that is the first coded picture in the bitstream.

pic_out_idx is proposed as a u(4) but it could be u(3) if deemed three bits is deemed adequate.

Relevant Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if (fixed_pic_rate_flag) { |  |
| repeat_pic_output_flag | u(1) |
| if (repeat_pic_output_flag) |  |
| pic_out_idx | u(4) |
| } |  |
| else if (!fixed_pic_rate_flag) { |  |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| } |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( RapPicFlag ) { |  |
| rap_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |

repeat_pic_output_flag is present if fixed_pic_rate_flag = 1.  repeat_pic_output_flag equal to 1 specifies that pic_out_idx is present.  repeat_pic_output_flag equal to 0 specifies that pic_out_idx is not present.  When repeat_pic_output_flag is not present, its value is inferred to be equal to 0.  repeat_pic_output_flag shall have the same value in all slices of the picture.

pic_out_idx indicates the entry of Table 7.x that specifies the picture output repetition pattern for the picture.  When pic_out_idx is not present, its value is inferred equal to 0.  pic_out_idx shall have the same value in all slices of the picture.

If fixed_pic_rate_flag = 1, output_flag_present_flag in the picture parameter set is irrelevant.  The presence of the output_flag_present_flag in the picture parameter set can be made contingent to fixed_pic_rate_flag = 0 as shown below.

Relevant Picture parameter set RSBP syntax

| if (!fixed_pic_rate_flag) |  |
| --- | --- |
| output_flag_present_flag | u(1) |

# Proposal B

The syntax and semantics of pic_output_flag are independent of fixed_pic_rate_flag.

repeat_pic_output_flag is present only if  fixed_pic_rate_flag = 1 and  pic_output_flag = 1 since a picture that is not output would not require an output repetition specification.

Relevant Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if (fixed_pic_rate_flag && pic_output_flag) { |  |
| repeat_pic_output_flag | u(1) |
| if (repeat_pic_output_flag) |  |
| pic_out_idx | u(4) |
| } |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( RapPicFlag ) { |  |
| rap_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |

repeat_pic_output_flag is present if  fixed_pic_rate_flag = 1 and  pic_output_flag = 1.  repeat_pic_output_flag equal to 1 specifies that pic_out_idx is present.  repeat_pic_output_flag equal to 0 specifies that pic_out_idx is not present.  When repeat_pic_output_flag is not present, its value is inferred to be equal to 0. repeat_pic_output_flag shall have the same value in all slices of the picture.

pic_out_idx indicates the entry of Table 7.x that specifies the picture output repetition pattern for the picture.  When pic_out_idx is not present, its value is inferred equal to 0.  pic_out_idx shall have the same value in all slices of the picture.

# Proposal C

repeat_pic_output_flag is present if  fixed_pic_rate_flag = 1 and  does not depend on the syntax or semantics of pic_output_flag

Relevant Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| first_slice_in_pic_flag | u(1) |
| pic_parameter_set_id | ue(v) |
| if( !first_slice_in_pic_flag ) |  |
| slice_address | u(v) |
| if( dependent_slice_enabled_flag && !first_slice_in_pic_flag ) |  |
| dependent_slice_flag | u(1) |
| if( !dependent_slice_flag ) { |  |
| slice_type | ue(v) |
| if (fixed_pic_rate_flag) { |  |
| repeat_pic_output_flag | u(1) |
| if (repeat_pic_output_flag) |  |
| pic_out_idx | u(4) |
| } |  |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( RapPicFlag ) { |  |
| rap_pic_id | ue(v) |
| no_output_of_prior_pics_flag | u(1) |
| } |  |

repeat_pic_output_flag is present if  fixed_pic_rate_flag = 1.  Otherwise, repeat_pic_output_flag is not present.  repeat_pic_output_flag equal to 1 specifies that pic_out_idx is present.  repeat_pic_output_flag equal to 0 specifies that pic_out_idx is not present.  When repeat_pic_output_flag is not present, its value is inferred to be equal to 0. repeat_pic_output_flag shall have the same value in all slices of the picture.

pic_out_idx indicates the entry of Table 7.x that specifies the picture output repetition pattern for the picture.  When pic_out_idx is not present, its value is inferred equal to 0.  pic_out_idx shall have the same value in all slices of the picture.

- Syntax and Semantics of “Repeat picture output” table

- If fixed_pic_rate_flag =1, a picture is output at each successive clock tick, tc.   The output of some pictures may need to be repeated, such as when the rate of the coded pictures in the CVS or bitstream is less than the fixed picture rate expressed by the number of clock ticks per second.

- Table 7.x -  Repeat picture output (fixed_pic_rate_flag = 1)

| pic_out_idx | Description | Output times of picture n | final_pic_out |
| --- | --- | --- | --- |
| 0 | picture n is output once if picture nn is output at the next clock tick. DiffPicOrderCnt( nn , n ) = tc | PicOrderCntVal (n) | 0 |
| 1 | picture n is output twice in two consecutively clock ticks. DiffPicOrderCnt( nn , n ) = 2* tc | PicOrderCntVal (n), PicOrderCntVal + tc | 1 |
| 2 | picture n is output twice at the first and third of three consecutively clock ticks. picture nn is output at the second of three consecutively clock ticks. DiffPicOrderCnt( nn , n ) = tc DiffPicOrderCnt( nn+1 , n ) = 3* tc | PicOrderCntVal, PicOrderCntVal + ( 2* tc ) | 2 |
| 3 | picture n is output thrice in three consecutively clock ticks. DiffPicOrderCnt( nn , n ) = 3* tc | PicOrderCntVal, PicOrderCntVal + tc PicOrderCntVal + ( 2* tc ) | 2 |
| 4..15 | Reserved |  |  |

VUI  Relevant text

fixed_pic_rate_flag equal to 1 indicates that the temporal distance between the any two consecutive HRD output times of any two consecutive pictures in output order is constrained as follows. fixed_pic_rate_flag equal to 0 indicates that no such constraints apply to the temporal distance between the HRD output times of any two consecutive pictures in output order.

When fixed_pic_rate_flag is not present, it is inferred to be equal to 0.

When fixed_pic_rate_flag is equal to 1 for a coded video sequence containing picture n, the temporal distance between any two consecutive HRD output times shall be equal to tc as specified in Equation C-1. The value computed for to,dpb( n ) as is specified in Equation C- shall be equal to tc as specified in Equation Error: Reference source not found (using the value of tc for the coded video sequence containing picture n) when one or more of the following conditions are true for the following picture nn that is specified for use in Equation C-:

–	picture nn is in the same coded video sequence as picture n.

–	picture nn is in a different coded video sequence and fixed_pic_rate_flag is equal to 1 in the coded video sequence containing picture nn and the value of num_units_in_tick  time_scale is the same for both coded video sequences.

- HRD  Relevant text

The following applies when fixed_pic_rate_flag =1.

C.2.2  Relevant text

If fixed_pic_rate =1, all the decoding units of each access unit are decoded during the same clock tick.

C.3.1  Relevant text

All pictures k in the DPB, for which all of the following conditions are true, are removed from the DPB:

–	picture k is marked as "unused for reference",

–	picture k has PicOutputFlag equal to 0 or marked no longer needed for output its DPB output time is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e., to,dpb( k ) <= tr( m )

When a picture is removed from the DPB, the DPB fullness is decremented by one.

C.3.2  Relevant text

For each picture n that is output and is not the last picture in the bitstream that is output, the final DPB output time of picture n is specified by:

tfi,dpb( n )  =  to,dpb( n ) + FinalClkTick ( n ) *  tc	(C-xx)

where:

to,dpb( n ) is the first DPB output time of picture n,

tfi,dpb( n ) is the final DPB output time of picture n,

FinalClkTick ( n ) is the number of clock ticks added to obtain the final picture output time of picture n, and

FinalClkTick ( n )  =  final_pic_out		(C-xx)

Let nn  indicate the picture in the CVS that follows picture n in output order that has PicOutputFlag equal to 1.  If  DiffPicOrderCnt( nn , n ) is greater  tc  , final_pic_out is specified in table 7.x as the entry corresponding to the value of pic_out_idx.  If pic_out_idx is not present, the value of pic_out_idx for picture n is inferred equal to 0 and final_pic_out equals zero.

The value of Dto,dpb( n ) is defined as follows for all values of pic_out_idx < 4.

If FinalClkTick ( n ) is not equal to 2 :

Dto,dpb( n )  =  to,dpb( nn ) – to,dpb( n )   =   tc * (  1 + final_pic_out )	(C-xx)

Else if FinalClkTick ( n ) is equal to 2 :

Dto,dpb( n )  =  to,dpb( nn ) – to,dpb( n )  =  	tc   ,

Dto,dpb( nn) = to,dpb( nn+1 ) – to,dpb( nn )  = tc  , and

DiffPicOrderCnt( nn+1 , n ) = tc * (  1 + final_pic_out ) =  3 * tc

where nn+1 indicates the picture in the CVS that follows picture nn in output order and has PicOutputFlag equal to 1.

C.3.3  Relevant text

For all pictures k in the DPB, mark picture k “no longer needed for output” if the final output time of picture k, tfi,dpb( k ), as specified by C-xx (in Subclause C.3.2), is less than or equal to the CPB removal time of the first decoding unit (denoted as decoding unit m) of the current picture n; i.e., tfi,dpb( k ) <= tr( m ).

If the final output time of the current output picture p, tfi,dpb( p )  > tr( m ) ,  the CPB removal time of the the first decoding unit in the next picture  is incremented by tc.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).