| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0447 |
| --- | --- |

| Title: | Crosscheck of Simplification of multiple sign bit hiding criterion (JCTVC-J0094) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gordon Clare (Orange Labs) Félix Henry (Orange Labs) | Tel: Email: | +33 2 99 12 45 58 gordon.clare@orange-ftgroup.com felix.henry@orange-ftgroup.com |
| Source: | Orange Labs |  |  |

_____________________________

# Abstract

Contribution JCTVC-J0094 proposes a modification Multiple Sign Bit Hiding (SBH), removing the need to determine the position of the last non-zero coefficient in all CGs.  Additional encoder-only changes were made to RDOQ calculations related to SBH.  The source code has been studied and appears to match the description of the proponent. The source code has been tested and matches the results provided by the proponent.

# Introduction

JCTVC-J0094 proposes to remove the need that the position of the last non-zero coefficient in each CG is determined during SBH.  Instead, the position of the last non-zero coefficient is only determined by the last CG.  For other CGs a threshold operation is performed on the number of non-zero coefficients present in the CG..

In addition, some encoder-only changes to RDOQ related to SBH costing treatment were performed to increase gains.

Only the final code version was tested here, containing all the code modifications mentioned above.

# Compression performance verification

The source code provided by the proponents of JCTVC-J0094 has been compiled and run under the common test conditions[1].  The results match those provided by the proponent.

Table 1: Rate-distortion and run-time performance measured by the crosschecker

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0,07% | -0,13% | -0,12% | -0,26% | -1,03% | -0,90% |
| Class B | -0,10% | -0,14% | -0,13% | -0,30% | -1,02% | -1,05% |
| Class C | -0,12% | -0,23% | -0,21% | -0,27% | -0,69% | -0,75% |
| Class D | -0,11% | -0,18% | -0,24% | -0,25% | -0,74% | -0,60% |
| Class E | -0,13% | -0,03% | 0,02% | -0,18% | -0,85% | -0,90% |
| Overall | -0,11% | -0,15% | -0,15% | -0,26% | -0,87% | -0,85% |
|  | -0,11% | -0,15% | -0,15% | -0,27% | -0,86% | -0,83% |
| Class F | -0,15% | -0,21% | -0,23% | -0,27% | -0,80% | -0,51% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0,10% | -0,33% | -0,15% | -0,27% | -1,21% | -1,27% |
| Class B | -0,07% | -0,04% | -0,15% | -0,36% | -1,10% | -1,02% |
| Class C | -0,08% | -0,02% | -0,05% | -0,41% | -0,67% | -0,74% |
| Class D | -0,04% | 0,25% | -0,11% | -0,43% | -0,76% | -1,07% |
| Class E |  |  |  |  |  |  |
| Overall | -0,07% | -0,03% | -0,12% | -0,37% | -0,95% | -1,03% |
|  | -0,06% | -0,04% | -0,10% | -0,44% | -0,95% | -1,08% |
| Class F | -0,05% | -0,08% | -0,27% | -0,31% | -0,13% | -0,82% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0,09% | 0,07% | 0,26% | -0,64% | -1,13% | -1,29% |
| Class C | -0,06% | -0,01% | -0,07% | -0,67% | -1,17% | -1,06% |
| Class D | -0,12% | 0,20% | 0,03% | -0,73% | -1,08% | -1,02% |
| Class E | -0,10% | 0,32% | -1,45% | -0,30% | -2,03% | -0,73% |
| Overall | -0,09% | 0,13% | -0,20% | -0,61% | -1,30% | -1,06% |
|  | -0,09% | 0,07% | -0,27% | -0,62% | -1,20% | -0,95% |
| Class F | -0,17% | 0,07% | 0,11% | -0,27% | -0,77% | -0,80% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Source code examination

The source code has been studied and matches the description of the algorithm provided in JCTVC-J0094.

# Bibliography

[1] F. Bossen, “Common test conditions”, JCTVC-I1100, Geneva, May 2012.