| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, Sweden, 11– 20 July 2012 | Document: JCTVC-J0450 |
| --- | --- |

| Title: | AhG6: Cross-check for decoupling SAO LCU on/off from SAO type (JCTVC-J0103) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, C.Kim, Y.Piao, Y.Hong Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 3026 1305 {elena_a.alshina, dionisim, yj1003.piao, yoonmi.hong}@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

_____________________________

# Abstract

In this contribution Qulacomm’s amd MediaTek’s proposal on SAO on/off flag and SAO type decoupling is verified. Average 0.3% Luma BD-rate gain is confirm in “main” test.

# Introduction

SAO type in HM7.0 is encoded for each LCU, using 2 ctx models.

The meaning of SAO type in HM7.0 is following:

0 – SAO is not applied in current LCU         ctx_0

1 – EO_0 (vertical template)

2 – EO_1 (horizontal template)

3 – EO_2 (vertical template)                ctx_1

4 – EO_3 (vertical template)

5 – BO (band offset)

Unary code is used for SAO type coding.

This means that actually SAO on/off flag and SAO type are separately coded in HEVC. An order of SAO syntax elements coding in HEVC are:

- SAO merge left

- SAO merge UP

- SAO on/off

- SAO type

- If SAO type is BO the SAO left band position

- SAO Magnitudes

- If SAO type is BO then SAO signs

In verified contribution Qualcomm and MediaTek propose changing this order and not encoding redundant merge left flag:

- SAO on/off

If (SAO on){

- If left LCU uses SAO the SAO merge left

- SAO merge UP

- SAO type

- If SAO type is BO the SAO left band position

- SAO Magnitudes

- If SAO type is BO then SAO signs

}

SAO on/off flag coding is modified in verified proposal. 2 Additional ctxs are added. These ctxs are switched according to SAO on/off flag in left LCU.

SAO type coding use HM7.0 function and ctxs. So in J0103 design EO_0 type use its own ctx model, remaining SAO types shares another ctx. There is no theoretical justification for this separation, we thinks that this is just a bug in implementation. This bug can be resolved if combination with J0104 is tested.

# Test results

We studied s/w provided by proponent and verified that implementation matches with description in [2]. S/w was tested according to [1] in Main AI, RA, LB and LP tests (according to proponents request). Results are summarized in Table 1. They are matched with results provided by proponent.

Table 1. Summary of cross-check test.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.1% | 0.1% |
| Class B | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | 0.2% | 0.2% |
| Class D | 0.0% | 0.2% | 0.3% |
| Class E | 0.0% | 0.3% | 0.4% |
| Overall | 0.0% | 0.2% | 0.2% |
| Class F | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class A | -0.3% | -0.3% | -0.2% |
| Class B | -0.2% | 0.0% | 0.0% |
| Class C | -0.3% | 0.0% | 0.0% |
| Class D | -0.6% | -0.2% | -0.2% |
| Class E |  |  |  |
| Overall | -0.3% | -0.1% | -0.1% |
| Class F | -0.3% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | -0.1% | -0.2% |
| Class C | -0.2% | -0.1% | 0.1% |
| Class D | -0.4% | 0.3% | 0.8% |
| Class E | -0.5% | -0.6% | -0.7% |
| Overall | -0.3% | -0.1% | 0.0% |
| Class F | -0.6% | -0.1% | -0.1% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay P Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.4% | -0.2% | -0.6% |
| Class C | -0.2% | -0.3% | -0.1% |
| Class D | -0.3% | 0.7% | 0.6% |
| Class E | -0.8% | -1.2% | -0.5% |
| Overall | -0.4% | -0.2% | -0.2% |
| Class F | -0.5% | -0.4% | -0.2% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Y | U | V |
| AVG: | -0.26% | -0.05% | -0.01% |

# Conclusion

In verified test 2 ctx models are added to HEVC design.

In averaged verified test provides 0.3% BD-rate gain for Luma without performance drop for Chroma.

Combination with JCTVC-J0104 and JCTVC-J043 is recommended, since it allows separation of context coded and by-pass bins.

# Reference

[1] F. Bossen, “Common HM test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-I1100, May, 2012.

[2] In Suk Chong, Marta Karczewicz, Chih-Wei Hsu, Chih-Ming Fu, Tzu-Der Chuang, Yu-Wen Huang, Shawmin Lei “AHG6: Decoupling SAO LCU on/off from SAO type,” Document of Joint Collaborative Team on Video Coding, JCTVC-J0103, July, 2012.