| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0452 |
| --- | --- |

| Title: | Cross Check of JCTVC-J0267 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera 5775 Morehouse Drive San Diego, CA 92121 | Email: | geertv@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports on cross check results for Sony’s proposal JCTVC-J0267, which proposes an extension of mode dependent transforms to 4x4 chroma TUs. In particular, test 2 results have been verified for solutions A and B in the proposal. The BD-rate results match those reported by the proponent and encoder/decoder output corresponds. The HEVC draft changes match the solution descriptions.

# Proposal Summary

Two solutions are proposed to extend mode dependent transforms to chroma.

## Solution A

- DST for both horizontal and vertical directions are used for all intra prediction modes of intra 4x4 TUs.

## Solution B

For special modes such as LM and DM in chroma, luma intra prediction mode is used. Then DST-DCT selection shares the same logic as simplification 2. The solution can be described as follows:

- For LM and DM modes of chroma, set the mode as intra prediction mode from luma.

- For Intra_DC mode, DCT for both horizontal and vertical directions are used.

- For all other prediction modes, DST for both horizontal and vertical directions are used.

# Cross Check of Results

Test 2 has been cross checked for solutions A and B. Test 2 follows the common test conditions for All Intra Main with QP=2,7,12,17. Macro settings in the source code are as follows:

Solution A:

#define CE1_SIMPLIFICATIONS          1

#if CE1_SIMPLIFICATIONS

#define SIMPLIFICATION1              1

#define SIMPLIFICATION2              0

#if SIMPLIFICATION1

#define SONY_CHROMA_DST               1

#endif

#endif

Solution B:

#define DST_SIMPLIFICATION2           1

#define SONY_DST_CHROMA               1

## Solution A – Test 2

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | -0.4% | -0.4% |
| Class B | -0.1% | -0.3% | -0.3% |
| Class C | -0.1% | -0.2% | -0.2% |
| Class D | 0.0% | -0.1% | -0.1% |
| Class E | -0.1% | -0.5% | -0.6% |
| Overall | -0.1% | -0.3% | -0.3% |
|  | 0.0% | 0.0% | 0.2% |
| Class F | 0.6% | 0.3% | 0.4% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

## Solution B – Test 2

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1% | -0.5% | -0.5% |
| Class B | -0.1% | -0.5% | -0.5% |
| Class C | -0.1% | -0.4% | -0.5% |
| Class D | -0.1% | -0.5% | -0.5% |
| Class E | -0.2% | -1.1% | -1.3% |
| Class F | 0.3% | -0.2% | -0.3% |
| Overall | -0.1% | -0.5% | -0.6% |
|  | 0.0% | -0.2% | -0.1% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 102% |  |  |