| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0466 |
| --- | --- |

| Title: | Performance of HEVC for Interlaced Video |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ajay Luthra Motorola Mobility David Baylon Motorola Mobility | Tel: Email: Tel: Email: | +1.858.404.3470 aluthra@motorola.com +1.858.404.2285 david.baylon@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

The performance of HEVC for interlaced scanned video is characterized. For the interlaced video sequences tested, the results show that when compressed in field mode the performance of HEVC can significantly degrade to require as much as 21% more (luma) bits in comparison to AVC for the sequences with smaller motion. Alternatively, if the interlaced video sequences are compressed in frame mode, the performance of HEVC can significantly degrade to require as much as 24% more (luma) bits in comparison to AVC for other sequences which have large motion.  For some sequences the change in the coding efficiency (number of bits required) of HEVC can be as large as 60% depending upon frame or field selection. Therefore, it is important to make a correct decision as to whether interlaced scanned pictures are compressed as fields or frames. It is to be noted that in HEVC frame mode, the intra coded frames in the sequence cannot be split into two fields to be compressed in, for example, an IP field structure. As the motion characteristics in many sequences can change with time, the inability to adapt the frame-field decision in HEVC at the level below the sequence level will therefore impact the performance of HEVC in comparison to AVC for these sequences as well.

# Introduction

Although the new higher resolution video formats, e.g. 4kx2k, are expected to support only progressively scanned pictures, interlaced scanned video is expected to be generated in 1080i and SDTV formats for a long time. Additionally, there is a huge amount existing material that is captured in interlaced format. In this contribution, the performance of HEVC is compared to AVC with interlaced tools (PAFF and MBAFF) turned on.

# Simulation results

Table 1 shows how HEVC performs in the field only modes in comparison to AVC with interlaced tools (PAFF and MBAFF) turned on. A negative value indicates (luma) bit rate savings of HEVC compared to AVC. For HEVC, HM 6.2 was used and for AVC, JM 18.3 was used. HM6.2 was configured using Random Access (Main Profile) settings, and for field coding the input sequences were converted into field sequences. JM 18.3 was used with comparable Random Access settings with High Profile and interlaced tools turned on.  It should be noted that due to issues with enabling a larger GOP size (e.g. 16) with the HM, both the field sequence for HEVC and the frame sequence for AVC had the same GOP size of 8.  However, the IntraPeriod used for the field sequence was 96 while for the frame sequence it was 48 in order to obtain more comparable results. For SDTV resolution, MPEG test sequences known as Mobile and Calendar (300 frames), Trapeze (220 frames), Tempete (220 frames), F1car (220 frames) and Stefan (300 frames) were used. For 1080i resolution, MPEG sequences known as Hockey (150 frames) and Tennis (150 frames) were used.

Table 2 shows how HEVC performs in the frame only mode in comparison to AVC. The configurations for the HM and JM software were the similar to those for Table 1, while IntraPeriod = 48 and GOP = 8 were used.

Table 3 compares HEVC field coding vs. HEVC frame coding. A negative value indicates (luma) bit rate savings of HEVC field coding compared to HEVC frame coding.

Figure 1 shows a cumulative distribution of the number of field pictures in AVC coding of Trapeze (qp27).  The field pictures tend to occur when there is relatively high motion, while frame pictures occur during low motion.  Figure 2 expands Figure 1 for pictures 40-70 and shows that even during intervals of mostly field pictures, there is still a mixture of both frame and field pictures.

| Sequence | HEVC field vs. AVC |
| --- | --- |
| Mobile and Calendar | 21.34% |
| Trapeze | -29.81% |
| Tempete | 5.96% |
| F1car | -35.32% |
| Stefan | -24.53% |
| Hockey | -51.10% |
| Tennis | -16.11% |

Table 1.  Bit rate (luma) performance of HEVC field coding vs. AVC

| Sequence | HEVC frame vs. AVC |
| --- | --- |
| Mobile and Calendar | -22.44% |
| Trapeze | -8.12% |
| Tempete | -25.39% |
| F1car | 13.11% |
| Stefan | 23.13% |
| Hockey | 24.61% |
| Tennis | -12.46% |

Table 2.  Bit rate (luma) performance of HEVC frame coding vs. AVC

| Sequence | HEVC field vs. HEVC frame |
| --- | --- |
| Mobile and Calendar | 57.27% |
| Trapeze | -23.79% |
| Tempete | 42.24% |
| F1car | -42.44% |
| Stefan | -38.26% |
| Hockey | -60.62% |
| Tennis | -2.78% |

Table 3.  Bit rate (luma) performance of HEVC field coding vs. HEVC frame coding

Figure 1.  AVC frame/field picture decisions for Trapeze (qp27)

Figure 2.  AVC frame/field picture decisions for Trapeze (qp27), pictures 40-70

# Discussion

It was shown that the performance of HEVC can vary significantly depending on whether an interlace-scanned sequence is compressed in field or frame mode. Sequences with large motion (e.g. Hockey) can be compressed more efficiently in the field mode. However, for sequences with smaller motion (e.g. Mobile and Calendar, Tempete), the performance of HEVC suffers significantly if they are compressed in field mode. The coding efficiency improves significantly if they are compressed in frame mode. However, it should be noted that in frame mode, where all the pictures in a sequence must be compressed as frame pictures, HEVC cannot compress IDR or I pictures as two fields (where the first field is Intra coded and the second field is coded by using motion prediction). Therefore, HEVC will generate relatively larger intra coded pictures.

Furthermore, it is expected that a typical interlaced sequence will exhibit properties that will change over time depending on motion and content. Even for the sequences that exhibit better overall coding efficiency in field mode, if frame-field decisions are allowed to be made adaptively at the picture level, such as in AVC (see Figures 1 and 2), performance can improve. For Mobile and Calendar, simulations have shown about 15% less (luma) bits with AVC PAFF+MBAFF vs. AVC frame coding only.  Therefore, looking at the large variation in performance based on correct frame-field mode selection, it is expected that the performance of HEVC can improve significantly if it is modified to allow the frame-field decision to be made at least at the picture level to allow adapting to the changes in the motion characteristics in the sequence.

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).