| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0467 |
| --- | --- |

| Title: | Verification of J0045 on SAO chroma processing |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Frank Bossen 3240 Hillview Ave Palo Alto CA 94304 USA | Tel: Email: | +1 650 496 4742 bossen@docomoinnovations.com |
| Source: | DOCOMO Innovations, Inc. |  |  |

_____________________________

# Abstract

Document J0045 proposes to use the same SAO type for both chroma components. RD performance results presented in J0045 are confirmed herein. It is further asserted that the modifications proposed in J0045 can reduce implementation complexity and also provide a minor RD performance benefit in the order of 0.2%.

# Introduction

The SAO inloop filter includes 5 modes of operations. Each of these modes requires a different sample access pattern. Indeed the neighbourhood of samples used to determine the value of a filtered sample is different for each mode.

It is not uncommon to interleave U and V samples for improving efficiency of memory access. For example, such a technique is used in the real-time decoder discussed in [2]. It is thus desirable to have the same memory access pattern for both chroma components to reduce the overhead of accessing samples in memory. Document J0045 [1] proposes to use the same SAO mode for chroma components to align the memory access pattern for both chroma components in the SAO context. The proposed modification thus enables the efficient use of the interleaved memory representation and contributes to reduce the implementation complexity associated with SAO.

# Results

A software patch provided by Samsung was applied to HM 7.0 and tested according to common test conditions [3]. Results provided below show a slight degradation in the chroma components that is more than compensated by a minor improvement in the luma component (considering for example a 6:1:1 weighting of the BD rate numbers).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.2% | 0.2% | 0.0% | 0.2% | 0.3% |
| Class B | -0.1% | 0.3% | 0.4% | 0.0% | 0.1% | 0.3% |
| Class C | -0.1% | 0.3% | 0.4% | 0.0% | 0.1% | 0.3% |
| Class D | -0.1% | 0.0% | 0.2% | 0.0% | 0.0% | 0.1% |
| Class E | -0.1% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| Overall | -0.1% | 0.2% | 0.3% | 0.0% | 0.1% | 0.2% |
|  | -0.1% | 0.2% | 0.3% | 0.0% | 0.1% | 0.2% |
| Class F | -0.1% | 0.3% | 0.4% | -0.1% | 0.2% | 0.3% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | 0.6% | 0.2% | -0.1% | 0.3% | 0.2% |
| Class B | -0.2% | 0.8% | 0.5% | -0.1% | 0.2% | 0.2% |
| Class C | -0.2% | 0.5% | 0.5% | -0.1% | 0.2% | 0.2% |
| Class D | -0.3% | 0.4% | 0.4% | -0.2% | 0.1% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.6% | 0.4% | -0.1% | 0.2% | 0.2% |
|  | -0.2% | 0.6% | 0.4% | -0.1% | 0.2% | 0.2% |
| Class F | -0.2% | 0.4% | 0.5% | -0.1% | 0.1% | 0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.6% | 0.1% | -0.1% | 0.1% | 0.4% |
| Class C | -0.1% | 0.5% | 0.4% | -0.1% | 0.0% | 0.4% |
| Class D | -0.2% | 0.5% | 0.8% | -0.2% | 0.4% | 0.5% |
| Class E | -0.3% | -0.2% | -1.0% | -0.1% | -0.5% | -0.1% |
| Overall | -0.2% | 0.4% | 0.1% | -0.1% | 0.0% | 0.3% |
|  | -0.2% | 0.4% | 0.2% | -0.1% | 0.1% | 0.4% |
| Class F | -0.4% | 0.0% | 0.4% | -0.1% | -0.4% | 0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |

The “low-delay P” configuration was also tested and yielded more significant improvements.

|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | 0.6% | -0.4% | -0.2% | 0.4% | -0.2% |
| Class C | -0.3% | 0.1% | 0.2% | -0.1% | -0.2% | 0.0% |
| Class D | -0.3% | 0.6% | 0.8% | -0.2% | 0.2% | 0.1% |
| Class E | -0.5% | 0.0% | -0.8% | -0.4% | -0.3% | -0.5% |
| Class F | -0.5% | -0.3% | -0.1% | -0.4% | -0.2% | -0.6% |
| Overall | -0.43% | 0.23% | -0.05% | -0.25% | 0.03% | -0.21% |
|  | -0.43% | 0.21% | -0.01% | -0.25% | 0.01% | -0.13% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |

# References

- E. Alshina et al., “AhG6: On SAO type sharing between U and V components”, JCTVC-J0045

- F. Bossen, “On software complexity: decoding 720p content on a tablet”, JCTVC-J0128

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).