| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11– 20 July 2012 | Document: JCTVC-J0468 |
| --- | --- |

| Title: | Combination of JCTVC-J0069 and JCTVC-J0093 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba, Tomoyuki Yamamoto, Tomohiro Ikai *1 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520, JAPAN Dake He, Jing Wang, Gaelle Martin-Cocher *2 Liwei Guo, Rajan Joshi, Marta Karczewicz *3 5775 Morehouse Drive San Diego, CA 92121 | Tel: Email: Tel: Email: Tel: Email: | +81-43-299-8526 tsukuba.takeshi@sharp.co.jp +1-226-338-5196 {dhe, wangj, gmartincocher}@rim.com +1-858-845-7532 liweig@qualcomm.com |
| Source: | SHARP Corporation *1, Research in Motion Ltd. *2, Qualcomm *3 |  |  |

_____________________________

# Abstract

This contribution proposes combination of two proposals JCTVC-J0069 part 1 and JCTVC-J0093, both of which are relating coefficient coding for transform skipping. The combination reportedly improves luma BD rate by 2.2% for Class F in All Intra HE10 condition, and improves the compression rate by 2.6% , 08% and 0.6% for all test sequences in All Intra, RA and LD Main conditions when the lossless mode is enabled. .

# Introduction

There are some proposals regarding coefficient coding in the block where transform skip is applied. JCTVC-J0069 part 1[1] proposes to use constant context irrespective of the spatial position. JCTVC-J0093 [2] proposes to effectively change scan order by rotating a block of residual samples by 180 degrees. Since these two methods change different parts of coefficient coding, the combination of these two proposals are evaluated both when transform skip is enabled and when the lossless (transquantbypass) mode is enabled.

# Experimental results

## Combination of J0069 and J0093 for transform skip

Both J0093 and J0069 (solution 1) are implemented in the common software, and the combination is tested. Performance of combination of J0093 and J0069, J0093, and J0069 are shown in Table 1, Table 2, and Table 3 respectively. For class F, combination of J0069 and J0093 provides 2.2% BD-rate gain over HM-7.0 [3]. This gain is larger enough compared to the gain by J0093 as well as the gain by J0069.

Table 1: Performance of combination of J0069 and J0093 (ref: HM-7.0)

Table 2: Performance of J0093 (ref: HM-7.0)

Table 3: Performance of J0069 Solution 1 (ref: HM-7.0)

## Combination of J0069 and J0093 for lossless mode (transquantbypass)

Using HM7.0-dev r2461 as the anchor (AHG11 scenario1), results in Table 4 show that the combination of J0093 and J0069 reduces the average compression rates in the (slice-level) lossless mode among all test sequences (Classes A-F) by 2.6%, 0.8%, and 0.6% in AI-Main, RA-Main, LD-Main settings, respectively. Performance of J0093 is shown in Table 5. To enable the frame-level lossless mode, the following encoding options are used:

QP=0

-1 TransquantBypassEnableFlag

-1 CUTransquantBypassFlagValue

Table 4 Compression rates of combination of J0093 and J0069 against HM7.0r2461 in lossless mode

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
| Class A | -2.8% |  |  |
| Class B | -1.7% |  |  |
| Class C | -2.7% |  |  |
| Class D | -3.1% |  |  |
| Class E | -2.5% |  |  |
| Class F | -3.0% |  |  |
| Overall | -2.6% |  |  |
| Enc Time[%] | 109% |  |  |
| Dec Time[%] | 110% |  |  |
|  | Random Access Main |  |  |
| Class A | -0.7% |  |  |
| Class B | -0.4% |  |  |
| Class C | -0.7% |  |  |
| Class D | -0.8% |  |  |
| Class E |  |  |  |
| Class F | -1.7% |  |  |
| Overall | -0.8% |  |  |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 107% |  |  |
|  | Low delay B Main |  |  |
| Class A |  |  |  |
| Class B | -0.3% |  |  |
| Class C | -0.6% |  |  |
| Class D | -0.5% |  |  |
| Class E | -0.3% |  |  |
| Class F | -1.1% |  |  |
| Overall | -0.6% |  |  |
| Enc Time[%] | 110% |  |  |
| Dec Time[%] | 109% |  |  |

Table 5 Compression rates of J0093 against HM7.0r2461 in lossless mode

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
| Class A | -2.5% |  |  |
| Class B | -1.5% |  |  |
| Class C | -2.4% |  |  |
| Class D | -2.7% |  |  |
| Class E | -2.2% |  |  |
| Class F | -2.2% |  |  |
| Overall | -2.2% |  |  |
| Enc Time[%] | 90% |  |  |
| Dec Time[%] | 91% |  |  |
|  | Random Access Main |  |  |
| Class A | -0.5% |  |  |
| Class B | -0.3% |  |  |
| Class C | -0.5% |  |  |
| Class D | -0.5% |  |  |
| Class E |  |  |  |
| Class F | -1.2% |  |  |
| Overall | -0.6% |  |  |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 93% |  |  |
|  | Low delay B Main |  |  |
| Class A |  |  |  |
| Class B | -0.2% |  |  |
| Class C | -0.4% |  |  |
| Class D | -0.3% |  |  |
| Class E | -0.1% |  |  |
| Class F | -0.9% |  |  |
| Overall | -0.4% |  |  |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 97% |  |  |

# Conclusion

In this contribution, coding efficiency of combination of JCTVC-J0069 and JCTVC-J0093 is evaluated. The results show that the combination of JCTVC-J0069 part 1 and JCTVC-J0093 achieves 2.2% BD rate gain for Class F sequences in All Intra HE10 condition compared to HM-7.0, and improves the compression rate by 2.6% for all test sequences in All Intra Main condition when the lossless (transquantbypass) mode is enabled.

Based on these results, it is recommended to adopt the combination of JCTVC-J0069 part 1 and JCTVC-J0093.

# References

[1] T.Tsukuba and T. Yamamoto ” Constant coefficient context for intra transform skipping” JCTVC-J0069, JCT-VC 10th Meeting: Stockholm, SE, July 2012

[2] D.HE, J.Wang, and G. M.-Cocher ” Rotation of Residual Block for Transform Skipping” JCTVC-J0093, JCT-VC 10th Meeting: Stockholm, SE, July 2012

[3] F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-I1100.

# Patent rights declaration(s)

SHARP Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Research In Motion Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).