| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0472 |
| --- | --- |

| Title: | Crosscheck of Syntax for restricting slices and WPP (JCTVC-J0040) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Wei-Jung Chien 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | 1-858-658-3973 mcoban@qualcomm.com wchien@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results for JCTVC-J0040 on Syntax for restricting slices and WPP.  It is confirmed that implementation is consistent with the proposal description and the experiment results are matched with the proponent’s data.

# Introduction

In this contribution [1], the signalling of end_of_slice_flag is changed to remove the redundancy on the signalling of the end_of_slice_flag that is introduced as a result of the restrictions on the behavior of slices and WPP. It also enforces the restrictions on slice and WPP behavior in an explicit way that may eliminate possible non-conformant encoder behavior. In this contribution when WPP is enabled, the end_of_slice_flag of the last slice granularity unit (LCU or CU) of a LCU row is not explicitly coded and is inferred as true if the slice starts in the middle of the LCU row.

# Results

Experimental results based on common test conditions [2] for A/RA/LB-Main configurations on HM-7.0-dev, revision 2458 software.

The configuration settings of encoder are as follow. WaveFrontSynchro and WaveFrontFlush are both set to 1 to enable WPP. SliceMode is set to 1 to enforce maximum number of LCUs in a slice. SliceArgument is set to 4 to represent maximum LCUs per slice. Simulation results are shown in Table 1. The anchor uses the original syntax in the current HEVC, while the test uses the proposed end_of_slice_flag syntax change.

Simulation results are shown in Table 1. The anchor uses the original syntax in the current HEVC, while the test uses the proposed end_of_slice_flag syntax change.

Table 1: HM 7.0-r2458 vs JCTVC-J0040 under multiple slices in a tile condition.

|  | All Intra Main |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Random Access Main |  |  |
|  | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 97% |  |  |
|  | Low delay B Main |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

# References

- C.-W. Hsu, C.-Y. Tsai, Y.-W. Huang, Y.-C. Chang, C.-M. Wang, C.-Y. Cheng, S. Lei, “Syntax for restricting slices and WPP”, JCVTC-J0040, Stockholm, July. 2012.

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC document JCTVC-I1100, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 9th meeting, Geneva, April 2012.