| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0476 |
| --- | --- |

| Title: | Cross-check of JCTVC-J0137 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 44 431 7658 hao@bu.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This document provides a cross-check report for the document of JCTVC-J0137. It is claimed that the author of this document understands what is being proposed in JCTVC-J0137. In summary, JCTVC-J0137 presents three proposals on stream splicing as follows.

- In addition to initial_cpb_removal_delay, cpb_removal_delay and dpb_output_delay are modified in terms of HRD conformance, when TFD pictures associated with a CRA picture are discarded from the bitstream. Alternative values for those syntax elements are embedded into buffering period SEI.

- Editing point SEI message is specified as an indicator of a valid editing point where the continuity of display time is kept even in spliced bitstream, when fixed_pic_rate_flag is equal to 1.

- A flag to indicate whether decoders can continue its decoding and display process through the associated BLA picture without HRD infringement, namely continuous_hrd_flag, is signalled in buffering period SEI message.

It is verified that the proposed text attached with JCTVC-J0137 conforms to those proposals. As for semantics of the proposed text on the third proposal, it should be discussed whether the proposed semantics of continuous_hrd_flag are necessary and sufficient as a solution of the raised issue, if the proposed syntax is adopted. The first proposal is a kind of bug fix to JCTVC-I0277 on HRD modification for the removal of TFD pictures, and therefore it is suggested that the issue should be fixed.

# Study of the first proposal in JCTVC-J0137

The issue raised in Section 2 in JCTVC-J0137 is as follows.

When TFD pictures associated with a CRA picture are discarded from the bitstream, initial_cpb_removal_delay, cpb_removal_delay and dpb_output_delay should be modified in terms of HRD conformance. However, the current specification does not specify the mean to modify the latter two syntax elements.

To fix the issue, a following solution is proposed in JCTVC-J0137.

Difference between original values of cpb_removal_delay and dpb_output_delay and alternative values in the case of the removal of associated TFD pictures is signalled in buffering period SEI message.

It is verified that the proposed text attached with JCTVC-J0137 conforms to the proposal, although there seems still room for improvement. The proposal is a kind of bug fix to JCTVC-I0277 on modification of initial_cpb_removal_delay. It is suggested that the issue should be fixed.

# Study of the second proposal in JCTVC-J0137

The issue raised in Section 3 in JCTVC-J0137 is as follows.

For bitstreams with fixed_pic_rate_flag equal to 1, splicers need to parse multiple AUs in order to find a valid editing point where the continuity of display time is kept even in spliced bitstream.

To fix the issue, a following solution is proposed in JCTVC-J0137.

An SEI message for indicating a valid editing point, namely editing point SEI message, is specified.

It is verified that the proposed text attached with JCTVC-J0137 conforms, in principle, to the proposal. It should be noted that cpb_offset included in the proposed syntax of editing point SEI message is not explained in the problem statement but seems related to the third proposal.

# Study of the third proposal in JCTVC-J0137

The issue raised in Section 4 in JCTVC-J0137 is as follows.

According to the current specification, decoders have to reset its decoding and display process at editing points, since there is no way to assure the validity of HRD parameters of the first picture after the editing point, i.e., of a BLA picture, even though carefully designed splicers can assure the validity.

To fix the issue, a following solution is proposed in JCTVC-J0137.

A flag to indicate whether decoders can continue its decoding and display process through the associated BLA picture without HRD infringement, namely continuous_hrd_flag, is signalled in buffering period SEI message.

It should be noted that it is the responsibility of splicers to actually keep the validity of HRD parameters through the editing point when continuous_hrd_flag is set to 1.

It is verified that the proposed text attached with JCTVC-J0137 conforms, in principle, to the proposal. As for semantics of the proposed text, it should be discussed whether the proposed semantics of continuous_hrd_flag are necessary and sufficient as a solution of the raised issue, if the proposed syntax is adopted.

# Patent rights declaration(s)

NEC Corporation does not have any current or pending patent rights relating to the technology described in this contribution.