| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0479 |
| --- | --- |

| Title: | Mental cross-check of JCTVC-J0249: Decoder parallelism indication |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jill Boyce 433 Hackensack Ave Hackensack, NJ 07601 USA | Tel: Email: | jill@vidyo.com |
| Source: | Vidyo |  |  |

_____________________________

# Abstract

This document provides a mental cross-check for JCTVC-J0249: Decoder parallelism indication.  The contribution has been reviewed and understood by this author.

The checked contribution proposes that a thread indicator syntax element be added to the SPS to represent the degree of spatial parallelism in a coded video sequence, to be used along with profile and level indicators to determine a decoder’s capability to decode a bitstream, considering the decoder’s multi-core capability.  The thread_idc syntax element indicates a constraint on the maximum number of samples in a slice or tile.

# Discussion

A syntax element thread_idc is added to the SPS, and variable is introduced, Threads = (threads_idc / 4) + 1.  Like level_idc, thread_idc does not directly impact the decoding process, but provides a hint about parallelism, by restricting the size of the spatial area that may be contained within a tile or slice.  No additional constraints other than spatial area are imposed.

The variable Threads gives a lower bound for the number of threads (number of parallel decodable parts) but does not specify an upper bound.  If tiles are used, a constraint of Floor( PicSizeLuma / Threads ) is placed on the maximum number of luma samples contained within a tile.  If tiles are not used, the same constraint is placed on the maximum number of luma samples contained with a slice.

The definition of how many luma samples are contained within a slice is: “A luma sample is said to be contained in a slice if the coding block that contains the luma sample is contained in the slice,” with a similar definition for tiles.

When wavefront parallel processing is used, a constraint is placed such that ( 2 * pic_height_in_luma_samples + pic_width_in_luma_samples ) * CtbSize  ≤ Floor( PicSizeLuma / Threads ).