| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0480 |
| --- | --- |

| Title: | Crosscheck of long term reference pictures in SPS (JCTVC-J0116) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | TK Tan | Tel: Email: | +65 9799 3861 tktan@m-sph.com.sg |
| Source: |  |  |  |

_____________________________

# Abstract

This contribution reports the findings of crosscheck performed on the proposal on long term reference pictures in sequence parameter set (SPS), JCTVC-J0116.

# Introduction

JCTVC-J0116 proposes to signal the syntax elements of candidate long-term reference pictures in the sequence parameter set.  When these long term pictures are used, an index lt_idx_sps[ i ] is sent instead of sending the poc_lsb_lt[ i ].  In JCTVC-J0116r1, a second proposal is added where the “used_by_current” is also added to the signal in the SPS.  This is accomplished by doubling the number of entries in the array of candidate long-term reference pictures and duplicating them, such that each long-term reference picture is associated with both cases of “used_by_current” and “not used_by_current”.

## Software

The software package was provided by Qualcomm.  The package includes

- 2 configuration files for the case of 10s_30s and 20s_60s intervals.

- 3 patch files for the cases of the anchor, the first proposed method and the second proposed method.

- The log files for the encoding and decoding.

- A software for creating the long test sequences (4800 and 9600 frames).  This was not used as the sequences were created by simply using the cat command in linux.

## Analysis

The following was done:

- The 3 patch files were applies to the software HM7.0-dev-ahg13 r2455

- In addition, #define PRINT_RPS_INFO                  2 was set to minitor the operations.

- Three executables corresponding to the 3 versions of the patched software were compiled into the “anchor”, “test” and “testr1”.

- The simulation was performed for the 3 executables with the 2 configuration files provided.

- The results tabulated below.

The patched software and the encoder output was analysed to understand what was implemented.  It was observed that 3 long-term picture candidates were used in “test” and 6 long-term picture candidates were used in “testr1”.  At any one time the last picture of the previous interval is added to the RPS of the slice.

It is noted that the design of the software seems to have to purpose of facilitation the counting of the bits related to the proposal.  The comments do not necessarily relate the proposal, but are more general comments.

- The software / common conditions for test 2.6 is hard wired into the source code and is not flexible hence not very helpful for further experimentation.

- The coding of the slices did not actually use the long term reference pictures.

- The long-term picture candidates are specific to the fixed intervals.  In intervals that are not as regular, the number of candidates required may be more than what was experimented with.

## Simulation results

The following simulation results were obtained. It was found that the simulation results for “testr1” matches the results reported in JCTVC-J0116r1.  However, the simulation results for “test” did not match the results reported in JCTVC-J0116 in the first simulation attempt.  However, on the second attempt, with setting #define WRITE_LTIDXSPS_UVLC 0, matching results were obtained.

|  | WD7 | Proposed method 1 | Proposed method 2 |
| --- | --- | --- | --- |
| Test 1: 10s – 30s – 10s – 30s |  |  |  |
| LTRP signalling-related bit-count in SPS | 1 | 30 | 60 |
| LTRP signalling-related bit-count in slice header | 56896 | 36507 | 32939 |
| Total LTRP signalling-related bit-count | 56897 | 36537 | 32999 |
| % reduction |  | 36% | 42% |
| Test 2: 20s – 60s – 20s – 60s |  |  |  |
| LTRP signalling-related bit-count in SPS | 1 | 30 | 60 |
| LTRP signalling-related bit-count in slice header | 114695 | 73906 | 66754 |
| Total LTRP signalling-related bit-count | 114696 | 73936 | 66814 |
| % reduction |  | 36% | 42% |