| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0486 |
| --- | --- |

| Title: | Mental cross-check of On tiles and wavefront parallel processing (JCTVC-J0123) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Michael Horowitz 1450-1055 W. Hastings Street Vancouver, BC V6E 2E9 Canada | Tel: Email: | +1 +1 778.331.3432 x323 michael@ebriskvideo.com |
| Source: | eBrisk Video Inc. |  |  |

_____________________________

# Abstract

This document reports a mental cross-check of JCTVC-J0123, “On tiles and wavefront parallel processing.” The contribution document was examined and understood by the cross-checker.

# The problem statement

The claim is that there is a decoding issue when one or more slice(s) contain multiple tiles that could lead a decoder to misinterpret the position of the next LCU it will process.

# The proposal

JCTVC-J0123 proposes a syntax that allows for combination of parallelism tools (tiles, WPP, and entropy slices). Specific proposed changes include:

- Replace the two-bit tiles_or_entropy_coding_sync_idc in the PPS with a three-bit parallelism_idc, where the first bit indicates the use of tiles, the second bit indicates the use of WPP, and the third bit indicates the use of entropy slices.

- For the combination of tiles and WPP, the proposal mandates that every tile start a new slice when both parallelism tools co-exist in a picture.