| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0487 |
| --- | --- |

| Title: | Scalable Video Coding Signalling in VPS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ajay Luthra Motorola Mobility | Tel: Email: | +1.858.404.3470 aluthra@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

In current HEVC standard’s text (JCT-VC I1003) syntax elements are added, e.g. Video Parameter Set (VPS), that describe how many layers are contained in the bit stream and the dependency indicators to indicate the dependency among various layers. This set up requires that if the layers are re-multiplexed such that a combination of layers that is different than the one in the incoming feed is put together then one has to go inside the video layer and modify it. That can become onerous at the multiplexing layer. Additionally, the current set up also does not allow one to add additional new layer(s) without modifying the existing layers. That may not be possible in some applications where one does not have the capability to modify the already processed and distributed content.  This contribution proposes to add indication(s) in the bitstream at VPS level that indicates the number of layers and their dependencies are unknown or unspecified. That information can be provided by the system layer or externally.

# Introduction

- In current HEVC standard’s text (JCT-VC I1003) syntax elements are added, e.g. Video Parameter Set (VPS), that describe how many layers are contained in the bit stream and the dependency indicators to indicate the dependency among various layers. An example of scalable layers is shown in Figure 1. It shows a scalable system with 5 layers, where S1 and S2 represent two different spatial resolutions, S2 is the higher resolution layer and depends on S1. V1 and V2 are view scalability layers providing depth and/or second eye view’s information for 3DTV applications. V2 may depend on S2 and/or V1. V1 depends on S1. SN1 is the SNR (Signal to Noise Ratio) scalable layer with improved SNR and may depend only on S2.

- Current set up in HEVC poses two problems:

- If an additional layer is added over existing layers then the elementary streams corresponding to the existing stream also need to be modified. This may not be feasible as one may not have access to the existing streams which may already be distributed. As an example, consider Fig.1. A service provider may elect to add the depth information (V1 and/or V2) after the 2D views (S1 and S2 )are already sent and may be stored in the receiving devices.

- In the re-multiplexing and head-grooming setting, the layers may need to be re-multiplexed or pulled apart differently than the incoming bundle. It is desirable to be able to do that at the system layer without requiring modification to the video layer. As an example consider Fig. 1. A service provider may choose to select, re-multiplex and send only S1 and S2 layers out of all the 5 layers being present in the input.

# Proposal

It is proposed that flag(s) and/or indicator(s) (which could be also be in form of special code value of one or more of an element in the syntax) that indicate to video decoder that the number of layers in the scalable streams and their dependencies are not known or unspecified or not available at the video layer. These indicators will convey the message that information is provided at the system layer so that a re-multiplexer can decide how many and which streams are included in a multiplexed bundle of streams without needing to parse and modify video streams.

A mix of such indicators can also be inserted where it is indicated that partial information about number of layers and their dependencies is available and rest is not. For example, it may be indicated that temporal scalability information in VPS is present or some of the information related to other scalability (e.g. 2 layers of spatial scalability) is present but information related to some other layers (e.g. layers corresponding to view scalability) is not available in a bit stream.

Figure 1. An example of scalable layers

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).