| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0488 |
| --- | --- |

| Title: | Cross-verification of JCTVC-J0254: Position-based Context Derivation of coeff_abs_level_greater1_flag Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution verified the BD-rate results of JCTVC-J0254. In J0254, the context assignment of coeff_abs_level_greater1_flag is derived based on scanning order. As a result, all the contexts can be derived in parallel. It is reported that 0.1% average BD-rate loss is observed due to the changes.

# Introduction

JCTVC- J0254 changes the context assignment for coeff_abs_level_greater1_flag. In the current HM7.0, an inter-bins dependency is used for the context derivation of coeff_abs_level_greater1_flag coding. This dependency is remove by changing the context derivation to scanning order based approach.

# Experimental results

The codebase is HM 7.0. The coding efficiency is evaluated under common conditions as specified in [1]. The coding efficiency is measured by using 4 point BD-Rate. The anchor is HM7.0 (for both Main and HE10 settings).

Conclusions  Test results

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.18% | 0.22% | 0.19% | 0.17% | 0.23% | 0.20% |
| Class B | 0.11% | 0.18% | 0.18% | 0.11% | 0.16% | 0.13% |
| Class C | 0.14% | 0.17% | 0.19% | 0.12% | 0.16% | 0.12% |
| Class D | 0.13% | 0.21% | 0.16% | 0.12% | 0.15% | 0.11% |
| Class E | 0.17% | 0.23% | 0.18% | 0.16% | 0.21% | 0.19% |
| Overall | 0.14% | 0.20% | 0.18% | 0.13% | 0.18% | 0.15% |
|  | 0.14% | 0.20% | 0.18% | 0.13% | 0.19% | 0.15% |
| Class F | 0.35% | 0.35% | 0.45% | 0.36% | 0.05% | 0.16% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.15% | 0.28% | 0.03% | 0.33% | 0.71% |
| Class B | 0.05% | 0.19% | 0.08% | 0.06% | 0.01% | 0.08% |
| Class C | 0.09% | 0.22% | 0.14% | 0.09% | 0.22% | 0.03% |
| Class D | 0.12% | 0.20% | 0.03% | 0.10% | 0.09% | -0.30% |
| Class E |  |  |  |  |  |  |
| Overall | 0.06% | 0.12% | 0.13% | 0.07% | 0.16% | 0.13% |
|  | 0.12% | 0.13% | 0.14% | 0.08% | 0.12% | 0.10% |
| Class F | 0.26% | 0.31% | 0.23% | 0.24% | 0.25% | -0.19% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.04% | 0.21% | -0.05% | 0.03% | 0.23% | -0.04% |
| Class C | 0.04% | 0.11% | 0.02% | 0.04% | 0.21% | 0.23% |
| Class D | 0.08% | 0.83% | -0.06% | 0.09% | 0.38% | -0.53% |
| Class E | -0.01% | 0.07% | 0.43% | 0.24% | -0.23% | -0.11% |
| Overall | 0.04% | 0.31% | 0.05% | 0.09% | 0.18% | -0.11% |
|  | 0.04% | 0.29% | 0.05% | 0.09% | 0.21% | -0.11% |
| Class F | 0.15% | 0.63% | 0.34% | 0.43% | 0.84% | 0.60% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

This contribution reports the cross-verification result of JCTVC-J0254. The BD-rate exactly matches the data reported by the proponent.

# References

- F. Bossen, “Common HM test conditions and software reference configurations”, JCTVC-I1100, Geneva, CH, April/May, 2012.