| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0489 WG11 Number: m26129 |
| --- | --- |

| Title: | Cross-check report for reference index coding (JCTVC-J0176) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Seung-Hwan Kim Andrew Segall 5750 NW Pacific Rim Blvd Camas, WA 98607 | Tel: Email: | +1-360-817-8487 kimse@sharplabs.com asegall@sharplabs.com |
| Source: | SHARP |  |  |

_____________________________

# Abstract

This contribution is a report of cross-check result for reference index coding in JCTVC-J0176. The experimental results perfectly match with the one provided by the proponent in the RD performance.

# Source code check

In the contribution JCTVC-J0176 [1], reference index is coded using CABAC bypass coding instead of context based one as shown in Table1 and Table 2.

Table 1 – Values of variable initValue for ref_idx_l0, ref_idx_l1 ctxIdx

| Initialization variable | ref_idx_l0, ref_idx_l1 ctxIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 32 | 43 | 5 |
| initValue | 102 | 118 | 103 | 118 | 118 | 134 |

Table 2 – Syntax elements and associated types of binarization

| ref_idx_l0 | 1 | TU, cMax = num_ref_idx_l0_active_minus1 | 21 | Error: Reference source not found | prefix: 0 suffix: na, (uses Decode Bypass) |
| --- | --- | --- | --- | --- | --- |
|  | 2 |  | 21 | Error: Reference source not found | prefix: 23 suffix: na, (uses Decode Bypass) |
| ref_idx_l1 | 1 | TU, cMax = num_ref_idx_l1_active_minus1 | 21 | Error: Reference source not found | prefix: 0 suffix: na, (uses Decode Bypass) |
|  | 2 |  | 21 | Error: Reference source not found | prefix: 23 suffix: na, (uses Decode Bypass) |

The source code provided by the proponent was confirmed.

# Simulation Results

The software to be tested was provided by the proponent and its performance relative to HM7.0 software was checked under the common test condition described in JCTVC-I1100 [2]. The computing platform is Linux 64bits and the executables were compiled on 64-bit Linux with gcc 4.1.2. Results are provided in the accompanying excel file.

Table 1. Simulation results in HM-7.0 (without bypass concatenation).

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.0% | -0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.1% | 0.0% | 0.1% | 0.1% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.3% |
| Class D | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.1% | -0.3% | 0.1% | -0.3% | 0.1% |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.1% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.1% | 0.2% |
| Class F | -0.1% | 0.2% | 0.1% | 0.1% | 0.4% | 0.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

# Conclusion

The result provided by the proponent was confirmed.

# References

- C. Rosewarne et al, “AHG5: reference index coding”, JCTVC-J0176, JCT-VC 10th Meeting, Stockholm, July, 2012.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-I1100, JCT-VC 9th Meeting, Geneva, May, 2012.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).