| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0494 |
| --- | --- |

| Title: | Crosscheck of JCTVC-J0286: AHG6: Reduction of block artifacts in HEVC for large blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Do-Kyoung Kwon Texas Instruments Inc., USA Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-1479 d-kwon@ti.com +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document provides cross-check of JCTVC-J0286 on reduction of block artifacts in HEVC for large blocks. It is confirmed that the software and the cross-check results matches to ones reported in JCTVC-J0286.

# Introduction

To remove block artifacts at large block size, JCTVC-J0286 [1] proposes to modify intra-prediction filtering, bS derivation, strong filter decision and luma deblocking filter coefficients. It was confirmed that the software provided by the proponent is written as described in JCVC-J0286. Its objective performance is also verified in this document.

# Experimental Results

The proposed method in JCTVC-J0286 is verified with two sets of sequences: common test sequences under common test conditions [2] and AHG6 test sequences at QP = 32, 37 and 41, LB-Main. The cross-check results with common test sequence are shown in Table 1. The results with AHG6 test sequences can be found in the attached spreadsheet, JCVTC-J0494_Test2.xls. It is confirmed that the results match to ones by the proponent.

Table 1. BD-rate results of JCTVC-J0286 versus HM7.0

|  | All Intra Main |  |  | All Intra HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.2% | 0.2% | 0.3% | 0.5% | 0.4% | 0.4% |
| Class B | 0.1% | 0.2% | 0.1% | 0.5% | 0.2% | 0.1% |
| Class C | 0.1% | 0.0% | 0.1% | 0.2% | 0.1% | 0.1% |
| Class D | 0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.1% |
| Class E | -0.1% | 0.0% | 0.0% | 0.1% | 0.1% | 0.0% |
| Overall | 0.1% | 0.1% | 0.1% | 0.3% | 0.2% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.3% | 0.2% | 0.1% |
| Class F | 0.0% | 0.0% | 0.1% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 94% |  |  | 100% |  |  |
| Dec Time[%] | 93% |  |  | 99% |  |  |
|  | Random Access Main |  |  | Random Access HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.3% | 0.0% | 0.3% | 0.4% | 0.1% |
| Class B | 0.0% | 0.2% | 0.1% | 0.2% | 0.1% | 0.0% |
| Class C | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% | 0.1% |
| Class D | 0.0% | 0.3% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.0% | 0.2% | 0.2% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.2% | 0.2% | 0.0% |
| Class F | 0.0% | 0.1% | 0.1% | 0.1% | 0.2% | 0.0% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.2% | 0.5% | 0.4% | 0.1% |
| Class C | 0.1% | 0.0% | 0.0% | 0.4% | 0.1% | 0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.1% | 0.3% | -0.3% |
| Class E | 0.3% | -0.2% | 0.0% | 1.9% | 0.4% | 1.0% |
| Overall | 0.1% | 0.1% | 0.1% | 0.6% | 0.3% | 0.2% |
|  | 0.1% | 0.1% | 0.1% | 0.6% | 0.4% | 0.3% |
| Class F | 0.1% | 0.2% | 0.7% | 0.3% | 1.0% | 0.8% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |
|  | Low delay P Main |  |  | Low delay P HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.2% | -0.2% | 0.1% | 0.3% | -0.1% |
| Class C | 0.0% | -0.1% | -0.1% | 0.2% | 0.0% | 0.2% |
| Class D | 0.0% | 0.0% | 0.6% | 0.1% | 0.4% | -0.8% |
| Class E | -0.2% | -1.0% | 0.5% | 0.7% | 0.6% | 1.0% |
| Overall | -0.1% | -0.2% | 0.1% | 0.2% | 0.3% | 0.0% |
|  | -0.2% | -0.1% | 0.1% | 0.2% | 0.4% | 0.1% |
| Class F | 0.0% | 0.2% | 0.0% | 0.1% | 0.7% | -0.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |

# Conclusion

The objective performance of JCTVC-J0286 has been verified. Its effect on visual quality needs to be evaluated though.

# Reference

- A. Norkin, “AHG6: Reduction of block artifacts in HEVC for large blocks,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-J0286, July 2012.

- F. Bossen, “Common HM test conditions and software reference configurations,” ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Document JCTVC-I1100, May 2012.