| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0496_r5 |
| --- | --- |

| Title: | Four additional larger MaxDpbSize values for smaller pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez, A. K Katti, and H-Y Hwang 5030 Sugarloaf Parkway Lawrenceville GA 30044 USA | Tel: Email: | +1 770 236 6483 rodrigaa@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This contribution proposes five MaxDpbSize values for luma picture sizes equal to a factor of 0.25, 0.5, 0.667, 0.75, and 1.0 of the maximum luma picture size (MaxLumaFS), with the amount of linear memory fixed for all MaxDpbSize values.  The amount of linear memory for any picture size is assumed constant and equal to the level’s MaxLumaFS. Any picture, sized in 64x64 luma sample increments, assumes the MaxDpbSize value that corresponds to the closest but larger of the five respective luma picture sizes.

Apology

The authors sincerely apologize for the tardiness of this contribution and would greatly appreciate its consideration.

# Introduction

In the following, let:

- MaxLumaFS denote a level’s maximum luma picture size;

- LumaFS represent a luma picture size less than or equal to MaxLumaFS;

- MaxLumaPR denote the maximum luma sample rate;

- LumaPR represent a luma sample rate less than or equal to MaxLumaPR;

- MaxPicDpbSize be the MaxDpbSize corresponding to MaxLumaFS;  and

- MaxMem represent the maximum amount of linear memory implied from the “MaxPicDpbSize” consecutive buffers of equal size, each corresponding to the MaxLumaFS of the level.

MaxMem is assumed constant for any picture size and implemented as a circular buffer.

Every LumaFS value corresponds to the smallest number of 64 horizontal units and 64 vertical units that respectively encompass the width and height of the luma samples of the picture.

The size of each DPB buffer also corresponds to 64x64 luma sample increments. When LumaFS is not equal to a factor of 0.25, 0.5, 0.667, 0.75, and 1.0 of MaxLumaFS, the consecutive DPB buffers of equal size in MaxMem that correspond to LumaFS are a larger number of 64x64 units than the actual LumaFS.  The benefit in exchange is that a decoder does not have to perform memory de-allocation or reallocation since each DPB buffer is demarcated by a pointer to the start of the buffer and the end by the LumaFS increment.

LumaFS in Table A-10-x represents a multiple of 64x64 units.

Option 1

If MaxDpbSize is equal to 6 for all levels, the MaxDpbSize column is removed from Table A-10-1, and the following table is proposed for MaxDpbSize.  Please refer to JCTVC-J0496-WD_Table_A-10-1-x_opt_1.

Table A-10-x  -   MaxDpbSize limits based on luma picture size (LumaFS)

| LumaFS | MaxDpbSize |
| --- | --- |
| LumaFS ≤ 0.25 * MaxLumaFS | 16 |
| 0.25 * MaxLumaFS < LumaFS ≤ 0.5 * MaxLumaFS | 12 |
| 0.5 * MaxLumaFS < LumaFS ≤ 0.667 * MaxLumaFS | 9 |
| 0.667 * MaxLumaFS < LumaFS ≤ 0.75 * MaxLumaFS | 8 |
| 0.75 * MaxLumaFS < LumaFS ≤ 1.0 * MaxLumaFS | 6 |

In the first row, the four picture buffers of LumaFS are mapped into each picture buffer corresponding to MaxLumaFS. One or more MaxLumaFS buffers may end of vacant.  In the second row, the two picture buffers of LumaFS are mapped into each picture buffer corresponding to MaxLumaFS. In the third row, three picture buffers are mapped into each respective set of two consecutive picture buffers corresponding to MaxLumaFS.  In the four row, four picture buffers are mapped into each respective set of three consecutive picture buffers corresponding to MaxLumaFS.

Option 2

For other MaxDpbSize is not equal to 6 or constant, it becomes a function MaxPicDpbSize in Table A-10-x. Please refer to JCTVC-J0496-WD_Table_A-10-1-x_opt_2.

Table A-10-x - MaxDpbSize limits based on luma picture size (LumaFS) and level’s MaxPicDpbSize

| LumaFS | MaxDpbSize |
| --- | --- |
| LumaFS ≤ 0.25 * MaxLumaFS | Min(4*MaxPicDpbSize, 16) |
| 0.25 * MaxLumaFS < LumaFS ≤ 0.5 * MaxLumaFS | Min(2*MaxPicDpbSize, 16) |
| 0.5 * MaxLumaFS < LumaFS ≤ 0.667 * MaxLumaFS | Min ( Floor(3*MaxPicDpbSize/2), 16) |
| 0.667 * MaxLumaFS < LumaFS ≤ 0.75 * MaxLumaFS | Min ( Floor(4*MaxPicDpbSize/3), 16) |
| 0.75 * MaxLumaFS < LumaFS ≤ 1.0 * MaxLumaFS | MaxPicDpbSize |

In Table A-10-x, the MaxDpbSize for any luma picture size corresponds to the smallest of the five preset luma picture sizes that is greater or equal to the picture size.  LumaFS represents a multiple of 64x64 units.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).