| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0496 |
| --- | --- |

| Title: | Variable MaxDpbSize for smaller pictures based on sample rate and maximum number of yet-to-be output pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez, A. K Katti, and H-Y Hwang 5030 Sugarloaf Parkway Lawrenceville GA 30044 USA | Tel: Email: | +1 770 236 6483 rodrigaa@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

The maximum amount of linear memory required for a level corresponds to the MaxDpbSize of the maximum picture size.  This contribution proposes to increase MaxDpbSize for pictures smaller than the maximum picture size as a function of units of 64x64 samples, maximum number of pictures in the DPB which are yet to be output and sample rate. Also, the mapping in linear memory of the smaller pictures’ DPB picture buffers to the DPB picture buffers corresponding to the maximum picture size is proposed.

Apology

The authors sincerely apologize for the tardiness of this contribution and would greatly appreciate its consideration.

# Introduction

Some applications require transitioning between two CVSes that have different picture sizes but have the same fixed picture rate (for output) or different sample rate.  They also require support to and from a CVS at twice the fixed picture rate or sample rate when the picture size is less than or equal to half the maximum picture size.

The size of a DPB picture buffer for the level’s maximum picture size, MaxPicSize (i.e, MaxLumaFS in Table A-10-1), is determined from the smallest number of 64 horizontal units and 64 vertical units that respectively encompass the width and height of the luma samples in the level’s maximum picture size.  The MaxPicDpbSize implied for a level corresponds to the MaxDpbSize of its MaxPicSize.

To support transitions from one to another picture resolution in two consecutive CVSes, it is assumed that the maximum amount of linear memory required for MaxPicDpbSize consists of the maximum number of DPB picture buffers of size equal to MaxPicSize, and the DPB picture buffers of MaxPicSize are consecutive in linear memory, without gaps between them.  The linear memory corresponding to MaxPicDpbSize is demarcated at the boundaries that start each of the consecutive DPB picture buffers of MaxPicSize. It should be noted that in some cases the demarcated boundaries may be different depending on the DPB picture buffer pointer allocation. We consider the two different models for DPB linear memory based on whether the memory wraps around.

- Circular buffer DPB model: the end of the linear memory is assumed to correspond to the end of the DPB picture buffer of MaxPicSize, and wrap around to the start of the linear memory, which corresponds to the start of a different DPB picture buffer of MaxPicSize.

- Non-circular buffer DPB model: the end of the linear memory is assumed to correspond to the end of the DPB picture buffer of MaxPicSize, and the memory does not wrap around to the start of the linear memory.

It should be noted that for decoder conformance, the MaxDpbSize may be implied by MaxLumaPR (maximum rate of decoded luma samples), but in some cases, it is implied by the maximum rate of output luma samples, such as when the rate of output luma samples equals the value of the level’s MaxLumaPR and the rate of decoded luma sample is less than MaxLumaPR.  The case may manifest when pictures are output more than once (e.g., when fixed_pic_rate_flag = 1 and the coded picture rate in a CVS is less than the output picture rate).

The proposed derivation of MaxDpbOccupancy, the maximum number of  pictures in the DPB which are yet to be output at the time of transition, is based on respective parameter or variable values of any two consecutive CVSes, namely the clock tick value, picture size, and the value of fixed_pic_rate_flag. MaxDpbOccupancy is always less than or equal to the maximum number of reordered pictures (num_reorder_pics) defined for the first of the two consecutive CVSes.

It should also be noted that since the DPB stores reference picture or pictures which are yet to be output, and includes a picture buffer for the current decoded picture, num_reorder_pics for the level’s MaxPicSize must equal MaxPicDpbSize minus one.  Unlike the prior coding video specification, num_reorder_pics must not equal MaxDpbSize because MaxDpbSize includes the current decoded picture.

fixed_pic_rate_flag = 1 in the two consecutive CVSes

The no_output_of_prior_pics_flag, if present, is according to its value, and not inferred.

Unless otherwise noted, the no_output_of_prior_pics_flag is present and equal to zero

Transitions:

- By considering transition from a CVS with pictures of MaxPicSize to another CVS, we derive MaxDpbOccupancy at the time of transition and MaxDpbSize for the second of the two consecutive CVSes.

- By considering transition from one CVS to a CVS with pictures of MaxPicSize, we derive MaxDpbOccupancy at the time of transition. Note that MaxDpbSize for CVS with pictures of MaxPicSize is MaxPicDpbSize.

The picture buffers are mapped to the larger consecutive DPB picture buffers of MaxPicSize in the demarcated linear memory that corresponds to the level’s MaxPicDpbSize. The picture buffer demarcation can change depending on the DPB picture buffer pointer allocation.

Constraints on the MaxDpbOccupancy in both of the above transitions is obtained by considering:

- The DPB pictures in the preceding CVS that are yet to be output; and

- The size of the DPB picture buffers and their respective mapping within the demarcated picture buffers of MaxPicSize in the linear memory corresponding to the level’s MaxPicDpbSize; and

The value of clock tick must be the same for any two consecutive CVSes, except for a CVS with a picture size less than or equal to half of MaxPicSize, when the clock tick may be half or equal to the clock tick of the other CVS.  Otherwise, the value of the no_output_of_prior_pics_flag must be set equal to 1 by the splicer. Note:  A decoded picture intended to be output should be honored either by outputting the picture or a corresponding blank picture.  Otherwise, CPB issues may manifest eventually.

Picture Sizes:

The following four different picture sizes are considered in relation to the level’s MaxPicSize during transition from one CVS to another:

- PicSize1: Less than or equal to   0.5  *  MaxPicSize,

- PicSize2: Greater than 0.5 * MaxPicSize but less than or equal to 0.667 *  MaxPicSize,

- PicSize3: Greater than 0.667 * MaxPicSize but less than or equal to 0.75  *  MaxPicSize, and

- PicSize4: Greater than 0.75 * MaxPicSize but less than or equal to 1.0  *  MaxPicSize

MaxDpbSize and MaxDpbOccupancy :

The tables below provide maximum limit on DpbOccupancy (MaxDpbOccupancy) at the time of transition from CVS with pictures of size MaxPicSize to CVS with pictures of sizes mentioned above. In each of these cases, the respective MaxDpbSize for the second of the two consecutive CVSes is also provided.

| Transition from CVS with MaxPicSize pictures to another CVS with circular DPB buffer |  |  |
| --- | --- | --- |
| Transition | MaxDpbSize after Transition | MaxDpbOccupancy at Transition |
| MaxPicSize to PicSize1 | 2*MaxPicDpbSize | MaxPicDpbSize - 1 |
| MaxPicSize to PicSize2 | 3*MaxPicDpbSize/2 | MaxPicDpbSize - 1 |
| MaxPicSize to PicSize3 | 4*MaxPicDpbSize/3 | 2*MaxPicDpbSize/3 |
| MaxPicSize to PicSize4 | MaxPicDpbSize | MaxPicDpbSize - 1 |

| Transition from CVS with MaxPicSize pictures to another CVS with non-circular DPB buffer |  |  |
| --- | --- | --- |
| Transition | MaxDpbSize after Transition | MaxDpbOccupancy at Transition |
| MaxPicSize to PicSize1 | 2*MaxPicDpbSize | MaxPicDpbSize - 1 |
| MaxPicSize to PicSize2 | 3*MaxPicDpbSize/2 | MaxPicDpbSize - 1 |
| MaxPicSize to PicSize3 | 4*MaxPicDpbSize/3 | 2*MaxPicDpbSize/3 - 1 |
| MaxPicSize to PicSize4 | MaxPicDpbSize | MaxPicDpbSize - 1 |

The tables below provide maximum limit on DpbOccupancy (MaxDpbOccupancy) at the time of transition from CVS with pictures of sizes mentioned above to CVS with pictures of sizes MaxPicSize. In each of these cases, the respective MaxDpbSize for the second of the two consecutive CVSes is fixed at MaxPicDpbSize.

| Transition from one CVS to CVS with MaxPicSize pictures with circular DPB buffer |  |
| --- | --- |
| Transition | MaxDpbOccupancy at Transition |
| PicSize1 to MaxPicSize | MaxPicDpbSize - 1 |
| PicSize2 to MaxPicSize | 3*MaxPicDpbSize/4 - 1 |
| PicSize3 to MaxPicSize | 2*MaxPicDpbSize/3 - 1 |
| PicSize4 to MaxPicSize | MaxPicDpbSize - 1 |

| Transition from one CVS to CVS with MaxPicSize pictures with non-circular DPB buffer |  |
| --- | --- |
| Transition | MaxDpbOccupancy at Transition |
| PicSize1 to MaxPicSize | MaxPicDpbSize - 1 |
| PicSize2 to MaxPicSize | MaxPicDpbSize/2 - 1 |
| PicSize3 to MaxPicSize | 2*MaxPicDpbSize/3 - 1 |
| PicSize4 to MaxPicSize | MaxPicDpbSize - 1 |

Picture size = PicSize1 = less than or equal to 0.5 * MaxPicSize

Two picture buffers are mapped into each of the respective demarcated MaxPicDpbSize picture buffers of MaxPicSize to obtain:

MaxDpbSize = 2 * MaxPicDpbSize

To allow a transition from a CVS with pictures of MaxPicSize, consideration is given to the worst case occupation of DPB picture buffers, each of size equal to MaxPicSize, in the linear memory that corresponds to MaxPicDpbSize.  The following is obtained:

MaxDpbOccupancy = MaxPicDpbSize - 1 : when DPB is either a circular or a non-circular buffer

To allow a transition back to a CVS with pictures of MaxPicSize, consideration is given to the worst case occupation of DPB picture buffers, each of size equal to 0.5 * MaxPicSize, in the linear memory that corresponds to MaxPicDpbSize.  The following is obtained:

MaxReorderNum = MaxPicSizeDpb - 1 : when DPB is either a circular or a non-circular buffer

Picture size = PicSize2 = greater than 0.5 * MaxPicSize but less than or equal to 0.667 * MaxPicSize

Three consecutive picture buffers are mapped into each respective set of two consecutive picture buffers of 1.0 * MaxPicSize, to obtain:

MaxDpbSize = floor (3 * MaxPicDpbSize / 2)

MaxReorderNumLimit = MaxPicDpbSize - 1

Picture size = PicSize3 = Greater than 0.667 * MaxPicSize but less than or equal to 0.75 * MaxPicSize

Four consecutive picture buffers are mapped into each respective set of three consecutive picture buffers of 1.0 * MaxPicSize, to obtain:

MaxDpbSize = floor (4 * MaxPicDpbSize / 3)

Picture size = PicSize4 = Greater than 0.75 * MaxPicSize but less than or equal to 1.0 * MaxPicSize

Each respective picture buffer is mapped into a corresponding respective picture buffer of 1.0 * MaxPicSize:

MaxDpbSize = MaxPicDpbSize

fixed_pic_rate_flag equals zero in one of both of the two consecutive CVSes

The no_output_of_prior_pics_flag, if present, is according to its value, and not inferred.

Unless otherwise noted, the no_output_of_prior_pics_flag is present and equal to zero

The value of clock tick must be the same for any two consecutive CVSes, except for a CVS with a picture size less than or equal half of MaxPicSize, when the clock tick may be half or equal to the clock tick of the other CVS.  Otherwise, the value of the no_ouput_of_prior_pics_flag must be equal or inferred equal to 1.

The respective MaxDpbSize and maximum number of reorder pictures are provided for two different picture sizes in relation to the level’s MaxPicSize:

- Less than or equal to   0.5  *  MaxPicSize, and

- Greater than 0.5 * MaxPicSize but less than or equal to 1.0  *  MaxPicSize

Picture size = Less than or equal to 0.5 * MaxPicSize

Two picture buffers are mapped into each of the respective demarcated MaxPicDpbSize picture buffers of MaxPicSize to obtain:

MaxDpbSize = 2 * MaxPicDpbSize

Picture size = Greater than 0.5 * MaxPicSize but less than or equal to 1.0 * MaxPicSize

Each respective picture buffer is mapped into a corresponding respective picture buffer of MaxPicSize to obtain:

MaxDpbSize = MaxPicDpbSize

Table A-10-1 – Level limits

| Level | Max luma sample rate MaxLumaPR (samples/sec) | Max luma picture size MaxLumaFS (samples) | Max bit rate MaxBR (1000 bits/s) | Min Compression Ratio MinCR | MaxDpbSize (picture storage buffers) | Max CPB size (1000 bits) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | 6 | 350 |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | 6 | 1,000 |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | 6 | 5,000 |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | 6 | 9,000 |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | 6 | 15,000 |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | 6 | 30,000 |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | 6 | 30,000 |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | 6 | 50,000 |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | 6 | 50,000 |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | 6 | 100,000 |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | 6 | 150,000 |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | 6 | 300,000 |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | 6 | 500,000 |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | 6 | 800,000 |

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).