| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0496_r4 |
| --- | --- |

| Title: | Four additional larger MaxDpbSize values for smaller pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Arturo A. Rodriguez, A. K Katti, and H-Y Hwang 5030 Sugarloaf Parkway Lawrenceville GA 30044 USA | Tel: Email: | +1 770 236 6483 rodrigaa@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

This contribution proposes five MaxDpbSize values for luma picture sizes equal to a factor of 0.25, 0.5, 0.667, 0.75, and 1.0 of the maximum luma picture size (MaxLumaFS), with the amount of linear memory fixed for all MaxDpbSize values.  The amount of linear memory for any picture size is assumed constant and equal to the level’s MaxLumaFS. Any picture, sized in 64x64 luma sample increments, assumes the MaxDpbSize value that corresponds to the closest but larger of the five respective luma picture sizes. This contribution also proposes further limits on DpbSize when the luma picture size changes and the no_output_of_prior_pics_flag equals zero to allow the option to output DPB pictures “yet to be output.” The option also limits changes in luma sample rate or requires fixed picture rate through the two consecutive coded video sequences with a change in picture size.

Apology

The authors sincerely apologize for the tardiness of this contribution and would greatly appreciate its consideration.

# Introduction

In the following, let:

- MaxLumaFS denote a level’s maximum luma picture size;

- LumaFS represent a luma picture size less than or equal to MaxLumaFS;

- MaxLumaPR denote the maximum luma sample rate;

- LumaPR represent a luma sample rate less than or equal to MaxLumaPR;

- MaxPicDpbSize be the MaxDpbSize corresponding to MaxLumaFS;  and

- MaxMem represent the maximum amount of linear memory implied from the “MaxPicDpbSize” consecutive buffers of equal size, each corresponding to the MaxLumaFS of the level.

MaxMem is assumed constant for any picture size and implemented as a circular buffer.

Every LumaFS value corresponds to the smallest number of 64 horizontal units and 64 vertical units that respectively encompass the width and height of the luma samples of the picture.

The size of each DPB buffer also corresponds to 64x64 luma sample increments. When LumaFS is not equal to a factor of 0.25, 0.5, 0.667, 0.75, and 1.0 of MaxLumaFS, the consecutive DPB buffers of equal size in MaxMem that correspond to LumaFS are a larger number of 64x64 units than the actual LumaFS.  The benefit in exchange is that a decoder does not have to perform memory de-allocation or reallocation since each DPB buffer is demarcated by a pointer to the start of the buffer and the end by the LumaFS increment.

LumaFS in Table A-10-x represents a multiple of 64x64 units.

Option 1

If MaxDpbSize is equal to 6 for all levels, the MaxDpbSize column is removed from Table A-10-1, and the following table is proposed for MaxDpbSize.  Please refer to JCTVC-J0496-WD_Table_A-10-1-x_opt_1.

Table A-10-x  -   MaxDpbSize limits based on luma picture size (LumaFS)

| LumaFS | MaxDpbSize |
| --- | --- |
| LumaFS ≤ 0.25 * MaxLumaFS | 16 |
| 0.25 * MaxLumaFS < LumaFS ≤ 0.5 * MaxLumaFS | 12 |
| 0.5 * MaxLumaFS < LumaFS ≤ 0.667 * MaxLumaFS | 9 |
| 0.667 * MaxLumaFS < LumaFS ≤ 0.75 * MaxLumaFS | 8 |
| 0.75 * MaxLumaFS < LumaFS ≤ 1.0 * MaxLumaFS | 6 |

In the first row, the four picture buffers of LumaFS are mapped into each picture buffer corresponding to MaxLumaFS. One or more MaxLumaFS buffers may end of vacant.  In the second row, the two picture buffers of LumaFS are mapped into each picture buffer corresponding to MaxLumaFS. In the third row, three picture buffers are mapped into each respective set of two consecutive picture buffers corresponding to MaxLumaFS.  In the four row, four picture buffers are mapped into each respective set of three consecutive picture buffers corresponding to MaxLumaFS.

Option 2

For other MaxDpbSize is not equal to 6 or constant, it becomes a function MaxPicDpbSize in Table A-10-x. Please refer to JCTVC-J0496-WD_Table_A-10-1-x_opt_2.

Table A-10-x - MaxDpbSize limits based on luma picture size (LumaFS) and level’s MaxPicDpbSize

| LumaFS | MaxDpbSize |
| --- | --- |
| LumaFS ≤ 0.25 * MaxLumaFS | Min(4*MaxPicDpbSize, 16) |
| 0.25 * MaxLumaFS < LumaFS ≤ 0.5 * MaxLumaFS | Min(2*MaxPicDpbSize, 16) |
| 0.5 * MaxLumaFS < LumaFS ≤ 0.667 * MaxLumaFS | Min ( Floor(3*MaxPicDpbSize/2), 16) |
| 0.667 * MaxLumaFS < LumaFS ≤ 0.75 * MaxLumaFS | Min ( Floor(4*MaxPicDpbSize/3), 16) |
| 0.75 * MaxLumaFS < LumaFS ≤ 1.0 * MaxLumaFS | MaxPicDpbSize |

In Table A-10-x, the MaxDpbSize for any luma picture size corresponds to the smallest of the five preset luma picture sizes that is greater or equal to the picture size.  LumaFS represents a multiple of 64x64 units.

# Limits when no_ouput_of_prior_pics_flag = 0 to allow the option to output DPB pictures “yet to be output”

The limits and constrained in this section enable the output of “pictures yet to be output” when the picture size changes in two consecutive CVSes.

The no_output_of_prior_pics_flag is present and equal to zero.

The two consecutive CVSes have fixed_pic_rate_flag = 1.

Let NumPicsNYO be the maximum number of pictures from the preceding CVS in the DPB that are “yet to be output” at the start of a CVS with a change in picture size.  NumPicsNYO is always less than or equal to the maximum number of reordered pictures in the preceding CVS.

Limits on  MaxDpbSize and NumPicsNYO are obtained by considering the mapping of DPB buffers in MaxMem for the worst-case DPB buffer occupation of pictures “yet to be output” at the transition, and roundtrip  transitions in the bitstream, which are typical in commercial insertion applications.  Namely, we consider transitions from:

- a CVS of MaxLumaFS to another CVS of PicSize_n, and

- a CVS of PicSize_n to a CVS of MaxLumaFS.

The following sets of tables specify the corresponding limits for respective transition types, assuming a MaxPicDpbSize of 6.

| Transition from MaxLumaFS to PicSize_n (for MaxPicDpbSize = 6) |  |  |
| --- | --- | --- |
| Transition type from MaxLumaFS to LumaFS: | NumPicsNYO (MaxLumaFS) | MaxDpbSize (LumaFS ) |
| LumaFS ≤ 0.5 * MaxLumaFS | 5 | 12 |
| 0.5 < LumaFS ≤ 0.667 * MaxLumaFS | 5 | 9 |
| 0.667 < LumaFS ≤ 0.75 * MaxLumaFS | 4 | 8 |
| 0.75 < LumaFS ≤ 1.0 * MaxLumaFS | 5 | 6 |

| Transition from PicSiz_n to MaxLumaFS (for MaxPicDpbSize = 6) |  |  |
| --- | --- | --- |
| Transition type From LumaFS to MaxLumaFS: | NumPicsNYO (LumaFS ) | MaxDpbSize (MaxLumaFS) |
| LumaFS ≤ 0.5 * MaxLumaFS | 5 | 6 |
| 0.5 < LumaFS ≤ 0.667 * MaxLumaFS | 4 | 6 |
| 0.667 < LumaFS ≤ 0.75 * MaxLumaFS | 3 | 6 |
| 0.75 < LumaFS ≤ 1.0 * MaxLumaFS | 5 | 6 |

DPBSize can be reduced within MaxMem to increase NumPicsNYO.

The no_ouput_of_prior_pics_flag may equal to 1 when the above limits are met or not met.

NumPicsNYO may be signalled anywhere in the prior CVS by an encoder, such as in the SPS’ VUI when the fixed_pic_rate_flag =1.  Or it could be signalled in an SEI.

If the bitstream is entered at a RAP where the no_ouput_of _prior_pics_flag = 0, it shall be inferred equal to 1.

Otherwise, the value of the no_output_of_prior_pics_flag must be set equal to 1.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).