Table A-10-1 – Level limits

| Level | Max luma sample rate MaxLumaPR (samples/sec) | Max luma picture size MaxLumaFS (samples) | Max bit rate MaxBR (1000 bits/s) | Min Compression Ratio MinCR | MaxPicDpbSize (picture storage buffers for MaxLumaFS) | Max CPB size (1000 bits) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 552,960 | 36,864 | 128 | 2 | L1 MaxPicDpbSize | 350 |
| 2 | 3,686,400 | 122,880 | 1,000 | 2 | L2 MaxPicDpbSize | 1,000 |
| 3 | 13,762,560 | 458,752 | 5,000 | 2 | L3 MaxPicDpbSize | 5,000 |
| 3.1 | 33,177,600 | 983,040 | 9,000 | 2 | L3.1 MaxPicDpbSize | 9,000 |
| 4 | 62,668,800 | 2,088,960 | 15,000 | 4 | L4 MaxPicDpbSize | 15,000 |
| 4.1 | 62,668,800 | 2,088,960 | 30,000 | 4 | L4.1 MaxPicDpbSize | 30,000 |
| 4.2 | 133,693,440 | 2,228,224 | 30,000 | 4 | L4.2 MaxPicDpbSize | 30,000 |
| 4.3 | 133,693,440 | 2,228,224 | 50,000 | 4 | L4.3 MaxPicDpbSize | 50,000 |
| 5 | 267,386,880 | 8,912,896 | 50,000 | 6 | L5 MaxPicDpbSize | 50,000 |
| 5.1 | 267,386,880 | 8,912,896 | 100,000 | 8 | L5.1 MaxPicDpbSize | 100,000 |
| 5.2 | 534,773,760 | 8,912,896 | 150,000 | 8 | L5.2 MaxPicDpbSize | 150,000 |
| 6 | 1,002,700,800 | 33,423,360 | 300,000 | 8 | L6 MaxPicDpbSize | 300,000 |
| 6.1 | 2,005,401,600 | 33,423,360 | 500,000 | 8 | L6.1 MaxPicDpbSize | 500,000 |
| 6.2 | 4,010,803,200 | 33,423,360 | 800,000 | 6 | L6.2 MaxPicDpbSize | 800,000 |

Table A-10-x - MaxDpbSize limits based on luma picture size (LumaFS) and level’s MaxPicDpbSize

| LumaFS | MaxDpbSize |
| --- | --- |
| LumaFS ≤ 0.25 * MaxLumaFS | Min(4*MaxPicDpbSize, 16) |
| 0.25 * MaxLumaFS < LumaFS ≤ 0.5 * MaxLumaFS | Min(2*MaxPicDpbSize, 16) |
| 0.5 * MaxLumaFS < LumaFS ≤ 0.667 * MaxLumaFS | Min ( Floor(3*MaxPicDpbSize/2), 16) |
| 0.667 * MaxLumaFS < LumaFS ≤ 0.75 * MaxLumaFS | Min ( Floor(4*MaxPicDpbSize/3), 16) |
| 0.75 * MaxLumaFS < LumaFS ≤ 1.0 * MaxLumaFS | MaxPicDpbSize |

    - General

The following is specified for expressing the constraints in this annex.

- Let access unit n be the n-th access unit in decoding order, with the first access unit being access unit 0.

- Let picture n be the coded picture or the corresponding decoded picture of access unit n.

- Let the variable fR be set to 1  300.

Bitstreams conforming to a profile at a specified level shall obey the following constraints:

- The nominal removal time of access unit n (with n > 0) from the CPB as specified in subclause Error: Reference source not found, satisfies the constraint that tr,n( n ) − tr( n − 1 ) is greater than or equal to Max( PicSizeLuma  MaxLumaPR, fR ), where MaxLumaPR is the value specified in Table A-10-1 that applies to picture n − 1, and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples in picture n − 1.

- The difference between consecutive output times of pictures from the DPB as specified in subclause Error: Reference source not found, satisfies the constraint that to,dpb( n ) >= Max( PicSizeLuma  MaxLumaPR, fR ), where MaxLumaPR is the value specified in Table A-10-1 for picture n, and PicSizeLuma is the number of pic_width_in_luma_samples * pic_height_in_luma_samples of picture n, provided that picture n is a picture that is output and is not the last picture of the bitstream that is output.

- pic_width_in_luma_samples * pic_height_in_luma_samples <= MaxLumaFS, where MaxLumaFS is specified in Table A-10-1

- pic_width_in_luma_samples <= Sqrt( MaxLumaFS * 8 )

- pic_height_in_luma_samples <= Sqrt( MaxLumaFS * 8 )

- sps_max_dec_pic_buffering[ sps_max_temporal_layers_minus1 ] <= MaxDpbSize, where MaxDpbSize is specified in Table A-10-1x, and MaxPicDpbSize is specified in Table A-10-1 .