| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0499 |
| --- | --- |

| Title: | AHG9: A mental cross-check of JCTVC-J0107 (On RAP pictures) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Tel: Email: | +358 7180 08000 miska.hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This document provides a mental cross-check for JCTVC-J0107-v1.

# Mental cross-check

It is claimed that the authors of the cross-check contribution basically understand what is being proposed in JCTVC-J0107-v1. The authors of this cross-check document would like to summarize the proposal in JCTVC-J0107-v1 as follows:

There are the following seven parts in the proposal:

- Reportedly when streaming adaptation is performed, servers or intermediate network elements would prefer avoiding re-writing bits in the video bitstream, such as changing the NAL unit type values such that a CRA picture at the switch point would become a BLA picture. It is therefore proposed that there can be external means to indicate to the decoder that a CRA picture shall be processed like a BLA picture.

- It is proposed that non-leading pictures can use decodable leading pictures (non-TFD leading pictures) as prediction sources.

- The definition of the RAP picture is proposed to be modified in such a manner that it would not impose additional constraints on certain GOP patterns starting with an IDR picture, as such constraints are not present in other parts the standard.

- It is proposed to activate parameter sets at a BLA picture similarly to how they are activated at an IDR picture.

- The ordering of leading pictures is proposed to be constrained as follows:

  - Decodable leading pictures, if any, associated with a particular CRA/BLA picture are proposed to succeed in output order TFD pictures, if any, associated with the same CRA/BLA picture.

  - To ease extraction of TFD pictures, it is proposed that a TFD picture associated with a CRA/BLA picture shall precede, in decoding order, any picture that follows the CRA/BLA picture in both decoding and output order.

- Changes to the inference of no_output_of_prior_pics_flag are proposed such that reportedly different parts of the specification would not contradict.

- It is proposed to add one NAL unit type to distinct whether TFD pictures are also TLA pictures.