| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0504-v1 |
| --- | --- |

| Title: | AHG9 & AHG13: Cross-check of JCTVC-J0120: Identical reference picture lists |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Philippe Bordes, Pierre Andrivon 1, av. De Belle-Fontaine – CS17616 35576 Cesson-Sévigné Cedex - France | Tel: Email: | +33-2-99-27-32-42 philippe.bordes@technicolor.com pierre.andrivon@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results on the signaling whether RefPicList0 and RefPicList1 are identical or not, as proposed in JCTVC-J0120.

When it is signaled the RefPicList0 and RefPicList1 are identical, and if weighted prediction is used, only the prediction weights for RefPicList0 are transmitted in the slice header, they are assumed to be the same for RefPicList1.

The verification task using the HEVC fade sequences test has been done successfully and the results match those provided by proponent. The encoding time and the decoding time of this cross-check are also similar to those provided by proponents.

# Cross-checking

The software based on HM version 7.0 and the experimental results were provided on July 9, 2012 by Qualcomm. The software was compiled fine and the cross-checking was performed on 64bit linux environment. The simulation has been conducted under common test conditions [1] for Main profile (MP) conditions and high efficiency (HE10) conditions in the case of random access (RA) and low delay B (LB) coding structures using black/white-fade sequences provided by [2].

The anchors were generated using the HM7.0, with weighted prediction enabled. In the software provided by the proponents the followed two macros were set as follows:

#define QC_IDENTICAL_LISTS	1

#define QC_WP_BITCOUNT		1

# Results

Table 1 and Table 2 show BD-bitrate and relative average encoding/decoding time with the proposed identical_list_flag flag signalling whether RefPicList0 and RefPicList1 are identical compared with HM7.0 weighted prediction anchor. The negative value of BD-bitrate indicates gain.

Table 1: BD-bitrate[%] and relative encoding/decoding time[%] of the proposed algorithm of JCTVC-J0120 compared with HM7.0 WP anchor for black-fade sequences

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.2% | -0.2% | -0.2% | -0.4% | -0.4% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.3% | -0.3% | -0.5% | -0.4% | -0.4% |
| Class C | -0.7% | -0.6% | -0.6% | -1.1% | -1.0% | -1.0% |
| Class D | -2.2% | -2.0% | -2.1% | -3.4% | -3.1% | -3.1% |
| Class E | -2.3% | -2.0% | -2.0% | -3.9% | -3.5% | -3.5% |
| Overall | -1.3% | -1.1% | -1.1% | -2.0% | -1.8% | -1.8% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 95% |  |  |

Table 2: BD-bitrate[%] and relative encoding/decoding time[%] of proposed algorithm of JCTVC-J0120 compared with HM7.0 WP anchor for white-fade sequences

|  | Random Access Main |  |  | Random Access HE10 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% | -0.1% |
| Class D | -0.3% | -0.3% | -0.3% | -0.5% | -0.4% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 91% |  |  | 97% |  |  |
|  | Low delay B Main |  |  | Low delay B HE10 |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.3% | 0.0% | -0.6% | -0.5% | -0.5% |
| Class C | -0.9% | -0.8% | -0.8% | -1.3% | -1.1% | -1.1% |
| Class D | -2.8% | -2.5% | -2.6% | -3.9% | -3.5% | -3.6% |
| Class E | -3.0% | -2.5% | -2.6% | -4.5% | -4.0% | -4.1% |
| Overall | -1.6% | -1.4% | -1.4% | -2.3% | -2.1% | -2.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 95% |  |  | 93% |  |  |

# Conclusion

We cross-checked the JCTVC-J0120 proposal signaling whether RefPicList0 and RefPicList1 are identical or not, in case of fade sequences.

We confirm the results presented in JCTVC-J0120.

# References

- F. Bossen, “Common HM test conditions and software reference configurations”, Document no JCTVC-I1100. May 2012.

- AHG report, " JCT-VC AHG report: Weighted prediction (AHG 18)," Joint Collaborative Team on Video Coding, JCTVC-F018, Torino, July 2011.