| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11 – 20 July 2012 | Document: JCTVC-J0509 |
| --- | --- |

| Title: | Cross-check: Subjective viewing on intensity dependant spatial quantisation (JCTVC-J0076) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kazuo Sugimoto Shun-ichi Sekiguchi Information Technology R&D Center 5-1-1, Ofuna, Kamakura, JAPAN | Tel: Email: | +81-467-41-2463 Sugimoto.Kazuo@ak.MitsubishiElectric.co.jp |
| Source: | Mitsubishi Electric Corporation |  |  |

_____________________________

# Abstract

This document reports a cross-check of the JCTVC-J0076, “Improving HEVC compression efficiency by intensity dependant spatial quantisation.” An informative subjective viewing on some of the test contents which resulted substantial bitrate reductions are carried out at Mitsubishi Electric.

.

# Introduction

According to the contribution JCTVC-J0076[1], it is reported that up to 25% bit rate reduction is achieved for the same perceptual quality level by applying the proposed intensity dependent spatial quantization. In this contribution, we report the result of our informal subjective viewing of the sequences which gives substantial bitrate reductions.

# Subjective viewing

We have evaluated the top five size sequences in terms of bit rate reductions listed below. We chose non-Class A and non-Intra only streams.

| Test | Sequence name | Configuration | QP | ΔRate [%] | ΔPSNR [dB] |
| --- | --- | --- | --- | --- | --- |
| 1 | ChinaSpeed | ldP_he10 | 22 | -19.72 | -2.09 |
| 2 | Cactus | ld_he10 | 22 | -16.73 | -0.29 |
| 3 | BasketballDrive | ld_he10 | 22 | -9.57 | -0.18 |
| 4 | Kimono | ld_main | 27 | -9.47 | -0.45 |
| 5 | ParkScene | ldP_main | 22 | -8.18 | -0.39 |

The anchor and the proposal sequences are shown to some researchers at Mitsubishi without let them know which is the anchor, and all viewer voted which looks better. The results are as follows:

| Test | Anchor looks better | Same Quality | Proposal looks better |
| --- | --- | --- | --- |
| 1 | 4 | 4 | 0 |
| 2 | 4 | 1 | 3 |
| 3 | 4 | 3 | 1 |
| 4 | 2 | 4 | 2 |
| 5 | 1 | 3 | 4 |

# References

- Matteo Naccari, Marta Mrak, David Flynn, Andrea Gabriellini, “Improving HEVC compression efficiency by intensity dependant spatial quantisation,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-J0076, Stockholm, SE, July 2012