| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0510 |
| --- | --- |

| Title: | AHG10: Mental cross check of JCTV-J0121 on motion related hooks for HEVC multiview/3DV extension |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Oguz Bici Miska M. Hannuksela Nokia Research Center Visiokatu 3, 33720 Tampere, Finland | Email: | ext-oguz.bici@nokia.com Miska.Hannuksela@nokia.com |
| Source: | Nokia Corporation |  |  |

_____________________________

# Abstract

This document provides a mental cross-check for the document JCTV-J0121. It is claimed that the authors of this document understand what are being proposed in JCTVC-J0121. In JCTVC-J0121, prediction of an MV belonging to a long-term reference picture from an MV belonging to a short-term reference picture or vice versa is disabled.

# Summary of JCTVC-J0121

In JCTVC-J0121, the following additional conditions to discard motion vector prediction (MVP) candidates are introduced depending on the short term reference picture (STRP) and long term reference picture (LTRP) types:

- For spatial MVPs in AMVP: If reference picture type for a neighbour MVP candidate is STRP and reference picture type for the current PU is LTRP, or vice versa, the MVP candidate is discarded. This means that they need to both STRP or both LTRP.

- For temporal MVP: If reference picture type for the co-located PU is STRP and reference picture type for the TMVP is LTRP, or vice versa, the TMVP candidate is discarded. This condition is applicable to the TMVP candidate in both AMVP mode and merge mode.