| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0512-r1 |
| --- | --- |

| Title: | BoG report on alternative coding modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono | Email: | chono@ct.jp.nec.com |
| Source: | Alternative coding mode BoG |  |  |

_____________________________

Summary

This BoG was established in order to study contributions on alternative coding modes (I_PCM, Transform skipping mode, and lossless coding). Proposals of each alternative coding mode were categorized and summarized in the attached XLS sheet. During the BoG discussion, it was generally agreed the following three items are discussed in the main session.

- Modified scanning for TS mode;

- TS mode for interTU;

- Non-normative change to TS mode desicion for chroma TS.

It is recommended that JCT-VC experts review the summary XLS sheet and discuss the necessity of presentations of categorized schemes in the main session.

- Procedure

For categorization, each proponent vocally reported purposes and technical elements of their proposal.

I_PCM and Transform skipping related proposals were reviewed.

For I_PCM related proposals, the schemes are categorized as follows:

- I_PCM signalling

  - Burst IPCM (JCTVC-J0153)

  - Conditional signaling of pcm_flag (JCTVC-J0277)

- Harmonization of filtering control of I_PCM and lossless coding modes

  - Syntax harmonization of the I_PCM and TransQuantBypass modes (JCTVC-J0168)

  - Semantics harmonization of loop filtering control for I_PCM and TransQuantBypass modes (JCTVC-J0169)

For transform skipping related proposals, the schemes are categorized as follows:

- Scanning

  - Rotate scan (JCTVC-J0053 and JCTVC-J0093)

  - V/H scan type swapping scan (JCTVC-J0202 Part2, JCTVC-J0213, and JCTVC-J0313 Part1)

- Additional context for TS mode coefficients

  - Context modeling for significance (JCTVC-J0069 Part1)

  - Context modeling for abs_one_greater_than_one  (JCTVC-J0069 Part2)

- Enable TS mode in inter-coded TU

  - TS mode flag sharing among Y/U/V components (JCTVC-J0077)

  - No sharing (JCTVC-J0237  Part1)

- QM for TS mode coefficients

  - QM using 16 for TS (JCTVC-J0237 Part2, JCTVC-J0184 part1, and JCTVC-J0201)

  - QM using DC for TS (JCTVC-J0092 and JCTVC-J0184 part3)

  - QM using explicit weight for TS	 (JCTVC-J0184 part2)

- TS high-level syntax

  - Move TS enabling flag from SPS to PPS (JCTVC-J0184 part4)

- Conditional signalling of TS flag in chroma

  - Luma part. dependent TS for chroma (JCTVC-J0171 and JCTVC-J0389)

  - Luma dependent TS for chroma	 (JCTVC-J0265 Part2)

  - CU-size dependent TS for chroma (JCTVC-J0265 Part1)

- Coding of TS mode coefficients

  - Skip lastX/lastY pos signalling in TS mode (JCTVC-J0309)

For lossless related proposals, the schemes are categorized as follows:

- Remaining residual coding (JCTVC-J0230)

- Lossless signaling based on QPy + TS flag

  - Lossless signalling based on QPy=4 and TSFlag=1 (JCTVC-J0238 and JCTVC-J0435)

- Region based lossless coding

  - Prediction based encoding of cu_transquant_bypass_flag (JCTVC-J0339 Part1)

  - Three contexts for encoding of cu_transquant_bypass_flag for region-based lossless coding  (JCTVC-J0339 Part2)

  - TransQuant group (JCTVC-J0340)

- Rotate scan (JCTVC-J0093)

# Reviewed documents

[I_PCM related] 4 proposals

JCTVC-J0153 Burst I_PCM Signalling [M. Coban, W.-J. Chien, J. Chon, M. Karczewicz (Qualcomm)]

What is the objective of your proposal? 	Simplification and coding gain in I_PCM signalling.

What are changed in the spec?	Signaling of the number of I_PCM blocks.

Category: I_PCM signaling.

It was agreed that decision on this proposal is deferred after LCU bit/bin limit discussion.

JCTVC-J0411 Crosscheck of Burst IPCM Coding Signalling (JCTVC-J0153) [G. Clare, F. Henry (Orange Labs)] [late]

Not present.

JCTVC-J0277 Interaction between pcm_flag and cu_transquant_bypass_flag [R. Joshi, G. Van Der Auwera, M. Karczewicz (Qualcomm)]

What is the objective of your proposal?	Restrict pcm_flag signaling when cu_transquant_bypass_flag is equal to 1.  Remove the redundancy between  pcm_flag and cu_transquant_bypass_flag.

What are changed in the spec? One more condition for pcm_flag parsing.

Category: I_PCM signaling.

JCTVC-J0168 AHG11: Syntax harmonisation of the I_PCM and TransQuantBypass modes [E. François, P. Onno, G. Laroche, T. Poirier, M.Shima (Canon)]

What is the objective of your proposal? Harmonization of signaling control of I_PCM and lossless coding modes.

What are changed in the spec? (1) Signal enable/disable flags of I_PCM/lossless modes at both SPS and PPS. (2) Signal lossless coding block size information as I_PCM. (3) Move the location of pcm_flag from PU to CU. (Note that the newest draft have already incorporated it.)

Category: High-level syntax issues for signaling IPCM and lossless modes.

JCTVC-J0426 Crosscheck of Syntax harmonisation of the I_PCM and TransQuantBypass modes (JCTVC-J0168) [G. Clare, F. Henry (Orange Labs)] [late]

Not present.

JCTVC-J0169 AHG11: Loop filtering control for I_PCM and TransQuantBypass modes [E. François, P. Onno, G. Laroche, T. Poirier, M.Shima (Canon)]

What is the objective of your proposal? Harmonization of loop filtering processes of I_PCM and lossless coding modes.

What are changed in the spec? (1) A new flag to indicate the presence of I_PCM and lossless coding modes in CTB. Signaling of SAO parameter is conditioned on this flag.  (2) Change the definition of cu_transquant_bypass_flag: This flag indicates only whether transform and quantization are skipped or not.

Category: Lossless coding.

Q: Why we need a new flag? SAO can be on/off at LCU level w/o the new flag.

A: There is no means to skip filtering on lossless coded CUs. The current SAO (and ALF) design does not allow the intent of proposal.

Q: Do we need such a capability for lossless coded CUs?

C: Lossless coded CU may contain lossy samples.

C: The new flag may interact with AHG5/AHG6 SAO coding.

C: The new flag seems to be redundant.

One expert expressed opinion that (2) can be considered in the main session.

It was remarked that JCTVC-J0288 Part3 is related to this and proposes to move cu_transquant_bypass_flag after pcm_flag.

[TS related] 20 proposals

JCTVC-J0436 AHG11: Study report of JCTVC-J0169 on loop filtering control for I_PCM and TransQuantBypass modes [K. Chono (NEC)] [late]

Results are verified.

JCTVC-J0053 Residue scan for intra transform skip mode [J. An, L. Zhao, Y.-W. Huang, S. Lei (MediaTek)]

What is the objective of your proposal? Coding efficiency improvement.

What are changed in the spec? Rotate the scanning for TS mode.

Category: Scanning for TS mode.

Similar proposal is presented in JCTVC-J0093 which applies the rotated scanning to lossless coding mode, as well.

Part2 of JCTVC-J0202 proposes to swap horizontal and vertical scanning (V/H scan type swapping scanning) for TS mode. Part1 of JCTVC-J0202 proposes to use DG scan always for TS mode. The proponent recommends the V/H scan type swapping scanning.

JCTVC-J0212 and Part1 of JCTVC-J0313 propose the V/H scan type swapping scan, too.

Results of the rotated scanning:

Class F: (AI-HE10, RA-HE10, LB-HE10) = (1.6%, 1.1%, 0.7%). Negligible changes in both encoding and decoding times.

ClassA-E: (AI-HE10, RA-HE10, LB-HE10) = (0.2%, 0.1%, 0.0%). Negligible changes in both encoding and decoding times.

Results of the V/H scan type swapping scanning:

Class F: (AI-HE10, RA-HE10, LB-HE10) = (0.9%, 0.7%, 0.3%). Negligible changes in both encoding and decoding times.

ClassA-E: (AI-HE10, RA-HE10, LB-HE10) = (0.0%, 0.0%, 0.0%). Negligible changes in both encoding and decoding times.

BoG recommendation::Suggest the presentation of the two schemes (rotated scanning and V/H scan type swapping scanning) in the main session. (Note that the application of the rotated scanning to lossless coding mode is regarded as a separate proposal.)

JCTVC-J0093 Rotation of Residual Block for Transform Skipping [D. He, J. Wang, G. Martin-Cocher (RIM)]

See the note in JCTVC-J0053.

JCTVC-J0100 Cross-check of transform skipping for inter prediction (JCTVC-J0077) [J. Sole, R. Joshi (Qualcomm)] [late]

Verified.

JCTVC-J0202 Coefficient scan for transform skip mode [H. Y. Kim, S.-C. Lim (ETRI), K. Y. Kim, G. H. Park (KHU), J. Lee, J. S. Choi (ETRI)]

See the note in JCTVC-J0053.

JCTVC-J0386 Cross check of Coefficient scan for transform skip mode (JCTVC-J0202) [T. Yamamoto (Sharp)] [late] [miss]

Verified.

JCTVC-J0212 Modified scan method for intra transform skipping [T. Sugio, T. Nishi (Panasonic)]

See the note in JCTVC-J0053.

JCTVC-J0244 Cross-check of JCTVC-J0053 and JCTVC-J0093 [J. Xu (Microsoft)] [late]

Verified.

JCTVC-J0313 Modified coefficient scan order mapping for transform skip mode [R. Cohen, A. Vetro (MERL)]

See the note in JCTVC-J0053.

JCTVC-J0327 Crosscheck of JCTVC-J0212: Modified scan method for intra transform skipping [M. Budagavi (TI)] [late]

Verified.

JCTVC-J0407 Cross-check of JCTVC-J0313 (Modified coefficient scan order mapping for transform skip mode) [C. Yeo, Y. H. Tan (I2R)] [late]

Verified.

JCTVC-J0069 Constant coefficient context for intra transform skipping [T. Tsukuba, T. Yamamoto (Sharp)]

What is the objective of your proposal? Coding efficiency improvement.

What are changed in the spec? (Part1) Change context derivation of significance map of TS mode. Use a constant context for coefficients of one component. U and V components share single context. (Part2) On top of part1, use a constant context for abs_greater_than_one_flag. U and V components share single context.

Category: Additional coefficient context for TS mode.

Results for Part1 (additional significance map context for TS mode):

Class F: (AI-HE10, RA-HE10, LB-HE10) = (1.0%, 0.8%, 0.6%). Negligible changes in both encoding and decoding times.

ClassA-E: (AI-HE10, RA-HE10, LB-HE10) = (0.1%, 0.0%, 0.0%). Negligible changes in both encoding and decoding times.

Results for Part2 (additional significance map and abs_greater_than_one contexts for TS mode):

Class F: (AI-HE10, RA-HE10, LB-HE10) = (1.5%, 1.1%, 0.9%). Negligible changes in both encoding and decoding times.

ClassA-E: (AI-HE10, RA-HE10, LB-HE10) = (0.1%, 0.1%, 0.0%). Negligible changes in both encoding and decoding times.

JCTVC-J0259 Cross-verification of Constant coefficient context for intra transform skipping (JCTVC-J0069) by BBC [M. Mrak, M. Naccari (BBC)] [late]

Verified.

Cross-checker reported that the combination of JCTVC-J0069 and the rotated scanning (JCTVC-J0093) was also checked and that the gains are additive. The cross-checker expressed opinion that the use of additional coefficient context for TS mode is a straightforward approach and should be reviewed with the scanning schemes.

One expert expressed opinion that the additional coefficient context scheme and a scanning scheme should be reviewed separately since the combination is a new proposal.

BoG recommendation: Report the summary of the additional coefficient context for TS mode in the main session in the main session and solicit the experts’ opinions on the necessity of the presentation.

JCTVC-J0077 Transform Skipping for Inter Predicted Coding Units [A. Gabriellini, M. Mrak, D. Flynn, M. Naccari (BBC)]

What is the objective of your proposal?  Coding efficiency improvement.

What are changed in the spec? Enable TS modes in inter-codes TUs. Single context is shared for TS flag in intra- and intra-coded TUs. In inter-coded TU, TS mode of chroma is inferred from that of the associated luma.

Category: TS for inter-coded TU.

Similar proposal in (Part1) of JCTVC-J0237.

JCTVC-J0100 Cross-check of transform skipping for inter prediction (JCTVC-J0077) [J. Sole, R. Joshi (Qualcomm)] [late]

Support the idea. BD-rates and encoding/decoding times are verified.

JCTVC-J0237 Inter transform skipping [J. Xu (Microsoft)]

What is the objective of your proposal?  Simplification and coding efficiency improvement.

What are changed in the spec? Two parts: (Part1) Simplification by removing intra/inter conditions in TS mode flag parsing. (Part2) QM matrix for TS mode (Use a fixed value of 16 .)

Category: TS mode for inter-coded TU.

What is the difference between JCTVC-J0259 and JCTVC-J0237? JCTVC-J0237 does not infer TS mode of chroma from luma, as the current TS mode specification. Thus JCTVC-J0237 also simplifies the parsing of TS mode.

Which one gives a better gain?

Encoder settings are different. Not be able to compare the results.

One expert asserted that omitting TS mode signaling in chroma does not change BD-rates even intra cases and reduces encoder running time.

JCTVC-J0237 proponent claimed that separate signaling allows a non-normative fast encoding and gives flexibility in encoder implementation.

JCTVC-J0259 proponent claimed that since motion vector is shared by Y/U/V components in inter-coded blocks, the same TS mode should be applied to all components.

Two non-proponents support separate signaling of TS mode flags in luma and chroma blocks.

JCTVC-J0326 Crosscheck of JCTVC-J0237: Inter transform skipping [M. Budagavi (TI)] [late]

Part1 is verified. Part2 QM is not cross-checked

JCTVC-J0092 Transform Skipping in the presence of Scaling Lists [D. Flynn, M. Mrak, M. Naccari, A. Gabriellini (BBC)]

What is the objective of your proposal?  Bug fix (Enhancement).

What are changed in the spec? Derive a flat matrix from DC coefficient of scaling matrix. No new syntax element.

Category: Scaling matrix for TS.

Improve visual quality? No changes with default scaling matrix.

Proponent claimed that a large weight in high frequency coefficients may result in larger distortion at boundary of TS blocks. The subsequent blocks can be degraded by prediction from the distorted boundary samples.

Why not using the value of 16 instead of DC?

It is expected to maintain the consistency among the blocks coded with TS and nonTS modes.

Why TS mode scaling matrix was not adopted in the last meeting?

It was concluded that encoders can handle it w/o using it.

JCTVC-J0184 On Transform Skip [Y Morigami, K Sato (Sony), T Sugio, H Sasai (Panasonic)]

Includes 4 parts: Part1 does not apply QM to TS mode (i.e., use 16 for TS mode block). Part2 sends single quantization factor shared for Y/U/V inside scaling list syntax. Part3 is the same as JCTVC-J0092 (i.e., use DC for TS mode block). Part4 is to move TS on/off flag from SPS to PPS.

JCTVC-J0201 QM bypass for transform skip mode [H. Y. Kim, S.-C. Lim (ETRI), K. Y. Kim, G. H. Park (KHU), J. Lee, J. S. Choi (ETRI)]

This is identical to the use of 16 for TS mode block (The Part1 of JCTVC-J0184).

JCTVC-J0420 Crosscheck report of QM bypass for transform skip mode (JCTVC-J0201) [M.Shima (Canon)] [late] [miss]

Not present.

BoG recommendation on scaling matrix for TS mode: Report the summary of three schemes (QM using 16 for TS, QM using DC for TS, and QM using explicit weight for TS) in the main session in the main session and solicit the experts’ opinions on the necessity of the presentations.

JCTVC-J0171 On transform skip mode for chroma TUs [E. François, P. Onno, C. Gisquet, G. Laroche (Canon)]

What is the objective of your proposal?  Simplification of TS mode signaling for chroma. Save encoder running time for equivalent BDR results for HM7.0

What are changed in the spec?  Two conditions (C1,C2) are tested to  disable TS for 4x4 chroma TU

C1) when luma TU is not split into 4 4x4-TUs

C2) when luma TU is not split into 4 4x4-TUs OR  when none of the 4 4x4-luma-TUs uses TS mode.

Two additional conditions for parsing TS flag in chroma TUs.

Category: Conditional signaling of TS mode flags in chroma.

Results for C1 (normative)

|  |  | Main |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| All Intra | Class A-E | -0.01% | 0.03% | 0.02% |
|  | Class F | -0.01% | -0.05% | -0.04% |
|  | Enc Time[%] | 95% |  |  |

Results for C2 (normative)

|  |  | Main |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| All Intra | Class A-E | -0.05% | 0.12% | 0.18% |
|  | Class F | -0.05% | 0.20% | 0.26% |
|  | Enc Time[%] | 93% |  |  |

Why chroma loss in some Class F sequence is large? (0.66% chroma losses in HE LB case.)

Because of high PSNR values in Class F results.

What happens when we do a non-normative encoder restriction?

Improvement ofnon-normative scheme is similar:  on average in AI Main for ClassF.

Results for C1 (non-normative)

|  |  | Main |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| All Intra | Class A-E | -0.01% | 0.03% | 0.04% |
|  | Class F | 0.00% | 0.05% | 0.08% |
|  | Enc Time[%] | 97% |  |  |

Results for C2 (non-normative)

|  |  | Main |  |  |
| --- | --- | --- | --- | --- |
|  |  | Y | U | V |
| All Intra | Class A-E | -0,03% | 0,17% | 0,21% |
|  | Class F | -0,04% | 0,41% | 0,49% |
|  | Enc Time[%] | 94% |  |  |

JCTVC-J0265 Intra Transform Skipping: Smallest CU and Implicit Chroma [G. Van der Auwera, M. Karczewicz (Qualcomm)]

What is the objective of your proposal? Part1) Restrict to use TS mode flag in 2Nx2N with RQT=2 and NxN part mode in SCU. Part2) Omitting signaling TS mode flag in chroma TUs.

What are changed in the spec? Both changes signaling conditions of TS flags in chroma TYs. (Part1) CU size dependent. ( Part2) Color component dependent.

Category: Conditional signaling of TS mode flag in chroma.

(Part2) is similar to JCTVC-J0171. Its number of additional conditions is less than JCTVC-J0171 but BD-rate results are quite different. One additional condition for JCTVC-J0265; Two additional conditions for JCTVC-J0171.

JCTVC-J0430 Cross-check of: "Intra Transform Skipping: Smallest CU and Implicit Chroma" (J0265) [A. Gabriellini, M. Mrak, M. Naccari (BBC)] [late]

BD-rate and encoding/decoding runtime were confirmed. Software and text match.

BoG recommendation on conditional signaling of TS mode flag in chroma.: Report the summary of three schemes in the main session in the main session and solicit the experts’ opinions on the necessity of the presentations.

JCTVC-J0309 Removal of last coefficient coding in transform skipping mode [K. Panusopone, Y. Yu, X. Fang, L. Wang (Motorola Mobility)]

What is the objective of your proposal?  Simplification. No signalling of lastX and last Y locations in TS mode.

What are changed in the spec?  Signaling of lastX and last Y locations. Two conditions.

Category: Skip x/y pos signaling in TS mode

Negligible changes in BD-rate.

Crosscheck J0508 (I2R.) Not present.

One expert expressed opinion that the proposal may not be simplification because of additional conditions for lastX and lastY position signaling.

BoG recommendation on JCTVC-J0309: Report the summary in the main session in the main session and solicit the experts’ opinions on the necessity of the presentation.

JCTVC-J0389 Simplification for intra transform skip mode [L. Zhao, J. An, Y.-W. Huang, S. Lei (MediaTek), D. Zhao (Harbin Institute of Technology)] [late]

What is the objective of your proposal?  Intra transform skip mode is selected only when the size of CU is 8x8 in luma and the partition mode is Intra_NxN. Non-normative change for encoder simplification. Normative change for both encoder and signaling simplification.

What are changed in the spec?  Intra transform skip mode is selected only when the size of CU is 8x8 in luma and the partition mode is Intra_NxN..

Category: Conditional signaling of TS mode flags in chroma.

BoG recommendation on JCTVC-J0389: Non-normative part is considered for adoption in software.

JCTVC-J0393 Cross-check of Simplification for Intra Transform Skip Mode in JCTVC-J0389 [W.-S. Kim (TI)] [late]

Not present.

[Lossless related] 5 proposals

JCTVC-J0061 AHG11: Study report of HEVC lossless coding [K. Chono (NEC)]

Informative contribution: This contribution compares the current HM lossless coding scheme with the previous scheme based on QPy value. It is reportedly shown that the side information associated with the CU-level flags increases the overall bit rates by 0.01%.

Particular action was not requested by the proponent.

JCTVC-J0230 Improvement of HEVC lossless coding using transform coefficient coding [Minsu Cheon, Yumi Sohn, Sunil Lee, Ilkoo Kim, Jeonghoon Park (Samsung)]

Improved lossless coding is achieved by additionally signalling the information lost in the process of transform and quantization of the residual signal on top of the existing HEVC lossy coding framework. The simulation results show that the coding gain of the proposed method is (w/o Class F) 8.8% in AI Main and 8.3% in AI HE10 configurations, on average. The proposed method can be combined with other methods for the performance improvement of HEVC lossless coding, and the additional simulation results show that (w/o Class F) the coding gain of 10.8% and 11.2% are respectively achieved in AI Main and AI HE10 configurations, when combined with sample-based angular prediction in JCTVC-I0117.

The improved scheme sends remaining residual between the original and reconstructed samples in addition to conventional prediction residual. It also skips in-loop filtering in the image blocks where remaining residual are transmitted. The remaining residual are entropy-coded by a similar approach as the current HM.

Any changes in entropy coding for remaining residual?

There are minor changes to the current HM design.

Results of RA/LB cases are available?

Not available.

The amount of changes seems to be too much for adoption at this stage. The additional entropy coding engine would require different contexts.

Further study is encouraged for future definition in lossless coding profile and scalable extension under some AHG. Proponent will report the progress of this on the main reflector.

JCTVC-J0232 HEVC lossless coding for medical image compression [Minsu Cheon, Yumi Sohn, Sunil Lee, Ilkoo Kim, Jeonghoon Park (Samsung)]

Informative contribution: The coding performance of HM7.1 lossless coding and its improvement is compared with the existing lossless image codec including JPEG Lossless, JPEG-2000 Lossless, and JPEG-LS. The simulation results show that the current HM7.1 lossless coding is outperformed by JPEG-2000 Lossless (14.7%) and JPEG-LS (10.8). The results also show that the performance of HEVC lossless coding can be significantly improved by applying the method proposed in JCTVC-I0230 – coding gain of 17.2% on average when combined with the sample-based angular prediction proposed in JCTVC-I0117.

HM lossless coding does not work as efficient as JPEG2000. Proposal with SAP (Sample-based Angular Prediction) can attain better coding efficiency than JPEG2000.

Proponent suggests that JCTVC make efforts for the performance improvement of HEVC lossless coding and for high-bit depth and gray color supports.

Does HEVC encoder complexity meet the requirements of medical image applications?

Complexity differences between JPEG2000 and HEVC are huge. However, light-weight HEVC encoders will be available in future.

One expert reported that the current low QP setting in HM-7.0 does not work appropriately when lossless coding is enabled. It is vocally reported that QP=8 setting gives 3% gain over QP=0 setting when lossless coding is enabled. Appropriate lambda setting should be well considered in the design of lossless coding tools.

Particular action was not requested by the proponent.

JCTVC-J0238 Lossless coding via transform skipping [J. Xu (Microsoft)]

This contribution shows that lossless coding can be achieved in the current design when QP=4 and both intra and inter transform skipping at 4x4 TU level are used. Besides inter transform skipping, the proposed scheme does not need other normative changes. The contribution also proposes to remove unnecessary QPs from the draft.

I_PCM is used in the tests? In low QP cases, I_PCM should give some gain.

I_PCM was not used.

It was reported that some regions in Nebuta sequence are coded with I_PCM when QP is low.

What reduces encoding and decoding times?  Deblocking is disabled?

Deblocking is not disabled. Deblocking can affect decoding time.

When large QP is mixed within a picture, the proposal does not guarantee lossless coding as the previous lossless coding scheme.

2.1% gains in AI setting can be come from lamda setting. Using a large QP value gives some gains in lossless coding. We should consider very well about the influence of QP setting since encoder mode decision can affect the results.

It is remarked that the proposal restrict the size of intra CU in lossless coding since TS is used in small CUs that can reach 4x4 TUs.

Decision on the first aspect (lossless coding signaling based on QPy) is deferred until the review of JCTVC-J0435.

Not reach the consensus on removing the least 4 QPs from DIS.

JCTVC-J0363 AHG11: Cross-check of JCTVC-J0238 [Wen Gao, Minqiang Jiang, Haoping Yu (Huawei)] [late] [miss]

Verified.

JCTVC-J0339 AHG11: Methods to Encode Lossless Coding Indication Flag [Wen Gao, Minqiang Jiang, Haoping Yu (Huawei)]

In [1], two different flags, i.e., a flag in PPS, transquant_bypass_enable_flag and a flag at each CU, cu_transquant_bypass_flag, in all slices that refer to the PPS are used to enable lossless coding operation through bypassing of transform, quantization, an in-loop filter and sign data hiding in residual coding. When transquant_bypass_enable_flag=1, cu_transquant_bypass_flag will exist at every coding unit (CU) and lossless operation only applies to a CU when cu_transquant_bypass_flag=1. The flag, cu_transquant_bypass_flag, is encoded using one context model that is initialized by setting most probable symbol (MPS) to 1 and the probability of MPS to 0.5. Here we propose two methods to encoder this flag cu_transquant_bypass_flag for better efficiency.

The following two methods are proposed:

Method1: Prediction based encoding of cu_transquant_bypass_flag

Method2: Three contexts for encoding of cu_transquant_bypass_flag for region-based lossless coding.

Proponent suggests considering the adoption of Method2. Method2 gives 1% gain in region-based lossless coding in LB setting.

Context modeling in Method2 uses up and left cu_transquant_bypass_flag values.

Expert expressed an opinion that Method2 increases line buffer requirement and that three additional contexts are too much for a limited use case (region-based lossless coding).

Two methods give marginal gains only in a region-based lossless coding scenario.

Have you tested a case that uses two context that uses left lossless flag?

Not tested.

No action on this proposal.

JCTVC-J0340 AHG11: New signalling mechanism for lossless coding [Wen Gao, Minqiang Jiang, Haoping Yu (Huawei)]

In the last JCTVC meeting, two flags [1], i.e., a PPS level flag transquant_bypass_enable_flag and a flag at each CU in a slice, cu_transquant_bypass_flag, were adopted to enable lossless coding operation through bypassing of transform, quantization, an in-loop filter and sign data hiding at residual coding. When transquant_bypass_enable_flag=1, cu_transquant_bypass_flag will exist at every CU and lossless operation only applies to a CU when cu_transquant_bypass_flag=1, otherwise the normal operation with transform/quantization/in-loop-filter will apply to all CUs with cu_transquant_bypass_flag=0. We observe that this signaling mechanism will cause significant overhead if only a small area in a slice is coded in lossless mode. Hence we propose an alternative signaling mechanism that can mitigate this overhead problem.

Proposal introduces a TransQuant(TQ) group concept for region-based lossless coding. TQ group is specified by new PPS-level side information and is similar to the quantization group concept. cu_transquant_bypass_flags are signaled in CUs within a TQ group. The proposal can omit signaling og some cu_transquant_bypass_flags. In ClassF, it gives 0.2% gain in LB in a region-based lossless coding scenario.

Two experts expressed an opinion that I_PCM-like design would be desirable for cu_transquant_bypass_flag. The similar idea is proposed in JCTVC-J0168.

Experts expressed an opinion that the gain is very limited and that the I_PCM-like design or TQ group should be designed with consideration of the balance of decoder changes and gains.

Not reached consensus on this topic.

JCTVC-J0435 Lossless Coding and Loop Filter Control for Transform Skip [G. Van der Auwera, R. Joshi, V. Seregin, M. Karczewicz (Qualcomm)] [late]

The contribution JCTVC-J0238 proposes the use of transform skip mode in conjunction with QPY value equal to 4. The problem is that cu_qp_delta is signalled only when a TU with non-zero residual is encountered. Secondly, it is claimed that the deblocking filter is disabled on the lossless samples if QPY value is equal to 4, however, this cannot be guaranteed. This contribution proposes and evaluates two alternatives to mitigate the observed problems. The first proposal (A) is based on JCTVC-J0238 and proposes to define a “slice_transquant_bypass_flag” to control the loop filters if the transform_skip_flag is equal to 1 and QPY is equal to 4, while enforcing that cu_delta_qp shall be signalled at the beginning of the CU or minimum CU quantization group size. The second proposal (B) unifies HM7 “TransQuantBypass” lossless coding with transform skipping to solve the reported issues.

The proposal states the following two problems:

- Lossless coding based on transform_skip_flag and QPy in JCTVC-J0238 is not able to guarantee lossless coding when cu_qp_delta is signalled only when a TU with non-zero residual is encountered.

- Lossless coding based on transform_skip_flag is not able to guarantee lossless coding when a large QP is mixed in a slice.

The following solutions are proposed:

Solution1: Move the location of cu_qp_delta to the CU header.

Solution2: Combine cu_level_lossless flag with transform_skip_flag.

Experts expressed an opinion that Solution2 seems to bea  straightforward way than Solution1 since Solution1 requires significant changes in the current text.

Decision on QPy based solution: The approach is not able to guarantee lossless coding. A QPy based scheme was the previous HM lossless coding design and fixed in the last meeting. QPy based lossless coding signaling should not be considered.

Decision on transform_skip_flag based solution: The approach is not able to guarantee lossless coding. Solution2 seems to be a straightforward way. But do we need to change the current design studied over one meeting cycle? One expert expressed an opinion that solution2 changes only the semantics of two flags. The approach should be well discussed.

Recommend discussing the necessity of lossless coding signaling based on transform_skip_flag.