| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0515 |
| --- | --- |

| Title: | Cross-verification of Combination of JCTVC-J0069 and JCTVC-J0093 (JCTVC-J0468) by BBC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Andrea Gabriellini Matteo Naccari Marta Mrak | Email: | andrea.gabriellini@bbc.co.uk matteo.naccari@bbc.co.uk marta.mrak@bbc.co.uk |
| Source: | BBC Research & Development |  |  |

_____________________________

# Abstract

This document reports the cross-check of proposal JCTVC-J0468 on Combination of JCTVC-J0069 and JCTVC-J0093. The proposal targets entropy coding for intra predicted residuals whereby transform skip is applied. For this purpose, the two methods proposed in JCTVC-J0069 and JCTVC-J0093 are combined to improve the compression efficiency. The software provided by the proponents was examined and coding of JCT-VC contents over common test conditions was performed. The simulation results match those provided by the proponents as well as the software and WD changes are consistent with the described method.

# Introduction

At the last JCT-VC meeting in Geneva, Transform Skip over 4×4 intra predicted residuals was adopted. When transform skip is applied, the entropy coding process is also influenced. In particular, given the different statistics associated to TS residues, the coefficient scanning and the CABAC context modelling should be modified accordingly. To this end, documents JCTVC-J0069 and JCTVC-J0093 address these issues as follows:

- JCTVC-J0069: Introduces two additional constant contexts for the SIG flag syntax element. These two contexts take into account the different statistics associated to the SIG flag when TS is applied.

- JCTVC-J0093: Performs a counter wise 180 degrees rotation of the residuals to compensate for the different statistics associated with the TS residuals and use the same contexts of the current HEVC specification.

In document JCTVC-J0468 the aforementioned proposals are combined together to improve the compression efficiency. The combination is tested both for lossy and for lossless coding. In the ensuing sections, the validation of the results, source code review as well as working draft text changes are presented.

# Results validation

## Lossy coding scenario

As request by the proponents, only All-Intra High Efficiency 10 was tested. The BD-rates are listed in the following table and match the one obtained by the proponents.

|  | All Intra HE10 |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.3% | -0.2% |
| Class B | -0.1% | -0.3% | -0.4% |
| Class C | -0.4% | -0.9% | -1.1% |
| Class D | -0.6% | -1.1% | -1.4% |
| Class E | -0.2% | -0.3% | -0.4% |
| Overall | -0.3% | -0.6% | -0.7% |
|  | -0.3% | -0.6% | -0.7% |
| Class F | -2.2% | -2.7% | -2.8% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 102% |  |  |

## Lossless coding scenario

As request by the proponents, only All-Intra main was tested. The BD-rates are listed in the following table and match the one obtained by the proponents.

|  | All Intra Main |
| --- | --- |
| Class A | -2.8% |
| Class B | -1.7% |
| Class C | -2.7% |
| Class D | -3.1% |
| Class E | -2.5% |
| Class F | -3.0% |
| Overall | -2.6% |
| Enc Time[%] | 103% |
| Dec Time[%] | 106% |

# Source code review

The provided source code matches the proposal text.

# WD text changes review

The proponents have also provided the WD changes required by the combination of both JCTVC-J0069 and JCTVC-0093. The modifications involve the following sections in the WD text:

- Scaling and transformation process

- Transformation process for scale transform coefficients

- Initialisation process for context variables

- Binarisation process

- Derivation process for ctxIdx

- Derivation process of ctxIdxInc for the syntax element significant_coeff_flag

The changes match the proposal text description.

# Conclusion

Document JCTVC-J0468 described the combination of proposals JCTVC-J0069 and JCTVC-J0093 for transform skip residuals entropy coding Given the different statistics associated to the coded levels when TS is applied, the usage of separated contexts and the counter wise 180 degrees rotation for the transform skip case improves the BD-rates, namely for Class F videos. The reported results match the one reported by the proponents. Moreover, the source code as well as the WD text changes match the proposal description.