| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0517 |
| --- | --- |

| Title: | Mental cross-check of JCTVC-J0185 (On short_term_ref_pic_set) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | ChongSoon Lim | Tel: Email: | (65) 6550 5368 chongsoon.lim@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This document provides a mental cross-check for JCTVC-J0185

# Problem statement

The cross-checker basically understands the proposal JCTVC-J0185 is describing two problems.

- Redundancy in the signaling of inter_ref_pic_set_prediction_flag when idx of short_term_ref_pic_set() is set to zero.

- Signaling of delta_idx_minus1 is always set equal to zero for all idx values in common test conditions.

# Mental cross-check

For the first problem, the cross-checker understands that the proposed method in JCTVC-J0185 is to conditionally signal the inter_ref_pic_set_prediction_flag when idx of short_term_ref_pic_set() is not equal to zero.  The cross-checker also notes that the current HEVC text specification potentially has a problem as it does not constraint the value of inter_ref_pic_set_prediction_flag to be set to zero when idx of short_term_ref_pic_set() is equal to zero.

For the second problem, the cross-checker understands that the proposed method in JCTVC-J0185 is to send two parameters in the SPS to avoid signaling of the same value of delta_idx_minus1 in all idx values of short_term_ref_pic_set().  The first parameter is a flag to indicate whether if delta_idx_minus1 in short_term_ref_pic_set() has the same value and the second parameter is the delta_idx_minus1 value that is common to all short_term_ref_pic_set().

The cross-checker would also like to comment that JCTVC-J0185 also introduces a new functionality with a new flag in SPS to disable RPS prediction of all short_term_ref_pic_set().

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).