| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0518 |
| --- | --- |

| Title: | Mental cross-check of JCTVC-J0183 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 44 431 7658 hao@bu.jp.nec.com |
| Source: | NEC Corporation |  |  |

_____________________________

# Abstract

This document provides a cross-check report for the document of JCTVC-J0183. It is claimed that the author of this document understands what is being proposed in JCTVC-J0183. In summary, JCTVC-J0183 presents three proposals removing redundancies of high-level syntax, although the second and the third proposals are put together in the document, as follows.

- temporal_id_nesting_flag is omitted when max_temporal_layers_minus1 is equal to 0, i.e. there is only one temporal layer.

- The range of diff_cu_qp_delta_depth is restricted by the value of log2_min_transform_block_size_minus2. In order to avoid the dependency of PPS on SPS, the syntactical locations of diff_cu_qp_delta_depth, slice_granularity and log2_min_transform_block_size_minus2 are changed.

- transform_skip_enable_flag is omitted when log2_min_transform_block_size_minus2 is equal to 0.

It is confirmed that the draft syntaxes conforming to those proposals are provided. It should be noted that the second proposal introduces alternative redundancy in PPS and SPS, but also seems to imply the question whether those syntax elements should be signalled in PPS, i.e., should be changed depending on picture characteristics, or not. It is recommended that the issues raised in this document be discussed at the meeting.

# Patent rights declaration(s)

NEC Corporation does not have any current or pending patent rights relating to the technology described in this contribution.