| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 10th Meeting: Stockholm, SE, 11–20 July 2012 | Document: JCTVC-J0519 |
| --- | --- |

| Title: | Mental cross-check of JCTVC-J0121 (Motion related hooks for HEVC multiview/3DV extension based on long-term reference pictures) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | ChongSoon Lim | Tel: Email: | (65) 6550 5458 chongsoon.lim@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This document provides a mental cross-check for JCTVC-J0121

# Problem statement

The cross-checker basically understands the proposal JCTVC-J0121 is attempting to avoid predicting a motion vector corresponding to inter-view reference using a temporal motion vector and vice versa in the case of 3D-HEVC to improve coding efficiency.  Existing HEVC text specification disables the motion vector scaling when at least one of the reference pictures is a long-term reference picture but the neighbour/collocated motion vector is still included as a candidate for motion vector prediction.

The cross-checker understands that to be compatible to the base HEVC text specification, an inter-view reference picture can be re-labelled as a long-term reference picture. However, the current design does not prevent prediction between inter-view motion vectors and temporal motion vectors.

# Mental cross-check

The cross-checker notes that to prevent prediction between inter-view motion vectors and temporal motion vectors, the proposal JCTVC-J0121 adds new conditions in the derivation process for motion vector predictor candidates to compare if the two reference pictures are from the same category (both long-term reference pictures or both short-term reference pictures).  In the case if the reference pictures are not from the same category, the neighbour/collocated motion vector is not added as a candidate for motion vector prediction.

The cross-checker adds that coding efficiency improvement is reported in the proposal JCTVC-J0121 in the case of 3 views as compared to an anchor without the proposed change.

In addition, the cross-checker notes that the proposed change will also affect derivation process for motion vector predictor candidates for non-3D cases when long-term reference pictures are used.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).